3GPP TSG-SA WG2 Meeting #140E 	S2-2005593

19 August – 2 September, 2020, Elbonia	 (revision of S2-200xxxx)

Source:	CATT

Title:	KI #4, Sol#14: Update the solution to remove ENs

Document for:	Approval

Agenda Item:	8.7

Work Item / Release:	FS_ID_UAS-SA2 / Rel-17

Abstract of the contribution: this paper proposes to update solution #14 in TR 23.754.

Discussion

This pCR is proposed to update solution#14 with following aspects:

- To delete the FFS of “whether there are other cases is FFS”, the following new cases for tracking are added:

  - UAV tracking based on flight path report generated by UE or UFES.

  - UAV tracking for UAV unknown to UTM.

- To delete FFS of “what information is included in UE trajectory and the trigger for UAVF to report to the UTM is FFS”, the following description is added:

  - The trajectory includes a number of waypoints defined as 3D locations and time stamps per way point.

  - When UFES decides that the UAV location deviate the way point, it sends Flight route tracking response (deviation from trajectory notify, way point, UE location, deviation distance) to the UTM.

- Rename the UAVF to UFES to align with solution #5.

Proposal

It is proposed to update solution #14.

First change

6.14	Solution #14: UAV and UAVC tracking

6.14.1	Introduction

This solution addresses KI#4: UAV and UAV Controller tracking:

-	What information is required for the 3GPP system to track the UAV and the UAV Controller.

6.14.2	Functional Description

In this solution, the FlightPathInfoReportConfig and the FlightPathInfoReport defined in TS 36.331 are reused. Based on the description in TS 36.331,

-	FlightPathInfoReportConfig includes:

-	The maxWayPointNumber: indicates the maximum number of way points UE can include in the flight path information report if this information is available at the UE.

-	The includeTimeStamp: indicates whether time stamp of each way point can be reported in the flight path information report if time stamp information is available at the UE.

-	FlightPathInfoReport includes the list of waypoints along the flight path and time stamps per waypoint if configured.

The UTM sends FlightPathInfoReportConfig to UE or UFES, the UE or UFES reports the flight path report to UTM. If the report is generated by the UE, the report transmission between UE and RAN defined in EPS is reused.

6.14.3	Procedures

6.14.3.x	UAV tracking based on flight path report generated by UE

Figure 6.14.3.x-1: UAV tracking based on flight path report generated by UE

0.	During UAV bootstrapping, the UTM sends flight path information report configuration to UAV. The configuration is the same as the FlightPathInfoReportConfig IE defined in clause 6.3.6 in TS 36.331 and includes the maximum number of way points UE can include in the flight path information report and indication of whether time stamp of each way point can be reported in the flight path information. This step is out of scope of 3GPP.

1.	The UTM sends Flight Path Request (CAA-level UAV ID) to UFES to request flight path information of the UAV.

2.	The UFES obtains UE ID mapped to the CAA-level UAV ID based on local storage or from HSS/UDM. In EPS, the UFES sends Subscriber Information Request (UE ID) to HSS to get serving MME of the UE. In 5GS, the UFES invokes Nudm_UECM_Get_Request (UE ID) to UDM to get serving AMF of the UE.

3.	The HSS/UDM returns the MME/AMF ID to the UFES.

4.	The UFES sends Flight Path Request (UE ID) to MME/AMF.

5.	The MME/AMF sends S1-MME/N2 message to E-UTRAN/NG-RAN. The message includes UE ID, flight path request indication.

6.	The E-UTRAN/NG-RAN sends UEInformationRequest (FlightPathReq) to UE. The detail of the message and parameter is described in clause 6.2.2 in TS 36.331.

7.	The UE sends UEInformationResponse (FlightPathInfoReport) to UE. The report includes a number of waypoints defined as 3D locations and can consist also time stamps per waypoint if configured as described in clause 23.17.5 in TS 36.300.

8.	The E-UTRAN/NG-RAN sends S1-MME/N2 message to MME/AMF. The message includes UE ID and flight path information report.

9.	The MME/AMF sends Flight Path Response (UE ID, flight path information report) to UFES.

10.	The UFES converts the UE ID to CAA-level UAV ID and sends Flight Path Notify (CAA-level UAV ID, flight path information report) to UTM. Based on the report, the UTM may decide whether to send warning to UAV.

6.14.3.y	UAV tracking based on flight path report generated by UFES

Figure 6.14.3.y-1: UAV tracking based on flight path report generated by UFES

1.	The UTM sends Flight path report configuration message to UFES. The message including the CAA-level UAV ID and flight path information report configuration. The configuration is the same as the FlightPathInfoReportConfig IE defined in clause 6.3.6 in TS 36.331 and includes the maximum number of way points UE can include in the flight path information report and indication of whether time stamp of each way point can be reported in the flight path information.

2.	The UFES invokes UE positioning procedure defined in TS 23.273 or TS 23.271 to obtain UE locations. The UFES stores UE location and also time stamps per UE location locally or in the UDM. When does the UFES invoke the positioning procedure is implementation dependent.

3.	The UTM sends Flight Path report Request (CAA-level UAV ID) to UFES to request flight path information of the UAV.

4.	The UFES sends Flight Path report Response (CAA-level UAV ID, flight path information report) to UTM. The flight path information report includes a number of waypoints defined as 3D locations and can consist also time stamps per waypoint if requested by the configuration. In the UFES stores the flight path information in UDM is step 3, the UFES obtains the information from UDM.

5.	Based on the flight path information report, the UTM may perform flight control, e.g. sends warnings or updates flight route. The step is out of 3GPP scope.

6.14.4	Impacts on services, entities and interfaces

UFES:

-	Acts as LCS Client to collect UE locations from 3GPP system.

-	Receives flight path information report configuration or CAA-level UAV ID from UTM.

-	Sends flight path information report generated by UFES or UE to UTM. If the report is gernerated by UE, the UFES supports to request and receive flight path information report from MME/AMF.

MME/AMF:

-	Receive Flight path request from UFES.

-	Request flight path information report from E-UTRAN/NG-RAN and provide the report to UFES.

NG-RAN:

-	UE Information procedure is enhanced to support flight path information report transmission.

NOTE 1:	This function is already supported by E-UTRAN.

UTM:

-	Provide flight path information report configuration to UFES/UAV.

End of changes