3GPP TSG-WG SA2 Meeting #140E e-meeting 	S2-2005625r05

Elbonia, August 19 – September 1, 2020	(revision of S2-200xxxx)

Source:	Huawei, HiSilicon

Title:	KI #2: evaluation and conclusion

Document for:	Approval

Agenda Item:	8.5

Work Item / Release:	FS_IIoT / Rel-17

Abstract: This contribution gives the evaluations for the solutions of KI#2 and proposes the conclusion for KI#2.

2. Text Proposal

It is proposed to capture the following changes vs. 23.700-20 V0.4.0.

# * * * * First change * * * *

# * * * * First change * * * *

8.X	Key Issue #2: UE-UE TSC communication

Editor's note:	This clause will capture conclusions for Key Issue #<X>.

The following is taken as the basis for the way forward:

-	(TSN) AF provides information about the UE-UE TSC stream separately to the 5GS for the PDU Session carrying the uplink traffic and for the PDU Session(s) carrying the downlink traffic.

-	For the PDU Session carrying the uplink traffic, the information about the UE-UE TSC stream includes the packet filters of UE-UE TSC stream, UE-UE TSC indication, QoS requirements for this PDU session carrying the uplink traffic such as delay, burst size, periodicity, and burst arrival time

-	For the PDU Session carrying the downlink traffic, the information about the UE-UE TSC stream includes the packet filters of UE-UE TSC stream, UE-UE TSC indication, QoS requirements for this PDU session carrying the uplink traffic such as delay, burst size, periodicity, and burst arrival time

Editor's note:	How AF determines the UE-UE TSC indication and whether this is required is FFS?

-

# * * * * End of changes * * * *