SA WG2 Meeting #140E	S2-2005653

19th August – 2nd September, 2020

Source:	Samsung

Title:	KI #2, Sol #26: Update to remove ENs

Document for:	Approval

Agenda Item:	8.1

Work Item / Release:	FS_eNA_ph2/ Rel-17

Abstract of the contribution: The contribution resolves EN’s and clarifies remaining technical issues of Solution #26.

- Discussion

The Solution #26 proposed a procedure for reselection of NWDAF which supports transmission of stored analytics data and/or input data between NWDAF instances when the serving NWDAF is changed. There are still three Editor’s Notes in the TR which were introduced in the previous #139 e-meeting. This Tdoc provides proposed resolution of those ENs and clarifies additional details of the solution.

The remaining ENs for Solution #26 are referred to as follows:

- Editor’s Note:	How the NWDAF decides the need of NWDAF instance change based on what trigger conditions is FFS.

- Editor’s Note: How the NWDAF finds an appropriate relocation candidate is FFS.

- Editor’s Note: The exact signaling message for the release of the subscription, e.g., Nnwdaf_AnalyticsSubscription_Notify or Nnwdaf_AnalayticsUnsubscribe_Request, is FFS.

The proposed resolution of each EN is discussed below:

1) How the NWDAF decides the need of NWDAF instance change based on what trigger conditions:

The solution assumed the scenario in which the relocation of the serving NWDAF for some consumers can be triggered by an NF or the OAM who has reported to the NWDAF the change of the network status (caused by events already listed in TR, that is, UE mobility, change in NWDAF profile, load rebalancing due to new addition or removal of NWDAF instances, etc.). The actual decision on whether the relocation should be executed shall be made by the current serving NWDAF and will be dependent on the implementation logic, operator configuration and so forth. It is proposed to revise the text accordingly to reflect clarification above.

2) How the NWDAF finds an appropriate relocation candidate

The solution assumes that the NWDAF will decide whether the relocation/reselection should be performed or not and also to which NWDAF instance. To select a target NWDAF, NWDAF may refer to the NF profile information from NRF using existing input parameters, such as Analytics ID(s), preferred target NF location and TAI,  for NRF service (Nnrf_NFDiscovery) in TS 23.501/502. It is proposed to add a text for this clarification.

3) Signaling message for the release of the subscription, e.g., Nnwdaf_AnalyticsSubscription_Notify or Nnwdaf_AnalayticsUnsubscribe_Request

According to clause 7.2 in TS 23.288, Nnwdaf_AnalyticsSubscription Service defines as follows:

7.2	Nnwdaf_AnalyticsSubscription Service

7.2.1	General

Service Description: This service enables the consumer to subscribe/unsubscribe for network data analytics.

When the subscription is accepted by the NWDAF, the consumer NF receives from the NWDAF an identifier (Subscription Correlation ID) allowing to further manage (modify, delete) this subscription. The modification of Analytics subscription can be enforced by NWDAF based on operator policy and configuration

The consumer is supposed to send Nnwdaf_AnalyticsUnsubscribe, including the Subscription Correlation ID which is received from the NWDAF, to cancel a subscription. And, for the communication from the NWDAF to a consumer, the NWDAF is supposed to include the Notification Correlation Information which is assigned by and used to identify the subscription within the consumer. It is proposed in the Tdoc to stick to this convention and use Nnwdaf_AnalyticsSubscription_Notify for release of a subscription by the NWDAF. That is, it is proposed to simply remove the EN from the TR. However, we don’t have strong preference on this and are open to whichever message SA2 decides to use.

2.	Proposal

It is proposed to adopt following changes in the TR 23.700-91.

*** Start of the 1st change ***

6.26	Solution #26: Reselection of NWDAF

6.26.1	Description

6.26.1.1 General

This solution is for Key Issue #2: Multiple NWDAF instances.

In scenarios that multiple NWDAF instances are deployed in the same 5G network, there are some cases that the serving NWDAF of a consumer NF may need to be changed during the operation due to, e.g., UE mobility, change in NWDAF profile, new addition or removal of NWDAF instances, etc. Upon change of a serving NWDAF instance, it is required to inform the related consumer NF of the event and possibly to update subscriptions between the NWDAF and the consumer NF.  The subscriptions can be renewed between the new NWDAF instance and the consumer NF, while some of the analytics data and stored input data can be retrieved from the previous serving NWDAF instance by the new NWDAF instance, if applicable, to facilitate producing analytics at the new NWDAF. The new NWDAF may provide the previous serving NWDAF with information on data types it wants to retrieve from the previous serving NWDAF.

6.26.1.1	Procedure

Figure 6.26.1.1-1: Reselection of NWDAF instance

- A consumer NF sends a Nnwdaf_AnalyticsSubscription_Subscribe message to the NWDAF #1 which request subscription to the analytics data indicated by Analytics ID, Target of Reporting and Event Reporting Information.

- The NWDAF#1 sends Nnf_EventExposure_Subscribe message to request subscription of input data from the source NF.

- The source NF responds to the NWDAF with Nnf_EventExposure_Notify to accept the requested subscription.

- The NWDAF #1 sends Nnwdaf_AnalyticsSubscription_Notify to the consumer NF to inform that the requested subscription has been accepted.

- The NWDAF #1 decides reselection of NWDAF instance, based on, e.g., UE mobility (event notification from AMF), change in NWDAF profile (e.g., change of NWDAF service area or selection priority), new addition or removal of NWDAF instances, etc.

.

- The NWDAF #1 sends a Nnwdaf_AnalyticsRelocation_Request message to the NWDAF #2 to trigger relocation. The message includes subscription information, consisting of Subscription ID, Analytics ID, Target of Reporting and Event Reporting Information, that the NWDAF#1 stores for analytics subscriptions of consumer NFs. To select a target NWDAF, the NWDAF #1 may retrieve the NF profile information of candidate target NWDAFs from the NRF, using Nnrf_NFDiscovery with input parameters, such as Analytics ID(s), preferred target NF location and TAI) as specified in TS 23.501/502.

- The NWDAF #2 responds to the NWDAF #1 with a Nnwdaf_AnalyticsRelocation_Response message which includes information on the subscription selected by the NWDAF #2.

- The NWDAF #1 sends Nnwdaf_AnalyticsSubscription_Notify to the consumer NF to inform the release of the subscriptions for the consumer NF.

- The NWDAF #1 sends Nnwdaf_AnalyticsRelocation_Complete to the NWDAF #2 which optionally includes the stored analytics data and/or the collected input data.

- The NWDAF #2 sends the source NF a Nnf_EventExposure_Subscribe which requests subscription for data collection from the source NF.

- The source NF responds to the NWDAF #2 with a Nnf_EventExposure_Notify.

- The NWDAF #2 sends Nnwdaf_AnalyticsSubscription_Notify to request to renew the subscription identified by the subscription IDs received from the NWDAF #1, using analytics subscription information received in step 6.

6.26.2	Impacts on Existing Nodes and Functionality

NWDAF:

- Need to upgrade to support a subscription transfer procedure

- Need to upgrade to transfer and receive stored analytics and input data

Consumer NF:

- Need to upgrade to support subscription renewal with a new NWDAF instance

- Need to support an additional parameter for Nnwdaf_AnalyticsSubscription_Notify which indicates reselection of NWDAF instance

*** End of the 1st change ***