SA WG2 Meeting SA2-140e	S2-2005685

19th August – 2nd September, 2020, E-meeting	(revision of S2-201xxxx)

Source:	Intel

Title:	KI #1, Proposed way forward

Document for:	Discussion / Approval

Agenda Item:	8.10

Work Item / Release:	FS_MUSIM / Rel-17

Abstract of the contribution: Proposes to interim conclusions on KI#1.

1	Discussion

During the FS_MUSIM conference call Intel proposed an evaluation of solutions and a way forward proposal on KI#1:

https://www.3gpp.org/ftp/tsg_sa/WG2_Arch/TSGS2_140e_Electronic/INBOX/CCs/2020_08_06_FS_MUSIM_PreSA2%23140E/INT%20S2-200xxxx_FS_MUSIM_wf.pptx

This contribution updates the way forward proposal and formats it to fit TR 23.761.

2	Proposal

It is proposed to update TR 23.761 as indicated below.

* * * * Next Change * * * *

7	Evaluation

Editor's note:	This clause will provide a general evaluation of the solutions.

* * * * Next Change * * * *

8	Conclusions

Editor's note:	This clause will capture conclusions from the study.

8.1	Key Issue #1: Handling of Mobile Terminated service with Multi-USIM device

Based on the evaluation in clause 7.1, the following interim concusions are agreed for the baseline functionality:

Among the solution principles with RAN impact:

-	A Negotiated Short Absence (as in Solution #2) is  considered for further study for 5GS, pending feedback from RAN WGs.

-	A Busy indication (as in Solution #3)  is  considered for further study supported for 5GS, pending feedback from RAN WGs.

Editor’s Node:	It is FFS whether this procedure of Busy indication can be as part of the procedure for coordinated leaving

-	MUSIM Assistance Information for paging filtering (as in Solution #10) can be considered for further study . Further conclusions on MUSIM Assistance Information for paging filtering is to be discussed.

Editor’s note:	The decision related to the other add-ons (Sol#9, Sol#11) is postponed until Q4.

Editor’s note:	The decision whether solutions without RAN impact based on notifications (e.g. Sol#7, Sol#12, etc.) are to be considered for normative work will be made in Q4.

* * * * End of Change * * * *