SA WG2 Meeting #140E	S2-2005734r02

19 August - 02 September 2020

Source:	Nokia, Nokia Shanghai Bell, Qualcomm Incorporated, NTT DOCOMO, Ericsson, Samsung

Title:	KI#1, Sol#1: Updates to Solution #1 for Uplink Time Synchronization

Document for:	Discussion/Approval

Agenda Item:	8.5

Work Item / Release:	FS_IIoT / Rel-17

Abstract of the contribution: This contribution updates solution #1 for Uplink Time Synchronization

1		Discussion

This contribution updates Key Issue #3B solution #1 for Uplink Time Synchronization.

2		Proposal

The following change is proposed for TR 23.700-20.

Meger of S2-2004918

*** Start Change ***

6.1	Solution #1: Uplink Time Synchronization for TSN

6.1.1	Introduction

The solution is proposed to solve Key Issue #1: the UL Time Synchronization. In this key issue, one or more TSN GM(s) are attached to the device side and are used to synchronize the TSN end stations behind the 5GS (i.e. NW-TTs) and behind the other UEs (i.e. DS-TTs). Based on clause 5.27.1 of TS 23.501 [2], the following principles are also applied to or UL TSN Time Synchronization:

-	5GS as a TSN bridge plays as a time-aware system.

-	Only TSN TTs at the edge of 5GS need to support the desired operations in the IEEE 802.1AS [6].

-	Two separate distribution time mechanisms are used for 5GS internal clock and TSN system clocks as defined in clause 5.27.1.2 of TS 23.501 [2].

-	All gPTP messages generated from TSN GM(s) are transmitted using user-plane resources in 5GS.

-	All the gNBs, DS-TT/UEs, and UPF/NW-TTs in the 5GS are time synchronized to the same 5GS GM.

Therefore, this solution proposes that similar operations defined for DL TSN Time Synchronization can be re-used for the UL TSN Time Synchronization i.e. for the TSN GM(s) attached to the device side.

Since UL TSN Time Synchronization will be distributed to the TSN end stations attached to 5GS (i.e. NW-TTs) and attached to the other UEs (i.e. DS-TTs), TSN GM(s) attached to the device will generate the UL gPTP messages and then DS-TT can perform exactly the same operations for the received DL gPTP messages as NW-TT performs for the DL gPTP messages defined in clause 5.27.1.2.2 of TS 23.501 [2]. Then, the modified UL gPTP messages will be further forwarded via the user-plane established between the devices (i.e. UE) which has TSN GM(s) attached to and the target UPFs.

After the NW-TT receives the modified gPTP messages for the case where delivery to end stations behind the 5G system (NW-TT) is required, the NW-TT can perform exactly the same operations as DS-TT performs for the received DL gPTP messages defined in clause 5.27.1.2.2 of TS 23.501 [2]. Finally, NW-TT can forward to the UL gPTP messages to the TSN end stations.

The main difference between the UL and DL Time Synchronization is that the UL gPTP messages are also required by the TSN end stations behind the other UEs (i.e. DS-TTs). For delivery of gPTP messages to TSN end stations behind other UEs, the UPF will forward the UL gPTP messages transparently to other UEs but not send it back to the source UEs. The forwarding process is done according to N4 rules configured by the SMF. Packet replication and detection carry on information attributes within the Packet Detection Rules (PDRs) support multicast traffic forwarding preventing the UPF from sending the multicast packets back to the source UE or source N19/N6. The DS-TT in the other UE can perform exactly the same operations as defined in clause 5.27.1.2.2 of TS 23.501 [2]. Finally, DS-TT can forward  the gPTP messages to the TSN end stations as described in the figure 6.1.2-1.

NOTE:	The UPF forwards the gPTP message to the Ethernet connected to the N6 interface. This enables other devices on N6 and also other DS-TT/UE connected to a different UPF to receive the gPTP message.

6.1.2	Functional Description

Distributing the UL gPTP messages from TSN GM attached to the device to all TSN end stations behind the UPFs (NW-TTs) and behind the other UEs (DS-TTs) can be achieved by using the following steps (referred to Figure 6.1.2-1)

-	DS-TT which is attached by one or more TSN GMs will perform exactly same operations for UL gPTP messages as NW-TT perform the operations for the DL gPTP messages as specified in clause 5.27.1.2.2 of TS 23.501 [2].

-	In case of synchronizing TSN end stations behind 5G System (NW-TT), NW-TT will perform exactly the same operations for UL gPTP messages as what DS-TT performs for the DL gPTP messages as specified in clause 5.27.1.2.2 of TS 23.501 [2]. After NW-TT on the UPF side receives the UL gPTP messages, the NW-TT forward the information to the TSN end stations, while adding 5GS residence time to the correction field and removing TSi timestamp from the suffix field.

-	In case of synchronizing TSN end stations behind other UE(s), the UPF will forward the received UL gPTP message transparently to the other UEs to distribute the gPTP messages further to the TSN end stations behind other UEs (other DS-TTs) by applying the same forwarding principles as described in clause 5.27.1.2.2 of TS 23.501 [2] except that the PDU session of the source device connected to the corresponding DS-TT port is not included in the forwarding.

- 	For traffic forwarding, dedicated Packet Detection Rules (PDRs) can include packet replication and detection carry on information to control the multicast forwarding functionality of the UPF (i.e. avoid play back to the source UE).

NOTE: Normal Ethernet behaviour is that the frame is not sent back to the source. So, sending back a frame on the incoming interface would be against Ethernet principles and results in risking creation of forwarding loops.

In case of synchronizing TSN end stations behind 5G System (NW-TT), all gPTP messages are transmitted using the user-plane resources in 5GS and follows the principle that one PDU session is established per DS-TT port for a UPF. To support the multiple TSN working domains attached to the device, the mechanism defined in clause 5.27.1.3 of TS 23.501 [2] can be re-used.

In case of synchronizing TSN end stations behind other UE(s), all gPTP messages are transmitted using the user-plane resources in 5GS and follow the principle that two PDU sessions are used.

To support the multiple TSN working domains attached to the device, the mechanism defined in clause 5.27.1.3 of TS 23.501 [2] can be re-used.

Figure 6.1.2-1: The distribution of UL Time Synchronization Information with the same UPF

6.1.3	Procedures

The procedure for synchronizing TSN end stations behind 5G System (NW-TT) with the TSN GM in the network attached to the device is described in Figure 6.1.3-1. It includes BMCA-related steps (step 1~3) because after those steps, the sync message forwarding tree is confirmed. In this case, TSN Node 0 should send sync messagess to 5GS TSN Bridge, and the 5GS TSN Bridge should send sync messages to TSN Node 2. The remaining steps (step 4~8) follow principles in clause 5.27.1 of TS 23.501 [2], with the Ingress TT of DS-TT1 and the Egress TT of NW-TT.

Figure 6.1.3-1: Procedure for synchronizing TSN end stations behind 5G System (NW-TT) with the TSN GM in the network attached to the device side via 5G System

1.	PDU Session Establishment includes UE MAC / Port Info for updates to TSN AF. After this step, TSN Node 0, 5GS logical bridge (DS-TT1/UE1, NW-TT/UPF) and TSN Node 2 can exchange Ethernet frames.

2.	The TSN AF triggers PDU Session Modification procedure to setup a QoS flow to transmit all gPTP messages, e.g. Announce messages, Sync messages.

[Optional] If the operator decides to follow the recommendation to limit the bridge residence time to 10 ms, as defined in IEEE 802.1AS [6], clause B2.2 (stated as recommendation), then, the PCF  sets up QoS parameter (i.e. PDB) to fulfil DS-TT to UPF delay less than 5 ms, such that the total limit is achieved while there is no need for the PCF to learn if this concerns a DS-TT/UE to NW-TT/UPF communication or a DS-TT/UE to DS-TT/UE communication.

NOTE 1:	The PDB value for the case of dedicated synchronization communication (i.e. a QoS flow dedicated for transmitting gPTP messages) is pre-configured by the operator at the PCF.

3.  At BMCA step, TSN nodes exchange Announce messages and build the Sync forwarding tree for TSN Sync messages. The Sync forwarding tree is per TSN working clock domain. BMCA procedure (6.1.3.3) is performed when a NW-TT port or a DS-TT port receives an Announce frame.

4.	Sync message is delivered from the previous TSN node to DS-TT/UE1. It has time stamp of TSN GM clock. It also includes correction and rateRatio fields. The rateRatio is (GM clock frequency) / (local clock frequency).

5.	DS-TT/UE1 updates the Sync message. It adds the link delay between the DS-TT/UE1 and the previous TSN node to the correction field. It also updates the rateRatio field with the previous rateRatio multiplied by neighborRateRatio. The neighborRateRatio = (local clock frequency of the previous TSN Node) / (local clock frequency). It also attaches Ingress Timestamp based on 5G GM clock to the Sync message.

6.	Sync message is delivered from DS-TT/UE1 towards NW-TT/UPF. It has time stamp of TSN GM clock. It also includes correction, rateRatio and Ingress Timestamp fields.

7.	NW-TT regenerates the Sync message. It calculates the residence time as Egress 5G GM Time - Ingress 5G GM TS) * rateRatio in the sync message. It updates the correction field as the previous correction field value + the residence time. Then, it removes the Ingress timestamp.

8.	Sync message is delivered from the NW-TT to TSN Node 2. It has time stamp of TSN GM clock. It also includes correction and rateRatio fields.

The procedure for synchronizing TSN end stations behind other UE(s) with the TSN GM in the network attached to the device is described in Figure 6.1.3-2. It also includes BMCA-related steps (step 1~3). After those steps, the sync message forwarding tree is confirmed. In this case, TSN Node 0 should send sync messages to 5GS TSN Bridge, and the 5GS TSN Bridge should send sync messages to TSN Node 3. The remaining steps (step 4~8/8a) follow principles in clause 5.27.1 of TS 23.501 [2], with the Ingress TT of DS-TT1 and the Egress TT of DS-TT2.

Figure 6.1.3.2-2: Procedure for synchronizing TSN end stations behind other UE(s) with the TSN GM in the network attached to the device side via 5G System

1.	PDU Session Establishment includes UE MAC / Port Info for updates to TSN AF. After this step, TSN Node 0 and 5GS logical bridge (DS-TT1/UE1, NW-TT/UPF) can exchange Ethernet frames.

1a.	For UE-to-UE case, PDU Session Establishment includes UE MAC / Port Info for updates to TSN AF. After this step, TSN Node 3 and 5GS logical bridge (DS-TT2/UE2, NW-TT/UPF) can exchange Ethernet frames. Local switching at the UPF is also enabled.

2.	At BMCA step, TSN nodes exchange Announce messages and build the Sync forwarding tree for TSN Sync messages. The Sync forwarding tree is per TSN working clock domain. BMCA procedure (6.1.3.3) is performed when a NW-TT port or a DS-TT port receives an Announce message.

3.	[Optional] If the operator decides to follow the recommendation to limit the bridge residence time to 10 ms, as defined in IEEE 802.1AS [6], clause B2.2 (stated as recommendation), then Based on the Sync frame forwarding information the TSN AF may trigger QoS setup for the Sync frame delivery. PDB1, which is the UL PDB for the QoS flow of PDU session for UE1, should be less than 10ms, as the residence time in a node should be less than 10ms.

3a.	[Optional] If the operator decides to follow the recommendation to limit the bridge residence time to 10 ms, as defined in IEEE 802.1AS [6], clause B2.2 (stated as recommendation), then, Based on the Sync frame forwarding information, the TSN AF may trigger QoS setup. As the TSN AF can know DS-TT1/UE1 forwards Sync frames to DS-TT2/UE2 via local switching at the UPF, it sets the QoS should meet the sum of PDB1, local switching delay and PDB2 should be less than 10ms. PDB2 is the DL PDB for the QoS flow of PDU session for UE2. For this, the TSN AF may trigger the QoS change for PDU session for UE1.

NOTE:	Calculation of the UE-UE delay can be consistent with the UE-UE TSC communication solution chosen.

NOTE 2:	The PDB value for the case of dedicated synchronization communication (i.e. a QoS flow dedicated for transmitting gPTP messages) can be pre-configured by the operator at the PCF. In that case, the PCF  sets up QoS parameter (i.e. PDB) in all cases fulfilling DS-TT to UPF delay less than 5 ms, such that the total limit is achieved.

NOTE:	Based on BMCA results, the TSN AF informs the SMF of the associated PDU sessions. That is, the TSN AF informs the SMF of the PDU Session for the DS-TT port with Slave role and the PDU Session for the DS-TT port with Master role.

4.	Sync message is delivered from the previous TSN node to DS-TT/UE1. It has time stamp of TSN GM clock. It also includes correction and rateRatio fields. The rateRatio is (GM clock frequency) / (local clock frequency).

5.	DS-TT/UE1 regenerates the Sync message. It adds the link delay between the DS-TT/UE1 and the previous TSN node to the correction field. It also updates the rateRatio field with the previous rateRatio multiplied by neighborRateRatio. The neighborRateRatio = (local clock frequency of the previous TSN Node) / (local clock frequency). It also attaches Ingress Timestamp based on 5G GM clock to the Sync message.

6.	Sync message is delivered from DS-TT/UE1 towards NW-TT/UPF. It has time stamp of TSN GM clock. It also includes correction, rateRatio and Ingress Timestamp fields.

6a.	For UE-to-UE case, the NW-TT/UPF performs local switching and forwarding for the Sync message. Before local switching, the NW-TT/UPF can perform multiplication if needed.

6b.	For UE-to-UE case, Sync message is delivered from the NW-TT/UPF to DS-TT/UE2. It has time stamp of TSN GM clock. It also includes correction, rateRatio and Ingress Timestamp fields.

7a.	For UE-to-UE case, DS-TT/UE2 updates the Sync message. It calculates the residence time as Egress 5G GM Time - Ingress 5G GM TS) * rateRatio in the sync message. It updates the correction field as (the previous correction field value) + (the residence time). Then, it removes the Ingress timestamp.

8a.	For UE-to-UE case, Sync message is delivered from the DS-TT/UE2 to TSN Node 3. It has time stamp of TSN GM clock. It also includes correction and rateRatio fields.

6.1.3.3 BMCA procedure

Method 1:

When a NW-TT port or a DS-TT port receives an Announce message, if the Announce information is better than the current best master information it knows about (per clock domain), the port reports the Announce information to TSN AF using PMIC signaling. NW-TT and DS-TT also inform TSN AF if a previously reported best clock is not available anymore (determined based on lack of Announce messages from that clock). Based on the received Announce information, the TSN AF decides the BMCA port roles for the ports in the 5GS bridge as described in IEEE 802.1AS-Rev [6] and configures each port role (per clock domain) accordingly using PMIC signaling. The port closest to the GM is configured to Slave role, the port receives Sync and Announce messages from a node outside 5GS and sends the Sync and Announce messages to other port(s) with Master role(s) inside 5GS. The port that is configured to Master role, receives Sync/Announce messages from the port with Slave role inside the 5GS bridge and sends Sync/Announce messages to a node outside the 5GS.

NOTE 1:	Depending on the solution to expose the time syncronization service, the BMCA port roles can be assigned either by TSN AF or NEF.

Method 2:

In this alternative, NW-TT receives Announce messages from any NW-TT and DS-TT ingress port (DS-TT forwards all Announce messages to NW-TT) and performs BMCA to determine the GM (per clock domain). The exchange of Announce messages inside 5GS can be minimized by DS-TT’s or NW-TT’s sending Announce messages only when there are changes comparing with the previous messages. The changes may include Announce message recipt timeout events. Upon completion of BMCA, NW-TT reports the BMCA result to TSN AF using BMIC signaling. Based on this, TSN AF configures the port roles in DS-TT using PMIC signaling. If TSN AF is not deployed then NEF terminates PMIC and BMIC signaling, i.e. NEF receives the BMCA result from NW-TT and configures configures the port roles in DS-TT using PMIC signaling.

NOTE 2:	NEF terminating BMIC/PMIC signaling does not have any impact on PCF/SMF as existing Rel-16 functionality is reused for this.

Port roles of NW-TT ports are configured locally by NW-TT based on the BCMA result. When a DS-TT configured to Master role receives Sync/Announce messages from the NW-TT via 5GS user plane, it sends Sync/Announce messages to nodes connected to DS-TT. When DS-TT port configured to Passive role receives a Sync message from outside of 5GS, it discards the message.

6.1.4	Impacts on services, entities and interfaces

For synchronizing TSN end stations behind 5G System (NW-TT) with the TSN GM in the network attached to the device, the impacts on UE, SMF, PCF, TSN-AF and UPF are like the following.

-	The Ingress TT is DS-TT.

-	The Egress TT is NW-TT.

For synchronizing TSN end stations behind other UE(s) with the TSN GM in the network attached to the device side via 5G System, the impacts on UE, SMF, PCF, TSN-AF and UPF are like the following.

-	The Ingress TT is DS-TT of a UE.

-	The Egress TT is DS-TT of the other UE.

-

-	[Optional] The TSN AF needs to trigger QoS setup procedures in case the operator decides to follow the recommendation to limit the bridge residence time to 10 ms.

-	The UPF needs to perform local switching for Sync messages without NW-TT interaction.

*** End Change ***