SA WG2 Meeting #140E (e-meeting)	S2-2005769

# Aug 19 – Sep 01, 2020, Elbonia

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.502 | CR | 2394 | rev | - | Current version: | 16.5.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction on Nnrf_NFManagement_NFStatusSubscribe service operation |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Tencent |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G_eSBA |  |  |  |  |  | Date: |  |  | 2020-07-20 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | For the Nnrf_NFManagement_NFStatusSubscribe service operation, some required input paramaters are actually optional input parameters and multully exclusive the stage 3 spec. - The NF Type, NF instance ID, NF service are optional input parameters in stage 3 but required input parameters in stage 2. They should be the optional input parameters. - In stage 3, the following parameters are optional and multually exclusive: NF Type, NF instance ID, NF service name, NF Set, NF Group, SCP domain, GUAMI(s) and AMF set or AMF region. The multually exclusive requirements should be clear in stage 2 spec. NF Set, NF Group, SCP domain should be added according to multually exclusive parameters. - Some existing parameters related with NWDAF and NEF in the optional input parameters are not existed in stage 3 multually exclusive parameters yet. It is proposed to add these two to the multually exclusive parameters. - In stage 3, callback URI is the required input parameter. So it is proposed to add callback URI as a required input parameter. - In stage 3, the Validity Time is an optional input parameter and a required output parameter. So it is proposed to add Validity Time in stage 2. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Remove NF Type, NF instance ID, NF service from required input parameters to the optional input parameters to align with stage 3. - To align with stage 3, it is proposed to add the full list of multually exclusive paramters to the optional input parameters in stage 2. - It is proposed to add the existing optional input parameters related with NWDAF and NEF to the multually exclusive parameters. - Adding callback URI as a required input parameter as defined in stage 3. - Adding Validity Time as an optional input parameter and a required output parameter. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | - Some parameters in Nnrf_NFManagement_NFStatusSubscribe service operation are not aligned with stage 3. - The parameters related with NWDAF and NEF are missing in the multually exclusive parameters. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.2.7.2.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

| This CR's revision history: |  |
| --- | --- |

**************** First Change ****************

##### 5.2.7.2.5	Nnrf_NFManagement_NFStatusSubscribe service operation

Service Operation name: Nnrf_NFManagement_NFStatusSubscribe.

Description: Consumer can subscribe to be notified of the following:

-	Newly registered NF along with its NF services.

-	Updated NF profile.

-	Deregistered NF.

Inputs, Required: callback URI

Inputs, Optional:

-	PLMN ID of the target NF/NF service, in the case of the subscription to the status of NF/NF service instance(s) in home PLMN from the serving PLMN.

-    Validity Time, in case to indicate the time instant after which the subscription becomes invalid.

-	For updated NF profile subscription, conditions that trigger a notification from NRF. Includes monitored attributes in the NF profile (changes trigger a notification) or unmonitored attributes in the NF profile (changes do not trigger a notification)

-    Following parameters are mutually exclusive:

-    NF Type (if NF status of a specific NF type is to be monitored).

-    NF Instance ID or NF Instance ID list (if NF status of a specific NF instance or a list of NF instance is to be monitored).

-    NF Service name (if NF status for NF which exposes a given NF service is to be monitored).

-    NF Set (if NF status of a set of NF Instances belonging to a certain NF Set is to be monitored).

-    NF Service Set (if the status of a set of NF Service Instances belonging to a certain NF Service Set is to be monitored).

-    NF Group (if the NF status of NF Instances identified by a NF (UDM, AUSF, PCF, CHF, HSS or UDR) Group Identity is to be monitored).

-   SCP Domain (if the status of NF or SCP instances belonging to a certain SCP domain is to be monitored).

-	For the UPF Management defined in clause 4.17.6: UPF Provisioning Information as defined in that clause.

-	For AMF, Consumer may include list of GUAMI(s), or AMF Set or AMF Region.

-	S-NSSAI(s) and the associated NSI ID(s) (if available).

-	For NWDAF, Consumer may include Analytics ID(s) and TAI(s). Details about NWDAF discovery and selection are described in clause 6.3.13, TS 23.501 [2].

-	For NEF, Consumer may include Event ID(s) provided by AF.

Outputs, Required: When the subscription is accepted: Subscription Correlation ID (required for management of this subscription), Validity Time.

Outputs, Optional: None.

NOTE:	Alternatively, other means such as OA&M can also be used to subscribe for NF status.

**************** END CHANGES **************