3GPP TSG-WG SA2 Meeting #140E e-meeting 	S2-2005390

Elbonia, August 19 – September 01, 2020	(revision of S2-200xxxx)

Source:	Huawei, HiSilicon

Title:	KI#5 Sol#16: Update to Solution #16 for roaming support

Document for:	Approval

Agenda Item:	8.4

Work Item / Release:	FS_eNS_Ph2 / Rel-17

Abstract: This contribution proposes roaming support updates to Solution#16.

1. Introduction/Discussion

In both LBO and HR roaming cases, the PDU session is associated to an S-NSSAI of the HPLMN. The UPFs in VPLMN enforce UL/DL bitrate quota according to the corresponding HPLMN S-NSSAI and related quota for LBO roaming. The UPFs in HPLMN enforce UL/DL bitrate quota for HR roaming in the similar way with non-roaming case.

2. Text Proposal

It is proposed to capture the following changes vs. TR 23.700-40.

# * * * * First change * * * *

6.16	Solution #16: Slice data rate enforcement and dynamic adjustment

6.16.1	Introduction

This solution is for Key Issue #5, "Dynamic adjustment to meet the limitation of data rate per network slice in UL and DL".

6.16.2	High-level Description

The main idea of the solution is that UPFs get quotas of UL/DL bitrate allowed to transfer and enforce them to meet the limitation of the slice. The primary PCF of the slice (i.e. Slice PCF) collects the actual bitrate in N6 interface per S-NSSAI from the UPFs and periodically adjusts the bitrate quota allocation according to the latest service distribution within the slice.

The SMF provides S-NSSAI of the PDU Sessions to UPF in N4 session. The UPF enforces the UL/DL bitrate quota across all GBR and Non-GBR QoS Flows of the PDU Sessions with the S-NSSAI as follows:

1)	The UPF shall ensure that the traffics of GBR QoS Flows do not exceed the sum of GFBR value of the respective QoS Flows. Whenever a GBR QoS Flow is established or modified, the UPF shall check if the sum of the GFBR values (including the new/modified QoS Flow) is larger than the bitrate quota value of the S-NSSAI. If so, the UPF rejects the new/modified QoS Flow, and then SMF may try to pre-empt any existing GBR QoS Flow within the slice based on the ARP values or rejects the new/modified QoS Flow.

2)	The UPF shall ensure that the aggregated bitrate across all GBR and Non-GBR QoS Flows is not exceeding the UL/DL bitrate quota value of the S-NSSAI by shaping the traffic of non-GBR QoS Flows and/or dropping packets exceeding the GFBR from GBR QoS Flows with MFBR>GFBR. The details of the UL/DL bitrate quota enforcement are implementation-specific.

NOTE:	Existing queue management mechanisms will handle the packet dropping. The ratio of packet shaping and packet dropping is up to operator configuration.

In LBO roaming case, the UPFs in VPLMN enforce UL/DL bitrate quota according to the corresponding HPLMN S-NSSAI and related quota.

In HR roaming case, the UPFs in HPLMN enforce UL/DL bitrate quota same as non-roaming case.

6.16.3	Procedures

6.16.3.1 Slice data rate enforcement at UPFs

The following figure represents a high-level procedure of the solution.

Figure 6.16.3-1: A high-level procedure of the solution

1.	During N4 association setup or update, the UPF which terminates N6 interface indicates the S-NSSAIs it supports to SMF. The SMF checks whether slice bitrate enforcement is necessary. If so, the SMF asks the UPF to do the throughput measurement and reporting for the indicated S-NSSAI periodically.

2.	The UPF measures the N6 interface UL/DL throughputs for the indicated S-NSSA and reports the average N6 interface UL/DL bitrate per S-NSSAI and the PDU session number of the slice to the Slice PCF via the SMF.

3.	The Slice PCF allocates UL/DL bitrate quota of the slice to the involved active UPFs for the S-NSSAI if it is needed.

NOTE 1:	The 1st time allocation of the bitrate quota of the slice to the UPF can be delayed until the 1st PDU Session is established via this UPF.

4.	A PDU Session within the slice has been established/modified.

5.	During the PDU Session establishment procedure, for the N4 interaction to configure the related N4 rules, the SMF provides the S-NSSAI of the PDU Session to the UPF. The UPF checks if the sum of the GFBR values (including the new/modified QoS Flow) is larger than the bitrate quota value of the S-NSSAI. The UPF also enforces the UL/DL date rate allowed to transfer through the N6 interface within the slice by shaping the traffic of related non-GBR QoS Flows and/or dropping packets exceeding the GFBR from related GBR QoS Flows with MFBR>GFBR.

NOTE 2:	The UL data rate enforcement could be more relaxed based on operator's policy as the received UL packet has already consumed the radio resources.

6.	The Slice PCF collects the actual bitrate and PDU session number report of each involved UPF in the slice periodically (e.g. per 10 minutes).

7.	The Slice PCF makes adjustments on the bitrate quota allocation between involved active UPFs. Because of the PDU session number reported, the Slice PCF may avoid allocating excessive bitrate quota to a UPF with very large throughput but few PDU Sessions.

8.	If need, the Slice PCF updates the bitrate quota allocated to each involved UPF according to the latest service distribution within the slice.

6.16.3.2 Slice data rate enforcement at UPFs in roaming

For the HR roaming, the UPFs in HPLMN enforce UL/DL bitrate quota same as non-roaming case.

For the LBO roaming, the AMF sends the corresponding S-NSSAI of the HPLMN from the Mapping of Allowed NSSAI to the SMF during the PDU Session establishment procedure. The UPF (PSA) in VPLMN enforce UL/DL bitrate quota according to the corresponding HPLMN S-NSSAI and related Quota.

The enhancement to the procedure defined in 6.16.3.1 is as following:

Step 1: During the N4 association setup/update, per the S-NSSAI reported by the UPF, the SMF check whether slice bitrate enforcement is necessary for the corresponding HPLMN S-NSSAI. If so, the SMF asks the UPF to do the throughput measurement and reporting for the indicated (HPLMN, HPLMN S-NSSAI) periodically.

Step 2: The UPF communicate with the Slice PCF, which is located in VPLMN and associated with the related (HPLMN, HPLMN S-NSSAI).

Step 3: When the slice PCF allocate the quota to the UPF, it also indicate the allocated quota is associated with (HPLMN, HPLMN S-NSSAI).

Depending on the operator’s deployment and SLA agreement, there two options on who manage the quota. Option A:  Quota is provided to VPLMN. In this case the Slice PCF allocates the quota by itself directly.

Option B:  Quota is managed by the HPLMN. In this case the Slice PCF need communicate with the Slice PCF at the HPLMN and applies the quota from HPLMN.

Step 5: During the PDU establishment, the SMF provide the (HPLMN, HPLMN S-NSSAI) associated with this PDU Session to the UPF.

Step 7: Depending on who manage the Quota, the Quota management either be done by the Slice PCF in VPLMN directly or it need involve the Slice PCF in HPLMN.

6.16.4	Impacts on services, entities and interfaces

PCF:

-	If bitrate control is needed for the slice, the Slice PCF allocates UL/DL bitrate quota to each of the involved active UPFs for the S-NSSAI or for (HPLMN, HPLMN S-NSSAI) in LBO roaming cases.

-	The Slice PCF collects the actual UL/DL bitrate of the slice from each involved UPF and updates the bitrate quota allocation to each active UPF according to the latest service distribution within the slice.

SMF:

-	Provide S-NSSAI or (HPLMN, HPLMN S-NSSAI) in case of LBO roaming in each N4 session establishment procedure to enable the PSA UPF to associate the traffic to a slice.

-	Relay quota allocation and actual bitrate report messages between the UPF and the Slice PCF.

-	Configure the UPF to periodically report the actual UL/DL bitrate of the slice.

UPF:

-	Enforce the bitrate quota allowed to transfer across all GBR and non-GBR QoS Flows per S-NSSAI or  (HPLMN, HPLMN S-NSSAI) in case of LBO roaming.

-	Measure the N6 interface UL/DL throughputs of per S-NSSAI.

-	Provide the actual UL/DL bitrate per S-NSSAI or per (HPLMN, HPLMN S-NSSAI) to the SMF per SMF instruction.

-	Check if the sum of the GFBR values (including the new/modified QoS Flow) is larger than the bitrate quota.

6.16.X	Evaluation

The evaluation of the solution is described based on the following criteria.

- Scalability:

The solution allows a scalable solution to support a single instance or multiple instances of quota enforcement NFs (UPF) without degradation in performance of the solution for slice data rate enforcement.

- Reusability:

The solution reusability for other Key Issues is very high. The PCF is a generic slice SLA Quota control management NF, which can apply for KI#1/ KI#2/KI#5. So similar the quota distribution service can be re-used in all three cases.

In addition, the PCF based solution maintains the global quota status of a slice SLA attribute. So that it can support the event notification of Network Slice quota of the slice SLA attribute to the consumer by covering KI#4.

- Accuracy of Quota Control and Enforcement:

The enforcement is based on actual data rate measurement, rather than authorized maximum data rate per UE or per session. It controls the actual slice data rate more accurately.

By providing quota control and management mechanisms by slice PCFs and UPF, 5GS can overcome inaccurate quota control and enforcement, for example, a quota enforcement NF may accept the requests over the limit of the global quota as the enforcement NF do not receive the status of global quota has been consumed before the UEs’requests have been accepted.

- Rapid Response of Quota enforcement:

The UPF allows instantaneous decision for an acceptance or rejection of the UEs’ requests based on the status of local quota of a slice SLA attribute during the PDU Session establishment/modification procedures.

Also the UPF can do the rapid decision, e.g. shaping/dropping packet per the current throughput of the traffic, in a timely manner.

# * * * * End of changes * * * *