SA WG2 Meeting #141E	S2-2007557

Electronic meeting, Oct 12 – Oct 23, 2020	(Revision of S2-200nnnn)

Source:	Nokia, Nokia Shanghai Bell

Title:	KI#1 Update of solution #5

Document for:	Approval

Agenda Item:	8.9

Work Item / Release:	FS_5MBS / Rel-17

Abstract of the contribution:

This paper updates solution #5 to clarify the case when MSF is slit into MSF-U and MSF-C, to clarify the presence of MSF-U in the architecture and to add the option of multicast tunnelling on N3.

# 1	Discussion

Solution 5 includes a note in which MSF-C is described as a mandatory NF in the architecture. The role of MSF-U is not clearly described. This contribution proposes to clarify the role of MSF-U and made the presence of MSF-U in the architecture conditional to whether the AF requests a user service [see 26.346] or a broadcast bearer service.

Solution 5 also currently does not describe the procedure when multicast tunnelling on N3 is used. The proposed changes in this document allows the N3 to use either multicast or unicast tunnelling.

# 2	Conclusions

Proposal: Agree the proposed changes to Solution #5.

Proposed changes to TR 23.757

## 6.5	Solution #5: Broadcast Session Start

### 6.5.1	Functional description

This solution addresses key issues 1 and 5 and enhances the existing 5GC architecture to provide broadcast communication services. The 5GC is responsible to manage the broadcast session based on, e.g., broadcast communication service information from AF. The baseline architecture 1 in clause A.1 is assumed.

For the broadcast session the MSF-C in architecture 1 is required. It can be also be implemented within the NEF. The presence of MSF-U in the architecture could be conditional. If the goal is to provide User Services as defined in [26.346] and offered to the AF via xMB then MSF-U is required. The 5GS could provide more basic broadcast bearer services, for which the MSF-U may not be required, to realize the broadcast communication services.

When the AF requests the 5GS to provide a broadcast communication service, the AF provides to the MSF-C directly or via NEF the broadcast communication service related information, a list of TAIs and a list of NR CGIs which defines broadcast service area, QoS requirement information. The AF can provide the broadcast service area as an array of strings, which syntax is left to the business agreement between content provider (the AF) and the operator and could also include a civic address. The MSF-C shall determine a list of TAIs and a list of NR CGIs from the broadcast service area information. The MSF-C discovers the involved SMF using the NRF services to support the requested service based on e.g. the service identifier, DNN, slicing information, or SMF capabilities. The MSF-C uses the SMF services to create the broadcast context for the broadcast session start providing the service identifier, the list of TAIs, the list of NR CGIs, QoS profiles etc. The involved SMF creates broadcast policy association and get authorization for the QoS profile from the PCF. The involved SMF queries the NRF for the AMF type instances to discover the AMFs serving the TAIs from the list of TAIs. The query may include the TAIs received from the MSF-C or the SMF filters the result themselves and selects the involved AMF(s). The SMF invokes an AMF service (e.g. non-UE specific N2 information transfer) to send a broadcast session resource setup message including the TAIs to the involved NG RAN nodes. The SMF subscribes for non-UE specific N2 information notifications to receive notification about success or failure of N2 operations. The AMF(s) select the involved NG RAN nodes based on the received TAIs and initiates N2 procedure, BC session resource setup, providing the NG RAN nodes with the TAIs, NR CGIs and QoS flows. The SMF selects the involved PSA UPF and other UPF(s) according to the service identifier. The involved PSA UPF is configured with the IP multicast address used by MSF-U, QoS Flows and, if unicast N3 tunnelling is used, DL TLN information.

There is one PSA UPF targeting N6 for one broadcast service, and the PSA UPA is responsible to distribute the broadcast traffic to multiple UPFs or NG RANs within the broadcast service area over N9 or N3 tunnels configured by the SMF.

### 6.5.2	Procedures

Figure 6.5.2-1: Broadcast Session Start procedure

0.	If a broadcast service is provided in association with a unicast service from a service provider (e.g. GC AS) perspective, the UE reports NR CGI and TAI to the AF.

1.	The AF requests the MSF (i.e. MSF-C) directly or via NEF to start one specific broadcast communication service, providing assistance information, MBS service identifier, a list of TAIs and a list of NR CGIs or a broadcast area that is mapped to a list of TAIs and a list of NR CGIs by the MSF, QoS requirements, DNN, NSSAI etc.

2.	The MSF creates the broadcast service and selects broadcast session parameters including BC Session ID, a global broadcast bearer identity (e.g. TMGI) and, if User Services (e.g. File delivery) are requested by the AF, IP multicast address(es) and other (e.g. FEC). Then the MSF initiates the start of broadcast session.

3.	The MSF sends BC service start response to the AF informing the AF about the determined broadcast session parameters.

4.	If the UE is not preconfigured with needed configuration (e.g. USD), the UE is provisioned with the configuration of broadcast session which may be performed using SIP signalling or methods described in TS 26.346 [18] including announcement over a broadcast session for service announcements.

Editor's note:	More information on radio-level broadcast session identifiers, e.g. TMGIs, to be added in coordination with RAN.

5.	If the MSF is not preconfigured with the information about the involved SMF, the MSF uses the NRF services to discover the SMF based on provided DNN, NSSAI and SMF's capabilities stored in the NRF.

6.	The MSF requests Nsmf_BCSession_CreateBCContext (IP multicast address, TAIs, NR CGIs, BC Session ID, S-NSSAI, [PCF ID]) service to create broadcast context in the SMF.

7.	The SMF may establish BC Policy Association and get PCC rules for the BC session which may be identified by multicast IP address.

8.	The SMF invokes the discovery service of NRF and queries for AMF type instances. The SMF may use TAI parameter for filtering.

9.	The NFR returns a search result with the AMF instances.

10.	Depending on the filtering criteria, the SMF may further filter the results and select the AMFs.

11.	The SMF uses the Namf_NonUEN2MessageTransfer (TAIs, N2InfoContainer) service of the AMFs to transfer BC Session Resource Setup Request message to the NG RAN nodes controlling the target cells where the broadcast communication service should be provided.

If transport layer IP Multicast (i.e. multicast on N3) is used, the SMF further allocates the DL tunnel information and includes the DL tunnel information in Namf_NonUEN2MessageTransfer message.

12.	The SMF subscribes for non-UE specific N2 information notifications to receive notification about success or failure of N2 operations. A new N2InformationClass may be created for broadcast related information.

13.	The AMF selects the involved NG RAN based on the TAIs.

14-15.	The AMF sends BC Session Resource Setup Request (TAIs, NR CGIs, QoS flows, a global broadcast bearer service identity (e.g. TMGI)) to the NG RAN nodes. If the NG RAN accepts the request and if unicast tunnelling on N3 is used, the NG RAN allocates the DL tunnel address information for the BC session. The NG RAN then responds to the AMF.

If transport layer IP Multicast (i.e. multicast on N3) is used, the BC Session Resource Setup Request includes the DL tunnel information. The NG RAN should establish the DL tunnel by joining the IP Multicast (identified by the IP multicast address and the IP address of the multicast source as described by the DL tunnel information) to enable reception of broadcast communication service data.

Editor's note:	More information on possible radio-level announcement of available broadcast service and related radio-level broadcast session identifiers, e.g. TMGIs, to be added in coordination with RAN.

16.	The AMF notifies the SMF about N2 information including n2InfoContainer carrying the BC Session Resource Setup Response.

17.	The SMF establishes N4 session with the UPFs, including, if unicast N3 is used, the DL tunnel information, QoS parameters. The SMF indicates that allocation of an ingress resources (i.e. ingress transport address and port) is needed. The UPF allocates the ingress resources. The multicast data received at the ingress resources will be forward to the RAN in corresponding QoS flows within the shared DL tunnel.

18.	The SMF notifies the MSF-C about successful setup of broadcast session resources. For User Services, the MSF-U is configured with the UPF’s ingress resources (i.e. the ingress transport address and port).

19.	The MSF notifies to the AF the status of the broadcast communication service. For Broadcast Bearer Services, the SMF-U is not used and the MSF notifies the ingress transport address and port to the AF.

20.	For User Services, the data is delivered, e.g. using the user plane procedures of xMB [29.116], from the AF to the MSF (i.e. MSF-U) which is configured for the User Service. In case the MSF-C and the MSF-U are not collocated, PFCP is assumed to be used on the reference point between the MSF-C and the MSF-U.

21. For User Services, the MSF (i.e. MSF-U) sends User Service data to the UPF(s) using the ingress transport address and port.

22. For Broadcast Bearer Service, the AF sends broadcast service data using the ingress transport address and port over N6 to the UPF.

23.	The UPF forwards the received data in QoS flows of a shared N3 tunnels according to the configuration received in step 17.

24.	The RAN transmit the multicast PDUs as a user payload of radio broadcast bearer over the air.

### 6.5.3	Impacts on services, entities and interfaces

In order to support this solution, the involved network entities need to support the following functionality.

MSF:

-	Receives the broadcast communication service information from AF, e.g. service identifier, TAIs and NR CGIs or a service area that can be mapped to TAIs and NR CGIs, DNN, NSSAI, and service requirements.

-	Determines the broadcast area and the involved SMF for the target broadcast service.

-	Selects broadcast session parameters including BC Session ID, a global broadcast bearer service identity (e.g. TMGI) and, if User Services (e.g. File delivery) are requested by the AF, may allocate IP multicast address(es) and other (e.g. FEC).

PCF:

-	Determines the policy and QoS profile for the broadcast service.

-	Authorization management.

SMF:

-	Receives the broadcast session information from the MSF, e.g. broadcast session identifier (BC Session ID),

-	Establishes broadcast policy association to obtain QoS profile

-	Determines the involved PSA UPF and other UPFs;

-	Determines the involved AMF(s);

-	Manage the N3/N9 tunnels for the broadcast service.

AMF:

-	Support the broadcast session information transfer between the SMF and the NG RAN.

-	Determines the involved RAN nodes for the target broadcast communication service based on TAIs.

PSA UPF:

-	The anchor UPF connects to broadcast service based DNN.

-	Distributes the broadcast traffic to the involved UPFs configured by SMF.

-	Enforce broadcast QoS rule.

-	Select the ingress resources (i.e. an ingress transport address and port).

NG RAN node:

-	Allocate the N3 tunnel address for the broadcast session.

-	Transmitting broadcast traffic with satisfying the QoS requirements.