SA WG2 Meeting #141E (e-meeting)	S2-2007561

Oct 12 – Oct 23, 2020

Source:	Ericsson

Title:	KI #13 evaluation and interim conclusion

Document for:	Approval

Agenda Item:	8.1

Work Item / Release:	FS_eNA_ph2 / Rel-17

Abstract of the contribution: This contribution proposes evaluation and interim conclusion for KI#13.

- Discussion

For Key Issue #13 "Triggering conditions for analytics", there are 7 solutions, here we propose a classification while leaving out of this discussion the propose definition of new Analytics:

- Definition on the triggers for requesting analytics: Solution #45: Triggers for requesting analytics and solution #74: Triggers for requesting analytics.

- Dynamically configuration of triggers to request Analytics, provided by the AF: Solution #73: Trigger configuration for data collection and analytics and Solution #75: AF influencing NFs triggers for interactions with NWDAF.

- Triggers to collect data from the UE: Solution #65: Triggers for data collection from the UEs

On the definition of triggers for requesting analytics, both solution#45 and #74 complement the existing procedures defined in 23.502 to show when NWDAF interaction is needed. This should be defined in normative work,

On the dynamic configuration of the triggers to request Analytics by either the PCF or the UDM, possibility based on information provided by the AF. In our view, there is no use case defined in TR 23.700-91, nor requirements on the KI description on the reason to motivate that the Application Function knows which Analytics should be consumed by the AMF and SMF or PCF and whether those should be used at the UE registration or at the PDU session establishment or modification.  Note that some examples of use cases have been provided for this SA2 meeting, without any previous discussion on whether those are applicable for this release. Therefore the existing solution defined in Rel-16, and part of Solution #45 and Solution #74 is that local policies (i.e. either OAM defined per DNN, or S-NSSAI or per Inter-Group-Id or any internal logic) covers the key issue description and requirements:

Based on the above analysis, the interim conclusion for KI#13 is proposed as follows:

-	Regarding triggers used by analytics consumers to request for or subscribe to analytics from the NWDAF, the existing solution to have local policies in the AMF, SMF or PCF are valid and enough to cover this key issue.

-	Regarding triggers used by the NWDAF for data collection from the UE we propose that solution #65 is discussed together with Key issue#8.

-	Regarding configuration of triggers used by the NWDAF for data collection from the NFs, NWDAF is configured by OAM; on the relevant data to be collected per Analytics.

-	Regarding definition on the triggers for requesting analytics solution #45 and #74 should be selected for normative work.

# 2. Proposal

It is proposed to include the following changes in TR 23.700-91.

***** Start of Changes (all new text) *****

## 7.X	Key Issue #13: Triggering conditions for analytics

For Key Issue #13 "Triggering conditions for analytics", there are 7 solutions, here we propose a classification while leaving out of this discussion the propose definition of new Analytics:

- Definition on the triggers for requesting analytics: Solution #45: Triggers for requesting analytics and solution #74: Triggers for requesting analytics.

- Dynamically configuration of triggers to request Analytics, provided by the AF: Solution #73: Trigger configuration for data collection and analytics and Solution #75: AF influencing NFs triggers for interactions with NWDAF.

- Triggers to collect data from the UE: Solution #65: Triggers for data collection from the UEs

On the definition of triggers for requesting analytics, both solution#45 and #74 complement the existing procedures defined in 23.502 to show when NWDAF interaction is needed and should be defined in normative work.

On the dynamic configuration of the triggers to request Analytics by either the PCF or the UDM, possibility based on information provided by the AF. In our view, there is no use case defined in TR 23.700-91, nor requirements on the KI description on the reason to motivate that the Application Function knows which Analytics should be consumed by the AMF and SMF or PCF and whether those should be used at the UE registration or at the PDU session establishment or modification.  Note that some examples of use cases have been provided for this SA2 meeting, without any previous discussion on whether those are applicable for this release. Therefore the existing solution defined in Rel-16, and part of Solution #45 and Solution #74 is that local policies (i.e. either OAM defined per DNN, or S-NSSAI or internal logic) covers the key issue description and requirements.

***** Next Change *****

## 8.X	Conclusion for Key Issue #13: Triggering conditions for analytics

Based on the above analysis, the interim conclusion for KI#13 is proposed as follows:

-	Regarding triggers used by analytics consumers to request for or subscribe to analytics from the NWDAF, the existing solution to have local policies in the AMF, SMF or PCF are valid and enough to cover this key issue.

-	Regarding triggers used by the NWDAF for data collection from the UE that solution #65 is discussed together with Key issue#8.

-	Regarding configuration of triggers used by the NWDAF for data collection from the NFs, NWDAF is configured by OAM; on the relevant data to be collected per Analytics.

-	Regarding definition on the triggers for requesting analytics solution #45 and #74 should be selected for normative work.

***** End of Changes *****