3GPP TSG-SA2 Meeting #141-E	S2-2007191

Elbonia, October 12 – October 23, 2020	(revision of S2-2005470)

Source:	Samsung

Title:	KI#1 overall evaluation proposal

Document for:	Approval

Agenda Item:	8.1

Work Item / Release:	FS_eNA_Ph2/ Rel-17

Abstract of the contribution: This contribution proposes an evaluation for KI#1.

# 1. Background

This paper proposes an evaluation on the key aspects of the NWDAF functional decomposition discussed in solutions adopted in the TR for Key Issue 1.

# 2. Text Proposal

It is proposed to adopt the following text within the TR.

*** Start of the change (all new text) ***

## 7.1	Key Issue #1: Logical decomposition of NWDAF and possible interactions between logical functions

The different solutions proposed for Key Issue #1 present alternatives for NWDAF functional decomposition. A total of nine solutions address Key Issue #1, but it is worth noting that some of these solutions also address other closely related Key Issues, namely Key Issue #2, Key Issue #11 and Key Issue #19.

Key aspects in the evaluation of Key Issue #1 are the following:

-	The number of entities decomposing NWDAF and their functionality.

-	The standardization effort required to support the decomposition.

Hence, in performing the evaluation the above two aspects are determined for all proposed solutions.

Table 7.1-1 proposes an evaluation framework for Key Issue #1 indicating NWDAF decomposed functionality and required standardization support. The following clarifications on Table 7.1-1 apply:

- The table columns show all potentially decomposable NWDAF functionality mentioned across the proposed solutions, but not all solutions necessarily refer to all functionality mentioned in Table 7.1-1. When the functionality is not mentioned in the solution description, ‘-’ applies.

- When a solution addresses other key issues in addition to Key Issue #1, it is mentioned in the first column of Table 7.1-1.

- When the decomposed functionality is mentioned but it is proposed to be part of NWDAF, the name of the corresponding NWDAF sub-function is shown by its acronym in parenthesis (e.g. NWDAF (AF)).

- Some solutions do not explicitly refer to a model repository functionality, but they assume pre-trained models can be stored in and retrieved from certain entity. If that is the case, such entity is shown performing model repository functionality in Table 7.1-1.

- The standardization impact of any decomposed functionality shown in Table 7.1-1 refers to its related interfaces. Hence, it is shown whether any of the interfaces related to each decomposed functionality requires additional standardization impact (*), does not require additional standardization impact (**) or is out of the scope of 3GPP (***). Data collection is not analysed in this respect as it should be done in the scope of KI#11.

Table 7.1-1: Summary of NWDAF functional decomposition solutions

| Solution |  | Data collection [KI#11] | Model repository | Training | Inference | Analytics exposure |
| --- | --- | --- | --- | --- | --- | --- |
| 5 (KI #19) |  | NWDAF | AIF*/ NWDAF (AIF)* | AIF*/ NWDAF (AIF)* | NWDAF** | NWDAF** |
| 6 | Opt. 1 | NWDAF data repository | ML designer* | NWDAF (TF)* | NWDAF (AF)* | NWDAF (AF)** |
|  | Opt. 2 |  | ML designer*** |  |  |  |
| 7 |  | NWDAF (DC) | - | NWDAF (TF)* | NWDAF (IF)* | NWDAF (IF)** |
| 8 |  | NWDAF (DC) | - | MT*/ NWDAF (MT)* | NWDAF (AC)** | NWDAF (AC)** |
| 9 (KIs #2, #11) |  | DCCF, Data repository | - | - | - | NWDAF (AF)** |
| 35 (KIs #2, #11) |  | DCNF | - | - | - | NWDAF** |
| 39 (KIs #2, #11) |  | DRSF | - | - | - | NWDAF** |
| 56 (KIs #2, #19) |  | - | - | NWDAF* | NWDAF* | - |
| 69 (KIs #2 ,#11) |  | NWDAF | - | - | - | NWDAF** |

Based on Table 7.1-1, the following statements can be made regarding NWDAF functional decomposition:

-	Analytics exposure is proposed to continue to be hosted by NWDAF by all solutions referring to this functionality. No specific standardization support is required by any solution for this purpose.

-	All solutions describing both training and inference functionality (Sols #5, #6, #7, #8, #56) allow for NWDAF to host it.

-	Inference functionality and analytics exposure functionality are provided by the same function.

-	Three solutions describing both training and inference (Sol#6, Sol#7, Sol#8) propose two separate sub-entities within NWDAF to carry out each functionality.

-	All solutions describing training functionality (Sols #5, #6, #7, #8, #56) assume standardization impact for training support.

-	Three solutions describing inference functionality (Sols #6, #7, #56) assume standardization impact for inference support. Two solutions describing inference functionality (Sols #5, #8) assume no standardization impact for inference support.

-	Two solutions describe functional entities for model repository (Sols #5, #6). Sol#5 assumes standardization impact and Sol#6 proposes two options with both standardization impact and absence thereof.

NOTE 1:	Whether a dedicated entity for data collection is required and whether specific standardization support for that entity is required is in the scope of KI#11.

NOTE 2:	The sharing of trained data models among NWDAF instances is in the scope of KI#19, so any decision on training and inference decomposition needs to be compatible with KI#19 conclusion and vice versa.

NOTE 3: Table 7.1-1 does not explicitly mention NRF impacts, but any required standardization impact in functional entities involving NRF is marked with ‘*’.

- *** End of the change ***