3GPP TSG-WG SA2 Meeting #141E e-meeting 	S2-200xxxx

Elbonia, October 12 – 23, 2020

Title:	[Draft] LS on Clarification on URLLC QoS Monitoring measurement period and reporting frequency

Response to:

Release:	Rel-16

Work Item:	5G_URLLC

Source:	SA2

To:	RAN WG3, CT WG4

Cc:	SA WG5, RAN WG2

Contact Person:

#### Name:	Susan Shi

E-mail Address:	susan.shishufeng@huawei.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	None

1. Overall Description:

SA2 discussed one CR S2-2007141 during their SA2#141E meeting and SA2 realized that there was misunderstanding of SA2’s answer provided in the reply LS S2-2003468 to the Q2 raised in RAN3’s LS R3-201372/S2-2002659 regarding “Does SA2 assume a measurement period is required for the RAN part of delay measurement?”, and would like to provide the following clarification:

SA2 has agreed in their specification, i.e. step 3b in the subclause of 4.3.3.2 of TS 23.502, excerpted below

3b.	For SMF requested modification, the SMF invokes Namf_Communication_N1N2MessageTransfer ([N2 SM information] (PDU Session ID, QFI(s), QoS Profile(s), [Alternative QoS Profile(s)], Session-AMBR, [CN Tunnel Info(s)], QoS Monitoring indication, QoS Monitoring reporting frequency, [TSCAI(s)]), N1 SM container (PDU Session Modification Command (PDU Session ID, QoS rule(s), QoS Flow level QoS parameters if needed for the QoS Flow(s) associated with the QoS rule(s), QoS rule operation and QoS Flow level QoS parameters operation, Session-AMBR))).

The SMF provides the QoS Monitoring reporting frequency IE to the RAN, and RAN is required to report the RAN part of delay result according to the QoS Monitoring reporting frequency. This reporting frequency can be used by RAN to determine the packet delay measurement frequency of RAN part. The measurement period of RAN part of delay is up to RAN’s implementation. Therefore, SA2 assumes that the QoS Monitoring reporting frequency defined in TS 23.502 is enough for RAN to enforce the QoS Monitoring, and an additional measurement period, apart from reporting frequency, is not required for the RAN part of delay measurement.

2. Actions:

To RAN WG3 and CT WG4

ACTION:

SA2 kindly asks RAN3 and CT4 to take the above information into consideration and update their specifications accordingly.

3. Date of Next TSG-SA WG2 Meetings:

TSG-SA2 Meeting #142		16-20 November			Electronic meeting