SA WG2 Meeting #141e	S2-2007337r22

Electronic Meeting, October 12 – 23, 2020

Source:	Convida Wireless LLC, Telecom Italia, Ericsson, Lenovo, Motorola Mobility, NEC

Title:	KI #7, Evaluation and Conclusion

Document for:	Approval

Agenda Item:	8.4

Work Item / Release:	FS_eNS_ph2 / Rel-17

Abstract of the contribution: This p-CR proposes a Key Issue #7 Evaluation and Conclusion Section.

# 1	Discussion

This paper provides an evaluation and conclusion for the five Key Issue #7 solutions.

# 2	Proposal

This contribution proposes to implement the following updates to TR 23.700-40 v1.0.0.

* * * Start of Change 1 * * *

## 7.x	Evaluation for Key Issue #7

Editor's note: 	This clause will need further updates based on RAN WGs feedback. This clause will need further update taking in consideration new solutions added in SA2#141e.

The main issue to be resolved in KI#7 is how the 5GS steers UEs to a 5G-AN (e.g. a specific frequency band supported by the UE) that supports the network slices that the UE can use. Solutions 17, 29, 30, 31 and 44 are for Key Issue #7. All solutions are compatible with assumption that each cell in the tracking area supports the same S-NSSAI(s).

Editor's note: 	Whether different cells within a tracking area can support different S-NSSAIs can be supported by the cells in the tracking area is based on RAN WGs feedback.

Solution 17, 29, and 30 focus on the Registration procedure while solutions 31 and 44 focus on the PDU session establishment procedure.

Solution 17 allows the UE to request registration for S-NSSAIs outside of the Allowed NSSAI. The 5GC then determines which slices are preferred, and steers the UE towards the preferred S-NSSAIs, or rejects the requested slice(s). Thus, the solution assumes that the network always can decide which network slices to register without explicitly knowing what network slice(s) the user prefers. This might not always be the case.

Editor’s note:	Whether there are use cases where the 5GC can not determine suitable which slices to register for a UE requesting slices that cannot be used at the same time due to the slices are not defined to be available in the same frequencies the user prefers is FFS.

Solution 29 also allows the UE to request registration for S-NSSAIs outside of the Allowed NSSAI. The UE is provided with frequency band information per network slice in the Configured NSSAI. When the UE wants to register for ato slice(s) the UE may performs cell selection/reselection based on the received frequency band information of thatthe slice(s). It is then understood that the UE will only request slices that are accessible simultaneously in the selected cell and the AMF will include in the Allowed NSSAI only S-NSSAI(s) that are accessible simultaneously. Solution 29 has an impact on the UE.

Solution 30 also allows the UE to request registration for S-NSSAIs outside of the Allowed NSSAI. The UE is provided with frequency band information per network slice in the Configured NSSAI. Each of a slice’s target carrier frequencies can also be associated with a priority. The UE uses this information during cell selection in Idle mode when the UE wants to use a network slice (as in solution 29). In Solution 30, the UE is allowed to register to multiple network slices which are accessible on different preferred frequency band(s) within the Registration Area. The solution mentions that secondary cell (e.g. Dual Connectivity) can be used in some scenarios, but this assumption is not valid in all scenarios (e.g. if the UE is registered to 3 slices that are not all available in the on a common operating band or if the UE is not capable of NR/NR DC). Solution 30 has an impact on the UE.

Editor's note: 	Impacts on the cell selection/reselection mechanisms would require feedback from RAN WGs and CT1.

Solution 31 and solution 44 propose mechanisms to steer UEs to a 5G-AN (e.g. a specific frequency band) during the PDU Session establishment procedure. Both solutions assume that the network can be configured with preferred frequency band information for each slice but this information is not provided to the UE. The difference is in solution 31 the AMF decides to steer the UE to a target RAN Node while in solution 44 the RAN decides to steer the UE to the target RAN Node.

Solution 44 doesn’t have any impact on the system. Solution 44 does not handle the case where the operator manages a different range of radio spectrums for some network slices in the same Tracking Area (i.e. some network slices cannot be accessed via certain frequencies).

Solutions 30 and 31 do not explain how to handle the case where the UE attempts to maintain PDU Sessions in slices that are not all accessible in the same operating band. Both solutions propose to optionally use NR/NR Dual Connectivity in order to use different frequency bands simultaneously when the UE supports NR/NR Dual Connectivity. This is not possible in all scenarios (e.g. if the UE is registered to 3 slices that are available on a common operating band in the primary cell or if the UE is not capable of NR/NR DC).

Table 7.x-1 summarized some differences between solutions 17, 29, 30, 31, and 44.

Table 7.x-1: Key Issue #7 Solution Comparison

| # | Criteria | Solution 17 | Solution 29 | Solution 30 | Solution 31 | Solution 44 |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | When slice(s), to which the UE wants to register, are not available in a current cell, is the UE able to select a different cell/band that supports the slice(s)? (i.e. Is the UE aware of what slices are supported by each operating band?) | No UE can select a slice not available in current cell and get re-directed to wanted slice/TA | Yes Supported bands are provided in the Configured NSSAI. | Yes Supported bands of the Serving PLMN are provided in the Configured NSSAI. | No Slice availability based on UP resources | No Slice availability based on UP resources |
| 2a | Can the UE have slices in its Allowed NSSAI that are not all available on some common operating band(s)Can the UE be registered to slices that are not available in the frequency band of the current/primary cell.? FFS: The need of this Criteria depends on RAN feedback. | No | No | Yes | No | No |
| 2b | Does the solution allow the UE to simultaneously use all slices in the Allowed NSSAI (i.e. simultaneously maintain PDU Sessions in all of the Allowed slices per TS 23.501 [2], clause 5.15.5.1.1)? FFS: The need of this Criteria depends on RAN feedback. | Yes | Yes | Yes Valid if all S-NSSAIs of the Allowed NSSAI operate common frequency band (e.g. in the primary cell). | Yes | Yes |
| 3 | How does 5GS steer UEs to a 5G-AN (e.g. a specific frequency band) that can support the network slices that the UE wants to use. | RAN Redirects the UE when it detects, as per NSSAI and RFSP provided by AMF, that the UE wants to access a slice that is not available in the current band. | The UE is told what operating bands will work with each slice. The UE in Idle mode selects a cell in the frequency band of the requested NSSAI. The network can consider the Allowed NSSAI when determining what cell to handover the UE to The network can consider the Allowed NSSAI when configuring the UE’s cell selection priorities. | The UE is told what operating bands will work with each slice. The UE in Idle mode selects a cell in the frequency band of the requested NSSAI. The network can consider the Allowed NSSAI when determining what cell to handover the UE to The network can consider the Allowed NSSAI when configuring the UE’s cell selection priorities. | RAN steers the UE based on the requested S-NSSAI provided by the AMF in the PDU Session Establishment Request procedure. | RAN steers the UE based on the requested S-NSSAI provided by the SMF in the PDU Session Establishment Request procedure, and when activating UP resources. |
| 4 | What information does the 5G-AN need to steer UE to a proper 5G-AN. | RAN uses the Target NSSAI and corresponding RFSP from the AMF | Existing Mechanisms (i.e. RFSP Index and the RAN can consider the Allowed NSSAI) | Existing Mechanisms (i.e. RFSP Index and the RAN can consider the Allowed NSSAI) | RAN uses the S-NSSAI provided by the AMF in the PDU Session Establishment Request procedure | RAN uses the S-NSSAI provide by the SMF in the PDU Session Establishment Request procedure and Existing Mechanisms (i.e. RFSP Index and the RAN can consider the Allowed NSSAI) for subsequent steering. |
| 5 | What information should be provided to the UE to select a proper 5G-AN and how it is sent to the UE. | No new information is sent to the UE. | Supported bands are provided in the Configured NSSAI. | Supported bands of the Serving PLMN are provided in the Configured NSSAI. | No new information is sent to the UE. | No new information is sent to the UE. |
| 6 | RAN Impacts | Support getting additional Target NSSAI and RFSP Index for the Target NSSAI, and support moving the UE accordingly. | None, the Allowed NSSAI is already provided to the RAN via N2. | None, the Allowed NSSAI is already provided to the RAN via N2. | RAN uses the S-NSSAI provided by the AMF in the PDU Session Establishment Request procedure to steer the UE. | None, the Requested NSSAI is already provided to the RAN via N2 SM message. |
| 7 | UE Impacts | No | Yes | Yes (UE needs to support DC) | No (UE needs to support DC) | No (UE needs to support CA/DC) |
| 8 | Can the UE change registration to network slices in CM-CONNECTED state? | YesYes Assumes AMF supports the slices or can re-allocate to new AMF | YesTBD | TBD | Yes | YesYes |

Editor's note: this table will need to be updated with new solutions from SA2#141e.

* * * Start of Change 2 * * *

## 8.x	Conclusion for Key Issue #7

Editor's note:	The following is an interim conclusion for the KI#7 and are subject to change dependent on RAN WGs feedback, and on evaluation of the new solutions agreed in SA2#141e..

The existing capabilities of the 5GS, e.g. the ability to steer UEs to certain frequencies based on RFSP, Allowed NSSAI and activated UP, together with a suitable resource partitioning of the NG-RAN resources, enable the 5GS to support the case where the network operator prefers that certain network slices use certain frequencies (i.e. certain network slices may get dedicated resources by NG-RAN resource partitioning in preferred frequencies). However, such mechanism relies on a possible partitioning of resources of a cell which is implementation specific and can’t be generalized by the standardization.

.

Existing capabilities of the 5GS do not fully support the case where certain frequencies cannot be used to access a slice, in particular as described in clause 5.7 "how to select a particular cell that can be used to access the network slice(s) when the operator manages a different range of radio spectrums per network slice".

-	Existing (Rel-15/16) 5GS behaviour is that when the UE attempts to simultaneously register with slices that are not available in a common operating band, it is assumed that the some requested slices will be not allowed based on the network policies. The UE then can retry with a different Requested NSSAI if the current Allowed NSSAI is not suitable for its needs. This can result in several trials and errors till a stable state between UE and network is achieved. Or, if the S-NSSAI not allowed is provided as a Rejected S-NSSAI for the RA, the UE may wait to request the S-NSSAI until the UE moves out of the RA.

The following interim conclusions are agreed:

-

Editor's note:	As per the statement GSMA statement "some terminals might be restricted in terms of frequencies to be used", the attribute may instead be understood as the vertical's terminal radio frequency support. It is FFS to clarify the GSMA's intention.

*** End of Changes ***