SA WG2 Meeting #141-e 	S2-2006850r03

October 12 – October 23, 2020, Online

(Merging S2-2006851, S2-2007116 and S2-2007390)

Source:	China Telecom

Title:		KI#12_Solution Evaluation for NWDAF-assisted RFSP policy

Document for:	Approval

Agenda Item:	8.1

Work Item / Release:	FS_eNA_Ph2 / Rel-17

Abstract of the contribution: Solution Evaluation for 23.700-91-050 of eNA Key Issue #12

# 1 Proposal

The aim of key issue 12 is to address the use case #1 NWDAF-assisted RAT/frequency selection. It is to study:

-	The kind(s) of information is made available to the NWDAF for the data analytics, e.g. the information collected for assisting RFSP index) and from which provided NFs.

-	The kind(s) of information should the NWDAF output to the NWDAF consumer as the data analytics.

-	The method(s) for NWDAF consumer of analytics, such as PCF, uses the NWDAF analytics to enhance the RFSP index value assigned to a UE.

It proposes to document the following texts into TR 23.700-91.

* * * * Start of Changes * * * *

# 7	Overall evaluation

## 7.X	Evaluation of solutions for Key Issue #12 - NWDAF-assisted RFSP policy

For Key Issue #12 – “NWDAF-assisted RFSP policy” five Candidate Solutions have been proposed. Candidate solutions #4, #40, #41, #42 and #43 all serve the purpose of providing analytics information by NWDAF to support NFs to assist on RAT and frequency selection. Solution #43 is merged into Solution #4.

Candidate Solution #4 proposes that the RFSP index value is assigned by PCF, as the consumer of NWDAF, using both the analytics result of “Service in use” and “Observed Service experience". “Service in use” is a new defined analytics in which NWDAF collects input data from the AF (possibly via NEF) to determine the applications a UE consumes. It also proposes to enhance the input of “Observed Service Experience” with “subscriber category” from UDR/PCF and RAT/Frequency information from OAM.

Candidate Solution #40 has no standardization impacts, as it proposes to use the output of User Data Congestion Analytics as defined in Clause 6.8 of TS 23.288 to a service consumer to derive a suitable RFSP index. All input/output/procedure in this solution are suggested as same as described in clause 6.8 of TS 23.288. However, RFSP selection should consider more than the congestion experience aspect, e.g. MoS of service experience and other evolving R17 study items aspects. These  could be regarded as complementary solutions.

Candidate Solution #41 suggests NWDAF provides a new analytic, “Radio/Frequency usage”, on information about the current/future usage of RAT/frequency for a specific area. Based on this analytics, PCF can decide to assign the RFSP index value to serve the usage demand per RAT/frequency. But it is unclear how the NWDAF determines these analytics results based on the input information which does not include any per RAT/frequency related information.

Candidate Solution #42 contains a functional diagram on what information from NFs and OAM could help NWDAF to perform analytics and generate statistics/prediction outputs related to RFSP index to NF consumers. It’s proposing a high-level framework and it needs to provide more detailed explanation before evaluation.

* * * * End of Changes * * * *