SA WG2 Meeting #140-e 	S2-2006851r01

October 12 – October 23, 2020, Online

(Merging S2-2006851, S2-2007116 and S2-2007390)

Source:	China Telecom

Title:		KI#12_Solution Evaluation and Interim Conclusion for NWDAF-assisted RFSP policy

Document for:	Approval

Agenda Item:	8.1

Work Item / Release:	FS_eNA_Ph2 / Rel-17

Abstract of the contribution: Interim conclusion for 23.700-91-050 of eNA Key Issue #12

# 1 Proposal

The aim of key issue 12 is to address the use case #1 NWDAF-assisted RAT/frequency selection. It is to study:

-	The kind(s) of information is made available to the NWDAF for the data analytics, e.g. the information collected for assisting RFSP index) and from which provided NFs.

-	The kind(s) of information should the NWDAF output to the NWDAF consumer as the data analytics.

-	The method(s) for NWDAF consumer of analytics, such as PCF, uses the NWDAF analytics to enhance the RFSP index value assigned to a UE.

It proposes to document the following texts into TR 23.700-91.

* * * * Start of Changes * * * *

# 8.  Conclusions

## 8.X 	Conclusion on solutions for Key Issue #12: NWDAF-assisted RFSP policy

Interim conclusion:

It is proposed to select  Solution #4,  for normative work for KI#12.The input data for service experience analytics is to reuse the ones defined in clause 6.4, TS 23.288 [5] with exception that Table 6.4.2-3: UE level Network Data from OAM is extended to further retrieve RAT/Frequency information related to the service experience observation period from OAM in order to allow NWDAF to be able to provide Service Experience per RAT type and Frequency.

And Solution #40 can be a complementary solution.

NOTE: NWDAF does not collect subscriber category as input for Observed Service Experience Analytics.

* * * * End of Changes * * * *