SA WG2 Meeting #141E (e-meeting)	S2-2006863r02

October 12 – October 23, 2020, Elbonia	(revision of S2-200xxxx)

Source:	LG Electronics

Title:	KI #3, Evaluation and Conclusion

Document for:	Approval

Agenda Item:	8.6 Study on Access Traffic Steering, Switch and Splitting support in the 5G system architecture Phase 2

Work Item / Release:	FS_ATSSS_Ph2 / Rel-17

Abstract of the contribution: This paper proposes evaluation and conclusion for KI#3.

# Background

# Proposal

It is proposed to capture the following proposal to TR 23.700-93.

* * * * Start 1st Change * * * *

# 7	Evaluation

Editor's note:	This clause will provide a general evaluation of the solutions.

## 7.3	Key Issue #3: Supporting MA PDU with 3GPP access leg over EPC and Non-3GPP access leg over 5GC

There are three solutions (Solution #5, #9 and #10) for KI#3.

All solutions are based on 5G RG solution in Rel-16. More in detail:

-	Solution #5 describes the mobility procedure from 3GPP access over 5GC to 3GPP access over EPC;

-	Solution #9 describes capability negotiation aspect between UE and CN and extends the usage of the Rel-16 solution to any N3GPP access (not only limiting it to W-5GAN);

-	Solution #10 extends the usage of the Rel-16 solution to support Ethernet PDU Session type.

Because all solutions are based on 5G RG solution, there is no significant impact to the MME or SGW.

These three solutions are complementary and needed for Rel-17 as it is very likely that there will be areas where 3GPP radio supports only LTE connected to EPC. Thus, ATSSS service continuity should be ensured in these areas.

Solution #10 suggests to fully reuse the procedure in TS 23.316 clause 4.12.3 without any enhancement. Some functionalities such as MPTCP or some steering modes may not be supported by this solution.

* * * * Start 2nd Change * * * *

# 8	Conclusions

Editor's note:	This clause will capture conclusions from the study.

## 8.3	Key Issue #3: Supporting MA PDU with 3GPP access leg over EPC and Non-3GPP access leg over 5GC

### 8.3.1	Interim Conclusion

It is proposed that Solutions #5, #9 and #10 are selected as a baseline for normative work.

Editor’s note: It is FFS whether Solution#10 is applicable to MPTCP or some steering modes.

* * * * End of Change * * * *