3GPP TSG-WG SA2 Meeting #141E e-meeting 	S2-2006990r04

Elbonia, October 12 – 23, 2020	(revision of S2-200xxxx)

Source:	Huawei, HiSilicon

Title:	KI #6: Conclusion

Document for:	Approval

Agenda Item:	8.9

Work Item / Release:	FS_5MBS / Rel-17

Abstract: This contribution contains the conclusion of KI#6.

1. Introduction/Discussion

According to the evaluation in S2-2006990, it is proposed to document the related interim conclusion for KI #6.

2. Text Proposal

It is proposed to capture the following changes vs. TR 23.757.

S2-2006898 (vivo) and S2-2006983 (LGE) are merged into this paper.

# * * * * First change (All Text New)* * * *

8.X	Conclusions for Key Issue #6: Local MBS service

8.X.1	Interim conclusions

Editor's Note: This conclusion may not be complete, additional concept/details/characteristics could be added.

The following interim agreements to support local MBS services apply as baseline for normative work:

-	It shall be possible to support MBS session where the content is not location-dependent, but the distribution is limited to a certain area, and MBS sessions where the content is location-dependent.

-	It shall be possible to locate MBS CN functions in proximity to NG RAN nodes serving the location area.

-	For the case that the multicast service is only available within a limited area, the UE shall be able to obtain service area information of the multicast service, to enable the UE to trigger the session join procedure only within the location area.

-	For the case that the MB service content is location-dependent (The content of the same MB service delivered to different sub-areas is different), it shall be possible that the UE is not aware of the available area information of the MB service.

-	It shall be possible to hide the internal PLMN topology from AF (e.g., AF is not aware of cell or TA information).

-	The network shall be able to enforce the area restriction for accessing a local MBS service.

-	The network shall be able to support multiple ingress nodes for an MBS session.

Editor's Note: The conclusion of KI#6 should align with the conclusion of KI#1.

Editor's Note: Alignment based on progress/decision in RAN WGs will be performed as part of the normative phase.

# * * * * End of changes * * * *