3GPP TSG-WG SA2 Meeting #141E	S2-2007024r013

Oct 12 – Oct 23, 2020, Elbonia	(revision of S2-2005399, merge of S2-2007385 and S2-2007580)

Source:	China Mobile, CATT

Title:	KI #1: Interim conclusions

Document for:	Approval

Agenda Item:	8.1

Work Item / Release:	FS_eNA_ph2 / Rel-17

Abstract: This contribution proposes interim conclusions on Key Issue #1.

# 1. Introduction

This contribution proposes interim conclusions on KI #1: Logical decomposition of NWDAF and possible interactions between logical functions.

# 2. Text Proposal

It is proposed to capture the following new text in TR 23.700-91.

* * * * Start of Changes * * * *

# 8	Conclusions

Editor's note:	This clause is intended to list conclusions that have been agreed during the course of the study item activities. This should also capture the guiding principles and documentation approach for creating CRs to normative specifications within the responsibility of SA WG2.

## 8.1	Key Issue #1: Logical decomposition of NWDAF and possible interactions between logical functions

Interim conclusion:

An NWDAF can be decomposed into Model Training and Analytics logical functions.

-	A Rel-17 NWDAF containing the Model Training logical function trains ML models and exposes training service (e.g. providing trained model to the NWDAF);

-	A Rel-16 and onward NWDAF containing the Analytics logical function performs inference by deriving analytics and exposes analytics service (i.e. the Nnwdaf_AnalyticsSubscription or Nnwdaf_AnalyticsInfo as defined in clause 7, TS 23.288).

NOTE 1: Model Training Functionality and Analytics Functionality can be deployed as standalone NWDAFs.

NOTE 2:	It can also be possible to identify other standalone NF(s) based on other KI results, e.g. Data Dollection function. These conclusions do not address possible NWDAF decomposition for data collection as this is part of KI#11.

NOTE 3: Messaging framework can be used between the decomposed functions from KI#11

NOTE 4: The definition of services and their associated parameters provided by the Training functionality needs to be aligned with the conclusion of Key Issue #19: Trained data model sharing between multiple NWDAF instances.

Pre-trained ML model storage and provisioning to NWDAF is out of the scope of 3GPP.

Model Training logical function may register the supported ML model information to NRF.

NOTE 5:	The details of ML model registration in NRF and impacts the sharing of trained data model between multiple NWDAF instances so it needs to be aligned with the conclusion of Key Issue #19: .

* * * * End of Changes * * * *