SA WG2 Meeting #141E e-meeting	S2-2007032r02

Elbonia, October 12 – 23, 2020	(revision of S2-200xxxx)

Source:	China Mobile

Title:	KI#2: Evaluation on exposing UE mobility analytics for multiple NWDAFs case

Document for:	Approval

Agenda Item:	8.1

Work Item / Release:	FS_eNA_Ph2/ Rel-17

Abstract of the contribution: This contribution proposes an interim evaluation and Interim conclusion for KI#2 regarding exposing UE mobility analytics.

1. Discussion

Sol#25 in the TR 23.700-91 address the exposing UE mobility analytics aspect of KI #2 in the multiple NWDAF instances deployment scenario.

-Solution #25: Exposing UE mobility analytics for multiple NWDAFs case

This contribution paper proposes to evaluate and conclude the solutions for the exposing UE mobility analytics aspect of KI #2.

2. Text Proposal

It is proposed the following changes to TR 23.700-91 [2].

# * * * * First change * * * *

7	Overall Evaluation

Editor's note:	This clause will provide evaluation of different solutions.

7.x  Key Issue #2

7.x.x	Exposing UE mobility analytics for multiple NWDAFs case

Solutions #25 proposes a solution of exposing UE mobility analytics for multiple NWDAFs. To obtain UE mobility analytics in Source Area of Interest and Target Area of Interest in target period, the NWDAF serving Source Area of Interest could trigger procedure to the NWDAF serving Target Area of Interest to figure out the SUPI list and their related mobility analytics, who once stayed in Target Area of Interest in target period.

# * * * * End of changes * * * *