3GPP TSG-SA/WG2 Meeting #141E	S2-2007128r02

12 -23 October 2020, Electronic meeting

Source:	Ericsson

Title:	KI #1, , Sol #12: Updates to the solution

Document for:	Agreement

Agenda Item: 	8.3

Work Item / Release:	FS_enh_EC / Rel-17

Abstract of the contribution: This contribution updates Solution #12: DNS-triggered re-anchoring, addressing the EN.

# 1	Introduction

This contribution provides updates to Solution #12: DNS-triggered re-anchoring proposed for Key Issue #1: Discovery of Edge Application Server, Key Issue #2: Edge Relocation and Key Issue #5: Activating the traffic routing towards Local Data Network per AF request. The main changes include updating the solution to support external LDNSR or LDNSR in UPF and by this to answer the relevant editor notes in the evaluations and conclusions related to solutions for KI#1 and distributed anchor connectivity model.

This tdoc revision does not deal with § 6.12.2.1 whose revisions are handled in Tdoc 7770

# 2	Proposal

It is proposed to have the following changes in TR 23.748:

* * * * 1st Change * * * *

## 6.0	Mapping of Solutions to Key Issues

Table 6.0-1: Mapping of Solutions to Key Issues

| Solutions | Key Issues |  |  |  |
| --- | --- | --- | --- | --- |
|  | 1 | 2 | 3 | 5 |
| #1: Provisioning URSP configuration to the UE to establish PDU Sessions for edge applications | X |  |  |  |
| #2: Local DNS based edge server address discovery | X |  |  |  |
| #3: DNS AF | X |  |  |  |
| #4: Providing the DNS authoritative server with IP addressing information about where the UE is located | X |  |  |  |
| #5: Server Discovery using DNS, IP Routing and URSP | X |  |  |  |
| #6: Discovery of EAS based on DNS | X |  |  |  |
| #7: SMF/I-SMF selection based on DNAI | X |  |  |  |
| #8: Edge Application Server discovery using anycast DNS | X |  |  |  |
| #9: Assist DNS resolution without connectivity between local and central data network | X |  |  |  |
| #10: DNS for Distributed Anchor | X |  |  |  |
| #11: DNS over HTTP | X |  |  |  |
| #12: PDU session re-anchoring | X | X |  | X |
| #13: 5GC support for UE selection of the DNS to use | X |  |  |  |
| #14: IP address discovery for the Service Switch mechanism- DNS handling in both UPF and EC | X |  |  |  |
| #15: IP address discovery for the Service Switch mechanism-DNS handling in UPF | X |  |  |  |
| #16: Edge Configuration Server Based Discovery | X |  |  |  |
| #17: Provisioning EC Parameters including EAS information to the UE | X |  |  |  |
| #18: Mapping the AS IP address to Edge Server IP address | X |  |  |  |
| #19: Edge Application Server discovery using an Address Resolution Function | X |  |  |  |
| #20: DNS Inspector based EAS Discovery | X |  |  |  |
| #21: Provisioning URSP configuration to the UE to establish PDU Sessions for edge applications based on Provisioning Domains | X | X |  |  |
| #22: DNS based EAS discovery supporting session breakout. | X |  |  |  |
| #23: DNS for AS Discovery at Edge Relocation |  | X |  |  |
| #24: Support of edge relocation, triggering of new DNS query by the UE |  | X |  |  |
| #25: Seamless Change of Edge for Stateful Applications |  | X |  |  |
| #26: Persistent address allocation for mobile UEs that need MEC access |  | X |  |  |
| #27: Reducing packet loss during EAS relocation |  | X |  |  |
| #28: Supporting application server change based on AF notification | X | X |  |  |
| #29: CN-based edge relocation |  | X |  |  |
| #30: UE Agnostic EAS IP address replacement for traffic subject to edge computing |  | X |  |  |
| #31: Application Relocation with UE assistance |  | X |  |  |
| #32: UE DNS cache flush | X | X |  |  |
| #33: IP preserving PSA relocation |  | X |  |  |
| #34: Local DN notification to the UE during ULCL operations |  | X |  |  |
| #35: Edge relocation considering with user plane latency requirement (SMF decision) |  | X |  |  |
| #36: Edge relocation considering with user plane latency requirement (AF decision) |  | X |  |  |
| #37: AF-based EAS End-Point-Address update via External Parameter Provisioning |  | X |  |  |
| #38: EAS change with reducing packet loss in uplink |  | X |  |  |
| #39: EAS relocation coordinated with PSA change |  | X |  |  |
| #40: Seamless change of Edge Application Sever for stateful applications by caching application status information in NEF |  | X |  |  |
| #41: Network Information Provisioning using the IP path |  |  | X |  |
| #42: Providing selected radio information to an App requiring it |  |  | X |  |
| #43: Low Latency exposure API by using the distributed CAPIF framework feature |  |  | X |  |
| #44: Network Information Exposure to Local AF with Low Latency |  |  | X |  |
| #45: Using AS or NAS message notify UE's application layer |  |  | X |  |
| #46: Local NEF Deployment for network information exposure to Local AF with Low Latency |  |  | X |  |
| #47: User Plane based Network Information Provisioning |  |  | X |  |
| #48: QoS monitoring information exposure based on unstructured data transmission mechanism |  |  | X |  |
| #49: Network Information Provisioning to EAS with low latency based on User Plane |  |  | X |  |
| #50: Activating the traffic routing towards Local Data Network per AF request |  |  |  | X |
| #51: Edge Relocation for all connectivity models |  | X |  |  |
| #52: Service Continuity at EAS relocation with PSA coexistence in session break-out scenarios |  | X |  |  |
| #53: Service Continuity at Edge Relocation with DNS triggered insertion of BP/ULCL and Edge PSA |  | X |  |  |
| #54: EAS relocation for SSC mode 3 PDU Session |  | X |  |  |
| #55: Multiple AFs |  | X |  |  |
| #56: Edge NEF based Network Information Provisioning |  |  | X |  |

* * * * 2nd Change  * * * *

## 6.12	Solution #12: PDU session re-anchoring

### 6.12.1	Solution description

This solution addresses the Key Issue #1: Discovery of Edge Application Server (EAS), Key Issue #2: Edge Relocation and Key Issue #5: Activating the traffic routing towards Local Data Network per AF request. The UE is Edge Computing Service agnostic.

The proposed solution supports the "Distributed Anchor Point" connectivity model. It is assumed that at PDU session establishment the SMF selects a central PSA, but local PSA is available to the SMF, and re-anchoring is used to transition to the "Distributed Anchor Point" model.

Re-anchoring could happen based on different triggers, e.g.:

1.	The UE issues a DNS query related to an EC service. The SMF will get information from LDNSR that a local PSA is needed.

NOTE:	The placement of LDNSR will decide the exact procedure. This is to be defined in another.

2.	Application AF initiates a traffic influence request to PCF (through the NEF in the case of external AF), upon which the PCF initiates a routing request for the PDU session to the SMF. Based on the requested target DNAI the SMF initiates re-anchoring.

3.	UE mobility cases which imply re-anchoring.

1. and 2. above are related to KI#1 and 3, is related to KI#2.

The following re-anchoring scenarios can be differentiated:

a	Support of KI#1, Re-anchoring without change of SMF where the SMF has full control over the re-anchoring. This is the case when the re-anchoring can be done using one of the following existing 5GC procedures for PSA change:

-	SSC mode 3 with IPv6 Multi-homed PDU Session (clause 4.3.5.3 of TS 23.502 [3]).

-	SSC mode 3 with multiple PDU Sessions (clause 4.3.5.2 of TS 23.502 [3]), without SMF Reallocation.

b	Re-anchoring with re-selection of SMF where the SMF may have or may not have full control over the selected re-anchored DNAI.

In b, old SMF sends "use SMF (set) for next PDU session" or "use DNAI for next PDU session" indication to the AMF in the Namf_Communication_N1N2MessageTransfer message. The AMF uses this information when selecting an SMF for the new PDU session.

Example use cases of re-anchoring where AMF does re-selection of SMF (the existing or new SMF) that could use the described functionality is listed below.

1.	KI#1, The selected DNAI is supported by SMF but only SSC mode 2 is supported.

2.

2.	KI#5, Requested DNAI is outside the SMF service area. PDU session is SSC#2 or SSC#3.

3.	KI#2, due to mobility, the current DNAI is not valid and a new DNAI is needed and the new UE location is outside the SMF service area. PDU session is SSC#2 or SSC#3.

### 6.12.2	Procedures

#### 6.12.2.1	EAS Discovery triggered re-anchoring without change of SMF, KI#1

Procedure for KI#1. The trigger for re-anchoring is the discovery (DNS) for EC server by UE and 5GS has already been configured with the EC AS-FQDN that may trigger a re-anchoring. The PDU Session supports SSC#3 or SCC#2 mode. The SMF supports Edge PSAs. The solution uses LDNSR as further described in solution 22  and a.

The solution is described by by referring to solution 22 and telling the difference due to SMF initiating SSC mode 2: mode 3 instead of inserting an UL CL / BP.

This is shown in Figure 6.12.2.2.1-1 below.

Figure 6.12.2.1-1 Re-anchoring without change of SMF during Edge Application Server Discovery

0.	PDU Session establishment. The UE PDU session is established via SMF with UPF1 (central PSA). 5G Core existing mechanisms are used to guarantee that, if for that user PDU session Edge Computing can be applied, the UE DNS queries to EC AS-FQDN are sent to the LDNSR. One such method is e.g., sending to the UE at PDU session establishment, the LDNSR address as DNS server in the PCO field. will trigger a re-anchoring

1.	The UE does a DNS discovery request to discover the EAS. The DNS request is (at least for EC AS-FQDNs) handled by the LDNSR.

Editor’sNote:	Steps 1 and 2 assume DNS query is triggering the procedure for the SMF to re-anchor. Other alternatives might be envisaged, e.g., the MNO uses a DPI engine (e.g., at the UPF) to differentiate traffic of specific applications. Discovery of certain applications for certain users will then trigger procedure. Such traffic differentiation can be based in DPI as an example on the TLS client hello Server Name Indication (SNI), on destination IP ranges published or provided by the Application provider, etc.  How such kind of solution would work (UPF having to buffer the corresponding traffic, etc…) is FFS

2a.	If early DNS handling applies, LDNSR forwards the DNS discovery request to C-DNS that responds with a central AS IP address. The UE connects to the AS via the existing PSA. This case is not further described as no re-anchoring is triggered in this case

2b.	If late DNS handling applies, then LDNSR process of the DNS request takes place as described in solution 22;.

As part of this process the LDNSR buffers the DNS answer and triggers/notifies the SMF.

3.	SMF decides on re-anchoring. That decision could either be based on SLA information locally configured in SMF, or on the PCCs rules previously received from PCF for the PDU Session. This is similar to the decision taken by SMF to insert an UL CL/BP in solution 22SMF shall select a UPF/PSA closest to the user. If latest UE Location is not available, SMF gets UE location from AMF by invoking Namf_EventExposure service with OneTime Report type (as in TS 29.518, ch 5.3.1).

If early DNS handling, DNS respond with the AS address, which will cause the UE to connect to the AS via the existing PSA (and once new PDU session is established, the UE should trigger a new DNS query, see solution #24, or other solution for KI#2, and executed in steps 7 and 8)

4.	SMF initiates Change of PDU Session Anchor using one of the following slightly modified SSC mode 2/3 related procedures :

-	SSC mode 3 with IPv6 Multi-homed PDU Session (clause 4.3.5.3 of TS 23.502 [3]). This aspect is described in solution 22

-	SSC mode 3 with multiple PDU Sessions (clause 4.3.5.2 of TS 23.502 [3])

-	SSC mode 2, clause 4.3.5.1 of TS 23.502 [3] where step 2 and step 3 are modified in the following way.

-	Step 2: The SMF indicates to AMF "use SMF (set) for next PDU session to the same (DNN, S-NSSAI)" to enforce that the new PDU Session will use the same SMF (set)

-	Step 3: AMF uses the indication in step 2 to select the SMF

5.	For the new PDU session, the UE may be configured with either the Local DNS or the C-DNS server address, see Solution #10. Usage reporting for the relevant EC flows is activated to track activity.

NOTE 2:	Further re-anchoring (to a central UPF) may be triggered if EC application terminates.

NOTE 3:	As for all use of SSC modes 2 and 3, all IP traffic on the PDU session is affected since UE IP address is changed from an UE IP address corresponding to the old PSA (in this case central PSA) to an UE IP address corresponding to the new PSA (in this case local PSA).

.	SMF shall select a UPF/PSA suitable for traffic offload This is done as described in solution 22.).

6	 when the new PDU Session has been established, the SMF indicates to LDNSR that DNS processing can resume as described in solution 22 (when SMF has inserted an UL CL/BP) but that the LDNSR cannot answer to the UE

In this case the LDNSR cannot answer to the UE as due to the establishment of a new PDU Session, the UE has changed of IP address

7.	The UE sends again the DNS query (after the establishment of the new PDU session). The DNS query goes through the Local PSA to the DNS resolver provided at the session establishment of the new session, and it is resolved to an Edge

8.	DNS response includes an EAS that is closest to PSA (with DNS state of the art many Authoritative (DNS) Name servers already today return different responses based on the perceived topological location of the user

9.	The Application Traffic then starts towards the selected Edge AS.

### 6.12.3	Impacts on services, entities and interfaces

-	SMF:

-	KI#1, Ensure UE DNS requests for EC AS-FQDN reach LDNSR, e.g., configure UE with the address of LDNSR as the DNS server during PDU session establishment via PCO.

-	KI#1,

as in solution 22 but with the behaviour in step 4: SMF initiates Change of PDU Session Anchor using slightly modified SSC mode 2/3 related procedures

-	KI #5 and KI#2, The re-anchoring decision and procedures including conveying the "DNAI for next PDU session" (and additional information if needed) to AMF as described in clause 6.12.2.2.

-	AMF:

-	KI#1, using SMF (set) information received from former SMF in the selection of the new SMF.

-	KI#5 & KI#2, using DNAI information received from former SMF in the selection of the new SMF and conveying the DNAI (and additional information if needed) to the new SMF as specified in Clause 6.12.2.2 above.

LDNSR:

-	KI#1, processing UE DNS queries and notifying SMF of specific AS-FQDN in the UE DNS (this depends on LDNSR placement, as described clause 6.12.2.1.)

-	KI#1, in the case of late DNS handling support: buffering the UE DNS query, getting notification from SMF and re-directing the DNS query to a local DNS Server by referral to this DNS server.

* * * * 2nd Change  * * * *

### 7.1.5	Evaluation for Key Issue #1: Solutions for Distributed Anchors

Many solutions propose using DNS for EAS discovery for distributed anchor connectivity model.

A DNS based mechanism for EAS discovery supports the different EAS deployments described in KI#1. It has no impact on the Application itself and keeps UE unaware of the application deployment (at edge or at central DN) and application ownership (e.g. the EAS is owned by the MNO or by a third party) aspects. The TR includes DNS based solutions for the other connectivity models as well, and so, selecting DNS, applications can be developed to rely on DNS for EAS discovery and be agnostic of the operator connectivity model chosen.

Distributed anchor part of Solutions #2, #4, #5 and #10 describe DNS based working solutions for EAS discovery for distributed anchor connectivity model. These solutions are using DNS state of the art: many Authoritative (DNS) Name servers already today return different responses based on the perceived topological location of the user, either using the source IP address of the DNS query or ECS when received according to RFC 7871 [7].

A pre-requisite for solutions in the distributed anchor point scenario is to anchor the UE according to subscription policies.

The goal is to provide to DNS addressing information related to the UE topological location. These solutions show that in 5GC that can be achieved by:

A.) Providing a DNS for the PDU Session that is near the PSA, the DNS request can be solved to an Application server which is closest to that PSA. Solution alternatives:

-	The 5GC(SMF) provides a DNS that is closest to the PSA

-	The 5GC(SMF) provides an Anycast DNS address

B.) Including in the DNS Query an ECS that is representative of the UE location /N6 interface, DNS can provide an EAS that is closest to the PSA. Solution alternatives:

-	The 5GC(SMF) provides a DNS that supports RFC 7871 [7] and adds an ECS that is representative of the UE location /N6 interface, e.g. based on the user IP address after (an optional) NAT.

The above summary plus a selection of procedures from solutions #2, #4, #5 and #10 are to be promoted into normative phase. Procedures from these solutions that illustrate recommended solution versions are: 6.2.3.1, 6.5.2.6, 6.5.2.7, 6.10.2.1 and 6.10.2.2.

These solutions solve KI#1 under certain conditions that are to be documented together with the solutions in normative phase:

-	These solutions require the corresponding geographical resolution support by the Authoritative (DNS) nameserver.

-	These solutions are guaranteed to work if the operator provided DNS settings are used by the UE for the DNS Query. Guidelines should be captured to cover scenarios where the OS, user or applications may override the operator-provided DNS settings.

-	When UE DNS Queries are sent to another DNS Resolver instead (that depends on the UE Application client, Browser and/or OS configuration) if that is centrally deployed and it does not support RFC 7871 [7], the selected AS might not be closest to the User PSA. Complementary application layer mechanisms may be needed to reselect AS based on the source IP address of the user application traffic.

-	UE IP address is subject to privacy restrictions and shall not be sent to authoritative DNS resolvers outside the network operator

Solution #12 enhances 5GC to dynamically re-anchor a PDU session at the Edge (e.g. re-anchoring the PDU Session from central to distributed anchor connectivity model triggered by the user DNS for the Application name resolution). Once the session has a distributed anchor, Solution #10 could be used for EAS discovery. Solution #12 thus provides an alternative to DNS triggered dynamic establishment of UL CL/BP and local PSA for Session breakout connectivity model by leveraging on SSC mode 2 and SSC mode 3 enablers, which requires a simpler LDNSR functionality and therefore it is recommended into normative.

Solution #14 is an application layer solution to discover an Edge AS with no impact on 5GC when used with distributed anchor connectivity model. It is up to the application provider to adopt or not this mechanism if desired. 5GC does not preclude this option. This solution may be mentioned during normative phase as a complement to DNS based discovery for distributed anchor connectivity. The solution assumes the Service Switch is pre-configured the mapping information between the IP address range supported by PSA and EAS information based on the agreement between the MNO and service provider. The solution does not provide a description of the procedure for this connectivity model, so no procedure of this solution is recommended into normative. Then, regarding the rest of the solutions for KI#1 and Distributed Connectivity model:

Solutions #6 should not be promoted into normative for this scenario, but it should be evaluated with the rest of solutions for session breakout connectivity model.

Solution #18 addresses the scenario where DNS might not be able to return an AS that is closest to the user location. Unless EC App flows can be differentiated on L3 level, the proposal implies buffering, mapping and remapping and resolving in UPF all first packet of flows showing a new destination IP, which will impact latency and throughput. It is also unclear how it coexists with application layer solutions for service continuity: UPF may overwrites any new target EAS selected at App layer for that app client, which could break procedures for seamless AS relocation for load balancing, resilience or to adapt to edge relocation. This solution relies on IETF standards, but reverse DNS lookup is not critical to the normal function of the internet, and so, it is not universally adopted, which questions the effectiveness of the solution. Rapid mapping/remapping of PTR, SRV and A/AAAA is not well-suited to DNS, and will also require cache flushing throughout the DNS hierarchy. This solution is not recommended into normative.

* * * * 3rd Change  * * * *

### 9.1.2	Conclusions regarding solutions for Key Issue #1 for Distributed Anchors

Distributed anchor part of Solutions #2,#4, #5 and #10 describe DNS based working solutions for EAS discovery for distributed anchor connectivity model and the following selection of procedures from solutions #2, , #5 and #10 are to be promoted into normative phase: Clauses 6.2.3.1, 6.5.2.6, 6.5.2.7, 6.10.2.1 and 6.10.2.2.

NOTE:	Clauses 6.5.2.6, 6.5.2.7 are included based on the understanding they no more refer to "Translate FQDN using LDN/N6 path information" and to "uses distance/closeness metrics".

Decision for anchoring of the UE in the distributed anchor point scenario for EC shall be described as

-	using subscription policy information to set proper UE policy (e.g. URSP via usage of dedicated DNN), or

-	to apply proper policies at session (SMF) level if the UE policy (e.g. URSP via usage of dedicated DNN) cannot be used.

Privacy issues derived from potentially using UE IP address for ECS DNS in an external authoritative DNS in the related solutions shall be addressed during the normative phase.

Solution #12 provides a simplified variant of LDNSR functionality for the distributed anchor connectivity model that leverages on the enablers provided by SSC mode 2 and SSC mode 3 PDU sessions, and therefore it is recommended into normative.

Solution #14 is an application layer solution to discover an Edge AS with no impact on 5GC when used with distributed anchor connectivity model. 5GC does not preclude this option. This mechanism may be informatively mentioned during normative phase as a complement to DNS based discovery for distributed anchor connectivity. The solution assumes the Service Switch is pre-configured with the mapping information between the IP address range supported by PSA (not the PSA information) and EAS information based on the agreement between the MNO and service provider.

Solutions #6 and #18 are not recommended into normative.

*************** End Changes ***************