SA WG2 Meeting #141E	S2-2007168

12 October - 23 October 2020, Elbonia 			 						(revision of S2-200xxxx)

Source:	Nokia, Nokia Shanghai Bell

Title:	Conclusion on KI#2

Document for:	Approval

Agenda Item:	8.7

Work Item / Release:	FS_ID_UAS_SA2 / Rel-17

Abstract:  This contribution makes proposal for conclusion texts on KI#2 in TR 23.754.

# 1.	Discussion

In TR 23.754 there exists multiple solution proposals to handle KI#2. Broadly the solutions can be categorised into two classes, User Plane and Control Plane based solutions.

The list of solutions in TR 23.754 that address KI#2 are, Solution #2, #3, #4, #5, #6, #7, #10, #11, #12, #17, #23, #24, #26

Out of these solutions:

Solution #2 only provides a reference architecture and does not provide any detailed procedure.

Procedure in Solution #3 is covered by Solution #5 and Solution #23.

Solution #6 does not provide any procedure for EPS and 5GS part of the solution is covered by Solution #5.

Solution #7 also does not provide procedure for EPS and 5GS part is partly covered by Solution #23.

Solution #11 is also covered by Solution #23.

Solution #12 & #17 does not provide any procedure for EPS.

Solution #24 & #26 re-uses procedure in Solution #5.

Overall, from a completeness pov, only Solution #4, Solution #5 and Solution #23 covers all the aspects of the KI#2, including procedure for 5GS and EPS. As remaining solutions are either incomplete or covered by these 3 solutions (#4, #5 & #23), to narrow down the scope of evaluation, the rest of this paper will only consider Solution #4, #5 and #23 for further evaluation and conclusion on KI#2

Solution #4 is a User Plane based solution and handles the application related information, UTM/USS - UAV Authentication/Authorization over the user plane

On the other hand, Solution #5 and #23 are control plane focused solutions, where UTM/USS - UAV Authentication/Authorization is performed using control plane signalling.

The paper provides a comprehensive evaluation of each of these solutions and proposes 3 different way forward depending on the consensus that can be reached during the discussion.

# 2.	Proposal

It is proposed to capture following texts as the conclusion for KI#2 in TR 23.754

* * * * First Change * * * *

# 7.	Evaluaiton

Editor's note:	This clause will provide a general evaluation of the solutions.

Below solution evaluation comments are w.r.t. KI#2

Solution #4

Solution #4 tries to provide procedure to address KI#2 limiting much of the impacts on existing core network functions.

Procedure for 5GS, as explained in clause 6.4.2.1 of TR 23.754, indicates at step 5, that UAS AF retrieves subscription information from PCF or UDM. Which means in a Roaming scenario (Local Breakout), the breakout DNN/APN (i.e. well-known UTM DNN/APN) would be served by a visited PCF which the UAS AF will interact with to get subscription info.

Procedure for EPS, as explained in clause 6.4.2.2 of TR 23.754, indicates that PGW does header enrichment and adds UAV subscription information. If TLS is used it would be able to decrypt the TLS and based on the session related information available in the PDN GW enrich the requests on HTTP level before forwarding it to the UAS-AF. Since the UAS-AF belongs to the operators domain the corresponding is owned by the operator and should not be a problem to get access to.  The PGW can get subscription information from PCRF based on the use of SPR or UDR. An alternative to use header enrichment in EPC is to only use the 5GC/EPC interworking architecture which will give the same solution as for 5GC.

The procedure defined (for both 5GS and EPS) can handle  the authentication mechanism where multiple messages may need to be exchanged between the UAV and the USS/UTM as it is done over the User Plane on application level and is out of 3GPP scope.

There are few other areas that are  clarified  in the proposed solution.

-	How does the UAV know the URL of UAS AF? Is it pre-provisioned? Does it mean, to cover roaming scenarios, the UAV is provisioned with UAS AF URL of all roaming PLMNs?

A solution to this is that the UAV is preconfigured URL/domain-name used for access to a UAS-AF which e.g. maybe with a globally common can be defined by GSMA. There is a method described in 3GPP TS 26.501  “5G Media Streaming (5GMS) General description and Architecture”  Annex B “MNO-specific Service Access Information acquisition” that should be possible to use to resolve the local UAS-AF URL/domain-name from the globally common one predefined in the UAV.

-	It is also not clear, when the USS/UTM receives a A&A request from the UAS AF, how does it know which NEF/SCEF have to be reached for invoking AsSessionWithQoS API, for policy modification of the PDU Session/PDN connection. Does it directly send the request to NEF/SCEF or via UAS AF?

A URL to the applicable NEF API to use can be included by the UAS-AF in the AA request sent to the USS for the specific UAV (see updates in S2-2007107).

-	What happens if the A&A by USS/UTM fails? Does the PDU session remain established for ever?

This is upto the application layer. The USS/UTM will be able to request specific action/policy to be applied via the network exposure interface or via UAS AF e.g. block all or a significant portion of the traffic, to disconnect the PDU-session, to deregister the UE etc.

Solution #5

Solution #5 provides 2 different procedures for UAV A&A by USS/UTM for 5GS and EPS. In case of 5GS the A&A is done after registration (before PDU session establishment) and the A&A result is known to the AMF. Whereas, in EPS the A&A is done during registration, at default PDN connection establishment, and the A&A result is known the to PGW.

This requires specific mechanisms for interworking between EPS and 5GS. For example, when the UAV UE is first registered via E-UTRAN, the A&A result is known to the PGW. Now if the UAV UE moves to NG-RAN, the AMF may need to re-initiate the A&A process. Similarly, when the UAV UE moves from NG-RAN to E-UTRAN, the PGW may need to perform the A&A again.

In the case where PGW-C + SMF and UPF + PGW-U are used for interworking between 5GC and EPC, when the UAV UE is on the EPC network, the A&A result is available in PGW-C + SMF function. Now when the UAV UE moves to 5GC, the A&A context available at PGW-C+SMF becomes irrelevant, as the AMF may need to perform the A&A in this case again, based on determination by the AMF. Moreover, this will further complicate the design of UEFS, as at each such handover (between 5GC and EPC) the contact point for UEFS changes (e.g. for UUAA revocation).

In this solution, the UFES at each handover between 5GC and EPC must change the contact point between AMF and SMF/PWG-C (e.g. for UUAA revocation).

Solution #23

Solution #23 proposes exactly same procedure for both 5GS and EPS. All the problems identified in Solution #5 are addressed in Solution #23. Solution #23 works for all the below scenraios:

-	UAV UE using only 4G subscription (no interworking with 5GS)

-	UAV UE using only 5G subscription (no interworking with 4GS)

-	The is no impact on N26 based interworking between EPC and 5GC, as the A&A context is not stored in MME or AMF

-	The solution also works seamlessly for EPC and 5GC interworking using combo functions PGW-C + SMF and UPF + PGW-U

-

Solution #23 	has no impact on N26 based interworking between EPC and 5GC, as the A&A context is not stored in MME or AMF.

Solution #23 has no impact on legacy network elements like MME, SGW, PGW and proposes common procedures in SMF and PGW-C + SMF combo function, that allows seamless interworking and movement of the UE between 4G and 5G access.

Solution #23 is depending on the support of EAP-based Authentication in the USS/UTM and needs approval from the USS/UTM industry.

* * * * Next Change * * * *

# 8.	Conclusions

Editor's note:	This clause will capture conclusions from the study.

The following principles are applied, as applicable, when developing UAV support in 3GPP:

No commercial relationship is assumed between the 3GPP Network and a USS/UTM, in order to enable an UAS operator to change the serving USS/UTM while remaining with same 3GPP Network subscription, and vice versa.  The 3GPP Network subscription for the UAV cannot be assumed to contain any information about the USS/UTM based on a commercial relationship between the 3GPP Network and the USS/UTM.

Key Issue #1:

An UAV is identified by USS/UTM using a CAA-level UAV ID assigned by USS/UTM, and identified by the 3GPP System using a 3GPP UAV ID assigned by the MNO.

The CAA-level UAV ID is used for Remote ID functionality (network or broadcast remote ID). Remote Identification support by 3GPP in the scope of this release applies to the UAV, not to UAV Controller. No assumptions are made limiting the type of information on UAV Controller provided via Remote Identification to satisfy regulatory requirements.

Various formats of CAA-level UAV ID must be supported by the UAV to support various geo-specific regulations. At least Serial Number Identification, a CAA-Issued Registration Identifier (aka Session ID), and USS/UTM-Issued UUID shall be supported

It is assumed that the mechanisms for resolution of CAA Level UAV ID to the USS serving the corresponding UAV, defined outside 3GPP, and available to entities outside the 3GPP system (e.g. the TPAE), are used in the 3GPP system to discover the USS for the UAV.  It may be also possible to use other UAV information (e.g. UAV-provided USS address or FQDN) sent by the UAV to 3GPP system, to be used by the 3GPP System, to discover the USS for the UAV.3GPP system is provided the CAA-level UAV ID by the UAV, and it may provide the CAA-level UAV ID to the UTM/USS when providing MNO services towards the UTM/USS.

The USS/UTM is made aware of the 3GPP UAV ID of the UAV during procedures of UAV authorization supported by the 3GPP network. . The USS/UTM uses the 3GPP UAV ID to invoke MNO services (e.g. exposure function or location services) or during authorization or authorization revocation. The 3GPP UAV ID is in the format of a GPSI, and at least the External Identifier is supported.

The External Identifier is allocated by the 3GPP network without interaction with the USS/UTM, and must be unique within the geography (e.g. at least country) of the 3GPP network.

Key Issue #2:

Proposal :	Keep only Solutions #4, #5 and #23 from TR 23.754 for further evaluation to select one of them for normative work.

NOTE 1:	If solution #23 is to be used it needs an approval from the USS/UTM industry that EAP based authentication can be used.
To make a selection beween a CP and UP solution requires a proper evaluation is needed comparing CP and UP effects on UAS functionality and effects on the 3GPP system.

An UAV may be authenticated and authorized by USS/UTM with the support of the 3GPP system before connectivity for UAS services (e.g. UAS-USS connectivity for NRID) is enabled. Existing authentication and authorization framework is leveraged as much as possible to minimize the impact on 5GS and EPS system protocols

A UAV includes a CAA Level UAV identity to the 3GPP system. The 3GPP system determines whether to initiate UAV authentication/authorization based on request from UAV, subscription, local policies, and results of previous authentication/authorization. The USS/UTM can revoke such UAV authorization.

NOTE 2:	The details of how the CAA Level UAV ID is provided (e.g. a specific parameter or a transparent container) will be defined during normative work.

UAV authentication and authorization by USS/UTM is conditional on the UE having performed successfully a primary 3GPP authentication and with Aerial UE function as part of the subscription.

An UAV is authenticated and authorized by USS/UTM using a CAA-level UAV ID. The credentials and related authentication method used by the UAV and UTM/USS are outside of the 3GPP scope.

The 3GPP network shall be informed of the UAV authentication and authorization result and enforce the result accordingly. Upon successful UAV authentication and authorization by USS/UTM, UAV is authorized to establish limited connectivity to communicate with USS/UTM.

A UAV request for user plane connectivity to the 3GPP system for UAV operations (i.e. C2 between a UAV and a networked UAV controller and/or flight authorization request) may also require additional authorization by the UTM/USS.

Other Aspects:

Single PDU session/PDN connection for USS and C2 connectivity, and separate PDU sessions/PDN connections for USS and C2 connectivity are supported. The mechanism that may be used is up to deployment.

The USS/UTM is not assumed to have knowledge of PDU sessions or PDN connections: the USS/UTM authorizes connectivity requests sent from the 3GPP system for a UAV or UAV controller, can revoke such authorization, and can provide information to control such connectivity (e.g. ACL, QoS information, etc.).

The functionality to support authorization of UAV and UAV controller pairing applies to networked UAV Controllers and non-networked UAV controllers that are connected to UAV via internet.

For geofencing, enable both the "direct query from USS" model, the "direct USS subscription" model, and the "area of interest subscription" model.

For geo-caging, both the option of the 3GPP system providing the UAV location to the USS during procedures, and the option where the USS retrieves it on demand, are supported.

Enable a USS UAV discovery mechanism where the USS/UTM query MNO for UAVs served by the MNO in a specific location.

Editor's note:	For NR to be used for UAVs, "aerial features" as defined in TS 36.300 [9] for E-UTRA, must be enabled and RAN work is needed.

* * * * End of Changes * * * *