3GPP TSG-WG SA2 Meeting #141E e-meeting 	S2-2007222r01

Elbonia, October 12 – 23, 2020	(revision of S2-200xxxx)

Source:	Huawei, HiSilicon, Interdigital Inc.

Title:	KI#4: Evaluation for KI#4 on L2 UE-to-UE Relay

Document for:	Approval

Agenda Item:	8.8

Work Item / Release:	FS_5G_ProSe/Rel-17

Abstract: This contribution proposes the overall evaluation and interim conclusion for KI#4 on L2 UE-to-UE Relay.

# 1. Introduction/Discussion

There are two types of solutions to address KI#4 documented in TR 23.752, L2 UE-to-UE Relay and L3 UE-to-UE Relay. This contribution provides the evaluation and interim conclusion on L2 UE-to-UE Relay.

The following table summarises the overall of solutions on L2 UE-to-UE Relay in this TR.

| Solution | Applicability | Main content |
| --- | --- | --- |
| #8 | Applicable to L2 and L3 Relay | UE-to-UE Relay Selection |
| #9 | Only applicable to L2 Relay | connection establishment, connection management and protocol stack |
| #11 | Applicable to L2 and L3 Relay | Relay discovery from group members |
| #31 | Applicable to L2 and L3 Relay | E2E QoS control |
| #33 | Applicable to L2 and L3 Relay | Network-Assisted U2U Relay Discovery and Selection |
| #36 | Applicable to L2 and L3 Relay | Authorization for 5G ProSe UE-to-UE Relay Service |
| #50 | Applicable to L2 and L3 Relay | Negotiated UE-to-UE Relay Reselection |

Sol#9 proposes the procedures of connection establishment and connection management, and the protocol stack for L2 UE-to-UE Relay. The UE-to-UE Relay performs the link mapping for data forwarding between Source UE and Target UE. Security is established end to end between Source UE and Target UE in the PDCP layer.

Sol#8 and #33 proposes Relay selection mechanisms. In Sol#8, the Target UE performs the UE-to-UE Relay selection and Source UE performs the path selection between the direct path and indirect path; In Sol#33, the core network assists Relay discovery and selection based the relative location between the UEs.

Sol#11 proposes the Relay discovery of group members using Model A and Model B. However, the solution is limited to Relay discovery of group members.

Sol#31 provides the QoS control mechanism with E2E QoS parameter division by the UE-to-UE Relay.

Sol#36 proposes PCF based service authorization and provisioning for UE-to-UE Relay Service.

Sol#50 proposes the Relay reselection mechanism, by the negotiation between Source UE and Target UE via the existing Relay connection.

Through the analysis and comparison on the above solutions, the following are taken as interim conclusions for the L2 Relay:

- For connection establishment and connection management, Sol#9 is selected.

- For Relay discovery, both Model A (initiated by UE-to-UE Relay) and Model B (initiated by Source UE) are supported.

- For service authorization, a PCF based solution is used and Sol#36 is selected.

- For QoS handling, Sol#31 is selected.

- For Relay reselection, negotiated UE-to-UE Relay reselection can be used and Sol#50 is selected.

# 2. Text Proposal

It is proposed to capture the following changes vs. TR 23.752.

# * * * * First change * * * *

## 7.4	Key Issue #4: Support of UE-to-UE Relay

The following are analysis of the L2 UE-to-UE Relay solution:

-	Sol#9 proposes the procedures of connection establishment and connection management, and the protocol stack for L2 UE-to-UE Relay. The UE-to-UE Relay performs the link mapping for data forwarding between Source UE and Target UE. Security is established end to end between Source UE and Target UE in the PDCP layer.

-	Sol#8 and #33 proposes Relay selection mechanisms. In Sol#8, the Target UE performs the UE-to-UE Relay selection and Source UE performs the path selection between the direct path and indirect path; In Sol#33, the core network assists Relay discovery and selection based the relative location between the UEs.

-	Sol#11 proposes the Relay discovery of group members using Model A and Model B. However, the solution is limited to Relay discovery of group members.

-	Sol#31 provides the QoS control mechanism with E2E QoS parameter division by the UE-to-UE Relay.

-	Sol#36 proposes PCF based service authorization and provisioning for UE-to-UE Relay Service.

-	Sol#50 proposes the Relay reselection mechanism, by the negotiation between Source UE and Target UE via the existing Relay connection.

# * * * * Second change * * * *

## 8.4	Key Issue #4: Support of UE-to-UE Relay

For key issue#4 "Support of UE-to-UE Relay", the following are taken as interim conclusions for the L2 UE-to-UE Relay:

-	For connection establishment and connection management, Sol#9 is selected.

NOTE:	Sol#9 may need further update according to RAN WG’s progress.

-	For Relay discovery, both Model A (initiated by UE-to-UE Relay) and Model B (initiated by Source UE) are supported.

-	For service authorization, a PCF based solution is used and Sol#36 is selected.

-	For QoS handling, Sol#31 is selected.

-	For Relay reselection, negotiated UE-to-UE Relay reselection can be used and Sol#50 is selected.

# * * * * End of changes * * * *