# 3GPP TSG SA2 Meeting #141E	S2-2007226r02

# October 12 – 23, 2020, Electronic meeting 	(revision of S2-200xxxx)

Source:	NTT DOCOMO

Title:	KI#1: Update the evaluation and conclusion

Document for:	Discussion/Approval

Agenda Item:	8.5

Work Item / Release:	FS_IIoT / Rel-17

Abstract of the contribution: Complete the evaluation of the BMCA methods for Key Issue#1. Update the conclusions of the Key Issue #1 for IP type PDU Sessions.

1		Proposal

The following change is proposed for TR 23.700-20.

*** Start Change ***

8.1	Key Issue #1: Uplink Time Synchronization

To enable support for Uplink Time Synchronization, following are some interim principles to be considered for the way forward:

-	The following interim agreement are applied for transmission of the gPTP event messages (carried within Sync message for one-step operation or Follow_up message for two-step operation):

-	DS-TT which is attached by one or more TSN GMs will perform exactly the same operations for UL gPTP messages as what the NW-TT performs for the DL gPTP messages as specified in clause 5.27.1.2.2 of TS 23.501 [2].

-	The PDU session of DS-TT port receiving the source GM's gPTP messages is used to forward the gPTP messages to the UPF/NW-TT:

-	the UPF/NW-TT forwards the gPTP messages to the NW-TT and DS-TT ports in Master state except for the PDU session of the source ("avoids play back to the source DS-TT port").

- 	If the egress port is in NW-TT, the NW-TT port performs exactly the same operations for UL gPTP messages as what the DS-TT performs for the DL gPTP messages as specified in clause 5.27.1.2.2 of TS 23.501 [2].

-

-	If the egress port is in other DS-TT, the other DS-TT(s) perform the operation as specified in clause 5.27.1.2.2 of TS 23.501 [2].

-	As specified in TS 23.501 [2] already for Rel-16 Time Synchronization, 5GS is required to guarantee delivery of (g)PtP message in less than 10ms as recommended by IEEE 802.1AS[6] clause B2.2, if the operator decides to follow such recommendation.

*** End of Changes ***