# 3GPP TSG SA2 Meeting #141E	S2-2007226r02

# October 12 – 23, 2020, Electronic meeting 	(revision of S2-200xxxx)

Source:	NTT DOCOMO

Title:	KI#1: Update the evaluation and conclusion

Document for:	Discussion/Approval

Agenda Item:	8.5

Work Item / Release:	FS_IIoT / Rel-17

Abstract of the contribution: Complete the evaluation of the BMCA methods for Key Issue#1. Update the conclusions of the Key Issue #1 for IP type PDU Sessions.

# 1		Proposal

The following change is proposed for TR 23.700-20.

*** Start Change ***

## 8.1	Key Issue #1: Uplink Time Synchronization

To enable support for Uplink Time Synchronization, following are some interim principles to be considered for the way forward:

-	The following interim agreement are applied for transmission of the gPTP event messages (carried within Sync message for one-step operation or Follow_up message for two-step operation):

-	DS-TT which is attached by one or more TSN GMs will perform exactly the same operations for UL gPTP messages as what the NW-TT performs for the DL gPTP messages as specified in clause 5.27.1.2.2 of TS 23.501 [2].

-	The PDU session of DS-TT port receiving the source GM's gPTP messages is used to forward the gPTP messages to the UPF/NW-TT:

-	the UPF/NW-TT forwards the gPTP messages to the NW-TT and DS-TT ports in Master state except for the PDU session of the source ("avoids play back to the source DS-TT port").

- 	If the egress port is in NW-TT, the NW-TT port performs exactly the same operations for UL gPTP messages as what the DS-TT performs for the DL gPTP messages as specified in clause 5.27.1.2.2 of TS 23.501 [2].

-

-	If the egress port is in other DS-TT, the other DS-TT(s) perform the operation as specified in clause 5.27.1.2.2 of TS 23.501 [2].

-	As specified in TS 23.501 [2] already for Rel-16 Time Synchronization, 5GS is required to guarantee delivery of (g)PtP message in less than 10ms as recommended by IEEE 802.1AS[6] clause B2.2, if the operator decides to follow such recommendation.

## 8.3	Key Issue #3B: Exposure of Time Synchronization

To enable Exposure of Time Synchronization, it is recommended to select solutions 7 and 9 as the basis for normative work. Solutions 7 provide the description to extend the exposure framework to support AF requesting time synchronization. Solution 7 considers the exchange of information required between the 5GS and the AF to support the 4 time synchronization methods of KI#3B and the functionality required at the DS/NW-TTs for every method. Solution 9 complements the exposure of time synchronization addressing the subscription and authorization of the AF's request. Additionally, it enables the time synchronization configuration before the PDU Session is established. Solution 9 adds the support of DS-TT capabilities indication towards the 5GS.

Following are principles to be considered as the basis for normative work:

-	Time Synchronization service can be provided for Ethernet PDU sessions and IP PDU sessions.

-	Four time synchronization methods are supported as described in Solution #7.

-	Time synchronization service may support GM residing in DS-TT(s) as described in Solution #9.

Editor's note:	whether there is a need to include validity time for AF time sync request is FFS. Also, how to support validity time is FFS.

Editor's note:	It is FFS how to configure the 5GC (e.g. the UE subscription) to support the AF requesting time synchronization.

The following interim agreement are applied for IP type PDU Sessions for the transmission of the GM time in PTP over UDP/IP messages (carried within Sync message for one-step operation or Follow_up message for two-step operation):

-	DS-TT which is attached by one or more GMs will perform exactly the same operations for UL PTP messages as what the NW-TT performs for the DL gPTP messages as specified in clause 5.27.1.2.2 of TS 23.501 [2].

-	The PDU session of DS-TT receiving the source GM's PTP messages is used to forward the PTP messages to the UPF/NW-TT:

-	the UPF/NW-TT forwards the PTP messages to DS-TTs and the N6 interface if it knows the DS-TT or N6 interface has active listeners for PTP multicast messages.

-	If the PTP message is sent to N6 interface, the NW-TT performs exactly the same operations for UL PTP messages as what the DS-TT performs for the DL gPTP messages as specified in clause 5.27.1.2.2 of TS 23.501 [2],

-	If the PTP message is sent to DS-TT/UE via a PDU Session,  the DS-TT(s) perform the operation as specified in clause 5.27.1.2.2 of TS 23.501 [2].

*** End of Changes ***