3GPP TSG-WG SA2 Meeting #141E e-meeting  	S2-2007231

# Elbonia, October 12 – 23, 2020	(revision of S2-200xxxx)

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.316 | CR | 2053 | rev | - | Current version: | 16.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarification on UDM and UDR services in 5WWC |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5WWC |  |  |  |  |  | Date: |  |  | 2020-10-02 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | For RGs connecting to 5GC via W-5GAN, there are additional UDM and UDR services applicable. However, in clause 8.6.1.1 where the description of UDR services for RGs are wrong. Firstly, the table 8.6.1.1-1 is actually UDM service instead of UDR service. Secondly, the reference of table 8.6.1.1-2 is wrong. It should refer to clause 5.2.12.2.1 in TS 23.502, where the table is about Data keys as shown below: Table 5.2.12.2.1-1: Data keys Data Set; Data Subset; Data Key; Data Sub Key. Access and Mobility Subscription data; SUPI; Serving PLMN ID and optionally NID. SMF Selection Subscription data; SUPI; Serving PLMN ID and optionally NID. UE context in SMF data; SUPI; PDU Session ID or DNN. Subscription Data (see clause 5.2.3.3.1); SMS Management Subscription data; SUPI; Serving PLMN ID and optionally NID. SMS Subscription data; SUPI; Serving PLMN ID and optionally NID. Session Management Subscription data; SUPI; S-NSSAI. DNN. Serving PLMN ID and optionally NID. Slice Selection Subscription data; SUPI; Serving PLMN ID and optionally NID. Group Data; Internal Group Identifier or External Group Identifier; -. Identifier translation; GPSI. SUPI; Application Port ID. |
| Summary of change: |  | - For the table 8.6.1.1-1 in the original text, it should be moved to clause 8.1.1.1 which descirbes UDM services and the reference shall be table 5.2.3.3.1-1 in TS 23.502 clause 5.2.3.3.1. - For the table 8.6.1.1-2 in the orginal text, it should refer to Table 5.2.12.2.1-1 in TS 23.502 clause 5.2.12.2.1. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | There are wrong descriptions and wrong references in clause 8.6.1.1. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.1.1.1, 8.6.1.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

#### 8.6.1.1	General

The UDM makes use of the Nudr_DM service to perform the mapping of the SUPI/IMSI associated with the Line ID or HFC-Identifier included in the SUCI.

In addition to the Subscription data types and corresponding Subscription Data keys used in the Nudr_DM_Service, as defined in TS 23.502 [3], the Subscription data types and corresponding Subscription data keys defined for the Nudr_DM Service in Table 8.6.1.1-1 and Table 8.6.1.1-2 are applicable for FN-RGs connected to 5GC.

Table 8.6.1.1-1: UE Subscription data types

| Subscription data type | Field | Description |
| --- | --- | --- |
| Subscription identifier translation | Other Identifier Of The Subscription (Global Line ID or Global Cable ID) | Global Line ID or Global Cable ID included in SUCI provided by the W-AGF to the 5GC for FN-RG support and used as pseudonym of the SUPI. |
|  | SUPI | Corresponding SUPI |

Table 8.6.1.1-2: Data keys

| Data Set | Data Subset | Data Key | Data Sub Key |
| --- | --- | --- | --- |
| Subscription Data | Subscription identifier translation | Global Line ID or Global Cable ID | - |

# * * * * End of changes * * * *