SA WG2 Meeting #141-E	S2-2007277

Elbonia, October 12 - 23, 2020

Source:	Samsung, NTT DOCOMO

Title:	KI#4: Evaluation update

Document for:	Approval

Agenda Item:	8.1

Work Item / Release:	FS_eNA_ph2 / Rel-17

Abstract of the contribution: This contribution updates the KI#4 evaluation

# Proposal

It is proposed to include the following text in TR 23.700-91.

******************************** START OF CHANGES **************************************

### 7.4	Key Issue #4: Remaining aspects on how to ensure that slice SLA is guaranteed

The following is a description of solution #2 and the aspects that are covered, there are no evaluation of the different solutions, since only one solution is defined. However, this clause evaluates some aspects of solution#2.

-	How does NWDAF determine slice service experience and slice load information and which entity or entities subscribe(s) to/request(s) the analytics?

-	Using the above analytics, how is slice load distribution carried out to assist with SLA?

For the first aspect: NWDAF determines the slice service experience and slice load information as follows:

-	NWDAF performs input data collection for slice service experience analytics as described in clause 6.4 of TS 23.288 [5].

-	NWDAF performs input data collection for slice load analytics as described in clauses 6.2.1.3 and 6.2.1.4.3.

-	Based on the overall input data collection information, NWDAF determines the slice service experience and slice load statistics and predictions.

-	NWDAF provides the output to the service consumer e.g. NSSF or AMF,

For the second aspects: NWDAF  provides statistics and predictions on Slice load or Observe Service experience or both and then the consumer NF take actions if needed to help maintaining the SLA as follows:

- Based on the NWDAF statistics and predictions mentioned above the service consumer (i.e. NSSF) may trigger an indication to the AMF  for a slice restriction for UE registrations and/or PDU sessions. AMF enforces the slice restriction by accepting or rejecting new UE registrations and/or PDU session establishments on the restricted slice following the procedures in TS 23.502 [3].

******************************** END OF CHANGES **************************************