SA WG2 Meeting #140E	S2-2007297

Electronic meeting, Oct 12 – 23, 2020	(Revision of S2-200xxxx)

Source:	Nokia, Nokia Shanghai Bell

Title:	KI#1 Update of Solution #33 to address editor´s note

Document for:	Approval

Agenda Item:	8.9

Work Item / Release:	FS_5MBS / Rel-17

Abstract of the contribution:

This paper updates solution #33 to address the editor´s note.

# 1	Discussion

Solution 33 contains the following editor´s note:

Editor's note:	It is FFS if N4 interface as specified in TS 29.244 [17] is suitable for the interaction between MSF-C and MSF-U. Collaboration with other WGs (e.g. SA WG4) is needed.

Possible candidates for a protocol between MSF-C and MSF-U could be:

- H.248. This is a well-known protocol and in use for many older 3GPP interfaces but suffers from a reputation of complexity and interoperability problems between vendors. It is anticipated that some extensions e.g. for FEC or ROHC would be required.

- PFCP (see TS 23.501 [2] and TS 29.244 [17]). This protocol is used successfully in the 3GPP ecosystem and provides capabilities e.g. for packet filtering, forwarding, repacketization of PDUs (i.e. manipulation of transport layer), event notifications and billing. It lacks media manipulation capabilities (for transcoding, FEC and ROHC) and will require related extensions; solution 33 contains proposals for such extensions. PFCP is already used to control UPFs and implementation synergies and even a possible collocation could be enabled if the MSF-U is also controlled by an extended PFCP.

- New protocol. The development of both H.248 and PFCP took several years and due to the complexity, it would be very challenging to develop a new stable protocol in a single release. Also, no synergies with existing implementations would be possible.

From the above candidates, it seems that PFCP is best suited. Solution 33 provides a framework for extensions for media processing and thus demonstrates that PFCP can be extended to accommodate relted requirements

Without doubt, detailed requirements for media processing go beyond the SA2 expertise and require coordination with SA4. A possible work split could be that SA2 defines high-level PFCP extensions to enable media processing extensions (e.g. in TS 23.501 where the current stage 2 for PFCP is defined), SA4 defines detailed stage 2 requirements for media processing and PFCP stage 3 extensions are defined on that basis either by CT groups (most likely CT4) or SA4.

An LS to SA4 asking for feedback is being proposed in S2-2007528.

# 2	Conclusions

Proposal: Agree the proposed changes to Solution #33.

Proposed changes to TR 23.757

## 6.33	Solution #33: PFCP-based interactions between MBSF and MBSU

### 6.33.1	Functional Description

#### 6.33.1.1	System Architecture

Baseline architectures Architecture 1 contains an MSF-U controlled by an MSF-C, and architecture 2 contains a MBSU controlled by an MBSF for similar tasks; the MBSF is integrated into the NEF. The proposed converged architecture also contains an MBSU controlled by an MBSF. It is expected that proposed solution can be applied to baseline architecture 1, baseline architecture 2, and the converged architecture.

; The terms CP function and UP function will be used in what follows. ACP function that could be MSF-C, NEF, or MBSF, and a UP function that could be a MSF-U or MBSU

NOTE:	Collaboration with other WGs (e.g. SA WG4) is needed. A possible work split could be that SA WG2 defines high-level extensions of the session management parameters in subclause 5.8.2.11 of TS 23.501 [2] to enable media processing , SA4 defines detailed stage 2 requirements for media processing and PFCP stage 3 extensions are defined on that basis either by CT groups (most likely CT4) or SA4.

#### 6.33.1.1	High Level Proposal

The procedures between UP function and CP function will be an based on extension on N4 session management parameters in subclause 5.8.2.11 of TS 23.501 [2] . N4 session management is used successfully in the 3GPP ecosystem and provides capabilities e.g. for packet filtering, forwarding, repacketization of PDUs (i.e. manipulation of transport layer), event notifications and billing. It lacks media manipulation capabilities (for transcoding, FEC and ROHC) and will require related extensions; solution 33 contains proposals for such extensions

This has the advantage that N4 session management is well known in the 3GPP ecosystem and that implementations collocating the UPF (that is already controlled using N4 session management) and the MBSU/MSF-U to minimize the user plane processing resources could also become possible in that manner.

#### 6.33.1.2	Requirements

The UP function will need to support transcoding, Forward Error Correction (FEC), and Robust Header Corrections (ROHC).

For FEC, two outgoing streams are created for one incoming stream: a main stream and a repair stream. If several content streams are received, they will be transmitted as several outgoing streams, but a single FEC repair stream can be send for all those content streams.

For ROHC, several outgoing content or FEC repair streams may be combined in one ROHC stream.

For Transcoding:

-	it may be necessary to receive an incoming RTCP stream with feedback for an outgoing RTP stream and adjust the transcoder accordingly

-	it may be necessary to send an outgoing RTCP stream with feedback derived by the transcoding for an incoming RTP stream.

Figure 6.33.1.2-1: Incoming ang outgoing data streams at the UP Function

#### 6.33.1.3	N4 session management extensions

It is suggested that the CP function supplies newly introduced "Packet Processing Rules" (PPR), extended Packet Detection Rules (PDR), and Forwarding action rules (FAR) to the UP Function as part of a PFCP session establishment or modification

The UP Function processes and modifies incoming media according to the received instructions in the PPR:

-	It transcodes the media as requested by the CPF.

-	It applies Robust header compression to outgoing media when requested by the CPF.

-	It applies Forward Error Correction to outgoing media and generates an associated outgoing FEC repair stream when requested by the CPF.

-	It monitors the incoming media stream and generates an associated RTCP feedback stream when requested by the CPF.

-	It receives an incoming RTCP feedback stream and may adjust the media processing based on the RTCP feedback when requested by the CPF.

-	It provides Event notifications about media processing related Events to the CPF when requested by the CPF.

A PPR contains:

-	An PPR identifier.

-	Information about the outgoing media format (e.g. CODEC, FEC, ROHC, and possible parameters for the outgoing media format:

-	Type (e.g. CODEC, FEC, ROHC).

-	Codec specific parameter, e.g. for AMR parameters as defined in clause 8.1 of IETF RFC 4867 [19].

-	Transport, e.g. RTP profile.

-	For ROHC (see IETF RFC 5795 [20] and IETF RFC 3095 [21]): Either profile, target periodicity for full header packets, MAX_CID parameter.

-	For FEC framework configuration information (see clause 5.5 of IETF RFC 6363 [22]).

-	opional FAR ID for the outgoing media stream.

-	optional subsequent PPR ID.

-	Optional information about the incoming media format:

-	Type (e.g. CODEC, FEC, ROHC).

-	Codec specific parameter, e.g. for AMR parameters as defined in clause 8.1 of IETF RFC 4867 [19].

-	Transport, e.g. RTP profile.

-	Optional information about associated outgoing media stream:

-	An FAR identifier and/or an PPR ID.

-	Type (e.g. RTCP feedback for incoming media format, FEC repair flow, ROHC flow).

-	Optional media format specific parameter.

-	Optional information about associated incoming stream:

-	Type (e.g. RTCP feedback for outgoing media format, ROHC feedback).

-	Optional media format specific parameter.

-	Events to be reported to CP function, e.g.:

-	Unexpected Input media.

-	Resource shortage.

-	For FEC and ROHC: Session establishment success or failure.

The Packet detection rule (PDR) is used as follows:

-	An optional IE to contain PPR ID to be applied for traffic matching the PDR. Several PPR IDs may be provided and all related PPR shall then be applied in consecution, in the order the PPR IDS are provided.

-	An indication of the usage of the traffic within the PPR (e.g. associated stream, RTCP, ROHC, media), can be omitted for media usage.

-	The existing FAR ID in the PDR applies for the main output stream of the PPR.

-	If the PDR contains an associated PPR ID or indicates that the traffic is not to be used for media (but e.g for RTCP) within the PDR, it does not contain a FAR ID.

Figure 6.33.1.2-1: Description of packet processing in the UPF using Packet Detection Rules (PDRs), Packet Processing Rules (PPRs), and Forwarding Action Rules (FARs)

### 6.33.2	Procedures

Figure 6.33.2-1: session management interactions to configure UP Function packet processing

1.	CPF sends PFCP Session Establishment Request and includes PDRs (with PPR ID, and possible PPR usage information), PPRs (with information about incoming and outgoing encoding and a possible application of FEC and ROHC, about associated incoming streams, and about associated outgoing streams with FAR IDs, and media processing related Events to be reported), and FARs).

2.	UP Function is configured accordingly.

3.	UP Function receives an incoming packet.

4.	UP Function processes the incoming packet:

-	It searches a matching PDR.

-	It select PPR(s) according to PPR ID(s) in PDR.

-	It transcodes and applies ROHC and/or FEC according to PPR(s).

-	If sends outgoing packets as described by FARs.

5.	UP Function sends outgoing packet.

6.	UP Function generates RTCP feedback based on PPR instructions and sends it in backward direction.

7.	UP Function may report media processing related events to CPF based on CPF subscriptions.

8.	UP Function receives an incoming RTCP packet in backward direction.

9.	UP Function processes the incoming RTCP packet:

-	It searches a matching PDR.

-	It select PPR(s) according to PPR ID(s) in PDR and forwards the stream as RTCP stream to PPR based on usage information in PDR.

-	It may adjust the media processing base4d on the received feedback.

To enable that several media streams are obtaining a single outgoing FEC repair stream, several media streams point to the same PPR for FEC. The pointer may be the PPR ID in the PDR of each media stream. A PPR for FEC may thus have several input media streams and will then produce an output media stream for each input media stream (and send this stream to the FAR ID in the corresponding PDR), and in addition an FEC repair stream (to be sent to the FAR ID for the outgoing media streams. The PPPR for FEC can be invoked after other PPRs for transcoding.

Figure 6.33.2-2: Description of packet processing in the UP Function when several media streams are obtaining a single outgoing FEC repair stream

To enable that several media streams are grouped together in a single ROHC flow, several media streams point to the same PPR for ROHC. The pointer may be the PPR ID in the PDR of each media stream and the PDR will then not contain a FAR ID. A PPR for FEC may thus have several input media streams and will then produce only a single ROHC stream (to be sent to the FAR ID for the outgoing media stream in the ROHC PPR). The PPR for FEC can be invoked after other PPRs for transcoding or FEC.

Figure 6.33.2-3: Description of packet processing in the UP Function when several outgoing media streams are encapsulated together in a single outgoing ROHC flow

### 6.33.3	Impacts on services, entities and interfaces

MBSF:

-	Support N4 session management extended by Packet Processing Rules.

MBSU:

-	Support N4 session management extended by Packet Processing Rules.

-	Support media processing such as transcoding, FEC and ROHC.