# 3GPP TSG-SA WG2 Meeting #141E	S2-2007303

# Oct 12 – 23, 2020, Electronic, Elbonia	(revision of )

# Title:	Draft LS Response on Clarification on AAA-Server address

# Response to:	S2-2004786 / C4-203452

# Release:	Release-16

# Work Item:	eNS

Source:	SA WG2

To:	CT WG4, SA WG3

Cc:

Contact Person:

Name:	Peter Hedman

Tel. Number:

E-mail Address:	peter (dot) hedman (at) ericsson (dot) com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

# Attachments:	S2-2007300r01, S2-2007301r01

1. Overall Description:

SA2 thanks CT4 for the LS regarding the potential lack of clarity in stage 2 specifications with regards to the handling of the AAA-S address in NSSAA procedures.

SA2 has agreed on the attached CRs fixing the wording in SA2 specifications and would like to answer CT4 questions as follows:

- Whether the AAA-S address is conveyed by AMF to the AUSF/NSSAAF in step4?

[S2 Response]: Yes, if the AMF has received a AAA-S address for the S-NSSAI from the UDM, it is provided in step 4.

SA2 would also like to highlight that the AAA-S address is optionally provided by the UDM to the AMF but this is not required, e.g. in roaming scenarios where this might cause privacy concerns, if the AAA-S is owned by a 3rd party.

- In clause 5.15.3 of 23.501, there is the text "the indication whether the S-NSSAI is subject to Network Slice-Specific Authentication and Authorization and associated AAA Server Address", does it imply that the AAA-S address is stored in UDM/UDR and AMF gets the associated AAA Server Address from the UDM/UDR? Or is there any other mechanism to get the associated AAA Server Address?

[S2 Response]: It is only optional (based on operator configuration) for the UDM to store the AAA-S address in the UE subscription for an S-NSSAI subject to NSSAA, and it is also optional (based on operator configuration) for the UDM to provide it to the AMF in roaming scenarios.

If the NSSAAF did not receive a AAA-S address from the AMF in the request, the NSSAAF needs to determine a AAA-S address to use for NSSAA procedures for this S-NSSAI based on local configuration at NSSAAF.

2. Actions:

To CT WG4, SA WG3 groups.

ACTION: 	SA2 kindly requests CT4 and SA3 to take into consideration the answers to the provided questions.

3. Date of Next SA2 Meetings:

3GPP TSG SA2#142e	November/2020	E-Meeting