3GPP TSG-SA WG2 Meeting #141E	S2-2007304r01

# October 12th – 23rd 2020, Elbonia	(revision of S2-xxxxx)

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 2484 | rev | - | Current version: | 16.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | N3IWF selection procedure when accessing SNPN via PLMN |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | Vertical_LAN |  |  |  |  |  | Date: |  |  | 22/09/2020 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | In the scenario of accessing SNPN via PLMN, the curent method is to use a configured N3IWF of SNPN. But there is a risk that if the country in which the UE is located requires lawful interception of SNPN traffic and the SNPN is not deployed locally at the country in which the UE is located, the regulatory requirement of the country in which UE is located can not be fulfilled. According to the LS reply S2-2004810 from SA3-LI WG: As there is no roaming defined for SNPN, an SNPN shall provide necessary facilities in each country in which they want subscribers to use their service. So, in case the SNPN has no facilites deployed in the country in which the UE is located, the UE needs to check if the SNPN is exempted from LI requirement in the country in which the UE is located. If it is exempted, UE still can use the SNPN service via PLMN, but otherwise it is not allowed to use the SNPN service via PLMN. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Adding a new clause to specify how the UE perform N3IWF selection when UE is accessing SNPN service via PLMN(s). |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Risk of not fulfill the regulatory requirement. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.3.6.2, 6.3.6.2a(new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | N | Other core specifications |  |  |  | TS/TR….CR…. |  |  |
| affected: |  |  | N | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | N | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * Start of Changes * * *

#### 6.3.6.2	Stand-alone N3IWF selection

The UE performs N3IWF selection based on the ePDG selection procedure as specified in the TS 23.402 [43] clause 4.5.4 except for the following differences:

-	The Tracking/Location Area Identifier FQDN shall be constructed by the UE based only on the Tracking Area wherein the UE is located. The N3IWF Tracking/Location Area Identifier FQDN may use the 5GS TAI when the UE is registered to the 5GS, or the EPS TAI when the UE is registered to EPS. The Location Area is not applicable on the 3GPP access.

-	The ePDG Operator Identifier (OI) FQDN format is substituted by with N3IWF OI FQDN format as specified in TS 23.003 [19].

-	The ePDG identifier configuration and the ePDG selection information are substituted by the N3IWF identifier configuration and the Non-3GPP access node selection information respectively. The UE shall give preference to the N3IWF in all PLMNs in the Non-3GPP access node selection information independent of the "Preference" parameter.

Network slice information cannot be used for N3IWF selection in this Release of the specification.

Accessing a standalone non-public network service via a PLMN, the UE uses a configured N3IWF FQDN to select an N3IWF deployed in the SNPN. In this release, the UE shall use the configured N3IWF FQDN to select an N3IWF deployed in the SNPN only if the UE determines it is located in the same country as the SNPN. The UE determines the country it is located in by using implementation specific means.

* * * End of Changes * * *