3GPP TSG-SA/WG2 Meeting #141E	S2-2007332r02

Elbonia, 12 – 23 Oct. 2020, Electronic meeting

Source:	Qualcomm

Title:	KI#6 evaluation and conclusion

Document for:	Agreement

Agenda Item: 	8.4

Work Item / Release:	FS_eNS_Ph2 / Rel-17

Abstract of the contribution: This contribution proposes evaluation and conclusions for KI#6

# 1	Proposal

The paper analyses solutions for KI #6 and proposes conclusions.

Start of Change

# 7	Overall Evaluation

## 7.x	Evaluation of Solutions for Key Issue #6 - Constraints on simultaneous use of the network slice

The related solutions are #26, #27, #28, #39, #40, #41, #42

| Solution #26 | Network controlled enforcement of simultaneous usage of network slices based on user preference | The advantages of this solution are: - it enables the UE to determine the priority for all the requested slices, or a subset of them, based on the services/applications active in the UE. - It enables the CN to determine the Allowed NSSAI even if the UE does not provide any prioritization The issue with the solution is that it requires a degree of coordination between the UE and the network, and that the network interprets the prioritization provided by the UE. |
| --- | --- | --- |
| Solution #27 | Network slices simultaneous usage incompatibility support | Somehow similar to solutions #28, #41, and #42 in the sense that the UE is provided configuration information wrt the sets of slices that are compatible. The problems with the solution are: - It only allows restriction on user plane connectivity. - it does not allow ever the selection of specific AMF ( simultaneous slices are registered even if they effectively cannot be used at the same time so it means that the selected AMF may not be optimal and probably it should be a default AMF). the assumed behaviour is that the UE remains registered with a slice even if it cannot use any PDU session in the slice - The behaviour of legacy UEs is totally not deterministic as they receive some unknow PDU session rejections: what do they do about that? - The roaming use case where the VPLMN does not support is not described. The advantages of the solution are: - It is deterministic in the CN knowledge of whether the UE can support network slices simultaneous usage incompatibility feature, due to the UE explicit indication - The UE is provided explicit incompatibility information, thus based on the UE priority for services/application, can deterministically decide how to create a Requested NSSAI that contains the high priority slices that are compatible. - It allows the UE to register to slices that are incompatible while using resources only for compatible sets of slices. - it relies on UE to deactivate PDU sessions if it needs to establish a PDU session for an incompatible slice - it does not require UE to re-register to get user plane resources for incompatible slices wrt the current active slices |
| Solution #28 | Compatibility Class based support for simultaneous use of the network slice | Somehow similar to solutions #27, #41, and #42 in the sense that the UE is provided configuration information wrt the sets of slices that are compatible. It proposes that the network provides to the UE for each S-NSSAI a "simultaneous use information" compatibility class defining the compatibility with other Network Slices, and that the UE shall not include in the Requested NSSAI S-NSSAIs that map to HPLMN S-NSSAIs which are incompatible with each other |
| Solution #39 | Support registration for incompatible network slice(s) in UE's current serving RA or different one | The solution is not complete and does not address all the scenario. The solution does not consider how the UE performs cell selection, since it implies that upon receiving a rejection cause in NAS wrt an S-NSSAI being available outside the current RA, the UE that desires such S-NSSAI would perform cell reselection to a cell outside the current RA, which is not defined at present and is not defined by this solution either. The solution seems rather indeterministic. |
| Solution #40 | Separate SUPI/GPSI per isolated set of S-NSSAIs | This creative solution enables different “UE profiles” associated to different SUPIs/GPSIs and that identify specific set of S-NSSAIs that are compatible. Roaming agreements ensure that compatibility between S-NSSAIs is coordinated. This may require in extreme rare occasion a replication of an S-NSSAI with a different SD in the VPLMN. The solution does not require UE to declare what it supports to the network no requires any coordination between VPLMN and HPLMN for upgrades since it is backward compatible with existing procedures and mechanism for allowed/configured/subscribed slices. What is not understood is just ignored. The solution has the following concerns/disadvantages: - it has NAS impacts in terms of new registration procedure and new NAS mechanisms internal to the UE (e.g. switch between SUPIs). |
| Solution #41 | Simultaneous use of the network slice via Configured NSSAI | The solution is similar to #27, #28, and #42 in the sense that the UE is provided configuration information wrt the sets of slices that are compatible. Similar to #28 and #42 it suffers from the issues, not present in #27, that: - the UE can never register at the same time for incompatible slices - it requires UE to re-register to get data resources for incompatible slices wrt the current active slices - does not allow an operator to configure incompatible S-NSSAIs within the same TA/RA, which restricts deployment scenarios |
| Solution #42 | UE handling of constraints on simultaneous use of network slices based on network assistance | The solution is similar to #27, #28, and #41 in the sense that the UE is provided configuration information wrt the sets of slices that are compatible. Similar to #28 and #41 it suffers from the issue, not present in #27, that: - the UE can never register at the same time for incompatible slices - it requires UE to re-register to get data resources for incompatible slices wrt the current active slices |

Next Change

End of Changes