3GPP TSG-WG SA2 Meeting #141E e-meeting 	S2-2007335

Elbonia, October 12 – 23, 2020	(revision of S2-200xxxx)

Source:	Qualcomm

Title:	Conclusions: additional conclusions for various key issues.

Document for:	Approval

Agenda Item:	8.7

Work Item / Release:	FS_ID_UAS / Rel-17

Abstract: This contribution proposes some simplification of the conclusions for the Release 17 work based on received input from the aviation community.

# 1. Introduction/Discussion

Additional conclusions are proposed for the various key issues and as general principles.

# 2. Text Proposal

It is proposed to capture the following changes vs. TR 23.754.

# * * * * First change * * * *

# 8	Conclusions

Editor's note:	This clause will capture conclusions from the study.

The following principles are applied, as applicable, when developing UAV support in 3GPP:

No commercial relationship is assumed between the 3GPP Network and a USS/UTM, in order to enable an UAS operator to change the serving USS/UTM while remaining with same 3GPP Network subscription, and vice versa.  The 3GPP Network subscription for the UAV cannot be assumed to contain any information about the USS/UTM based on a commercial relationship between the 3GPP Network and the USS/UTM.

The following conclusions are proposed for normative work:

1)	standardize a new 3GPP UAS Application Function for support of aerial functionality related to UAV identification and tracking, and to support Remote Identification. The UASAF is based on the functional definition of UFES of solution #5 plus the UAVF of solution #25.

2)	Standardize the interface between the 5GS/EPS and USS/UTM based on the NEF/SCEF principles, using web-based interfaces.

3)	Consider the integration of the new AF in the NEF/SCEF.

4)	Standardize a control plane solution for interfacing of the UAV and the 5GS/EPS as proposed in solution #5

The following conclusions apply:

-	A UAV is registered with CAA authorities either before connecting with the 3GPP system or using plain internet connectivity via the 3GPP system. During such registration the UAV is assigned a CAA-level UAV-ID.

-	A UAV that registers for UAS services (i.e. to take advantage of aerial features, connectivity with USS/UTM for Networked Remote Identification, and C2 connectivity) indicates its UAS capability to the 5GS system. The UAV also provides the CAA-Level UAV ID and aviation level information in both 5GS and EPS. The aviation level information is transparent to the 5GS/EPS. A UAV that has not performed a registration with CAA authorities shall not attempt to register for UAS services.

-	The 5GS/EPS verifies during registration/attach that the UAV UE has a subscription for aerial use before any authentication/authorization procedure is supported with the USS/UTM.

-	In 5GS, the authentication/authorization is triggered during the registration procedure as defined in solution #5, and as described in the details of key issue #1 conclusions.

-	In EPS, the authentication/authorization is triggered during the attach procedure and supported via exchange of parameters in the PDN connection establishment performed at attach, as described in solution #5.

-	A dedicated APN/DNN is used for UAV connectivity for UAS services (i.e. connectivity to USS/UTM, C2 traffic to UAVC, etc.)

-	The 3GPP systems provides a mechanism to support the authentication/authorization procedure between the UAV and the USS/UTM. During this procedure, the USS/UTM may access additional information from the 3GPP system to decide about the authentication/authorization and provides information about the success of the authentication/authorization.

-	The EPS/5GS discovers the USS/UTM as described in solution #5.

-	During the authentication/authorization procedure, the 3GPP system provides 3GPP identities/information for the UAV including the GPSI, IP Address, optionally the PEI, the CAA-Level UAV ID and any aviation level information the UAV has provided to the 3GPP system. The USS/UTM uses such information for the authentication/authorization and for future interactions with the 3GPP system regarding the UAV via NEF/SCEF.

-	the 3GPP system supports USS/UTM authorization of pairing between a UAV and a networked UAVC or a UAVC that connects to the UAV via Internet connectivity during the establishment of the PDN connection/PDU session for UAS services (i.e. connectivity to USS/UTM and for C2 traffic). Modifications to the pairing or re-authorization take place via modification of the established PDN connection/PDU session. During such procedures, the USS/UTM provides to the 3GPP system information (e.g. QoS requirement, data flow descriptors, etc.) that enable traffic between the UAV and the UAVC. How the USS/UTM is made aware of the UAVC is outside the scope of 3GPP in this release.

-	USS discovery is based on the mechanisms defined in solution #5 in clause 6.5.2.3.

Key Issue #1:

A UAS operator registers the UAV to a UTM/USS with a procedure which is out of scope of 3GPP. The UTM/USS assigns a CAA-Level UAV ID

An UAV is identified by USS/UTM using a CAA-level UAV ID assigned by USS/UTM, and identified by the 3GPP System using a 3GPP UAV ID assigned by the MNO.

The CAA-level UAV ID is used for Remote ID functionality (network or broadcast remote ID). Remote Identification support by 3GPP in the scope of this release applies to the UAV, not to UAV Controller. No assumptions are made limiting the type of information on UAV Controller provided via Remote Identification to satisfy regulatory requirements.

Various formats of CAA-level UAV ID must be supported by the UAV to support various geo-specific regulations. At least Serial Number Identification, a CAA-Issued Registration Identifier (aka Session ID), and USS/UTM-Issued UUID shall be supported

It is assumed that the mechanisms for resolution of CAA Level UAV ID to the USS serving the corresponding UAV, defined outside 3GPP, and available to entities outside the 3GPP system (e.g. the TPAE), are used in the 3GPP system to discover the USS for the UAV.  It may be also possible to use other UAV information (e.g. UAV-provided USS address or FQDN) sent by the UAV to 3GPP system, to be used by the 3GPP System, to discover the USS for the UAV.3GPP system is provided the CAA-level UAV ID by the UAV, and it may provide the CAA-level UAV ID to the UTM/USS when providing MNO services towards the UTM/USS.

The USS/UTM is made aware of the 3GPP UAV ID of the UAV during procedures of UAV authorization supported by the 3GPP network. .

The USS/UTM uses the 3GPP UAV ID to invoke MNO services (e.g. exposure function or location services) or during authorization or authorization revocation. The 3GPP UAV ID is in the format of a GPSI, and at least the External Identifier is supported.

The External Identifier is allocated by the 3GPP network without interaction with the USS/UTM, and must be unique within the geography (e.g. at least country) of the 3GPP network.

Key Issue #2:

An UAV may be authenticated and authorized by USS/UTM with the support of the 3GPP system before connectivity for UAS services (e.g. UAS-USS connectivity for NRID) is enabled.

The UUAA procedure during UAV registration with 3GPP system as defined in Solution #5 is proposed to be taken into account during the normative phase. The execution of the UUAA is optional and is supported based on specific PLMN policies, USS requirements, and geographic regulatory requirements. The UUAA procedure is supported as follows:

-	A UAV registers with the 3GPP system using 3GPP credentials

-	During UAV registration a UAV includes a CAA Level UAV identity to the 3GPP system. The 3GPP system determines whether to initiate UAV authentication/authorization based on request from UAV, subscription, local policies, and results of previous authentication/authorization. The USS/UTM can revoke such UAV authorization. The CAA-Level UAV identity and the associated 3GPP UAV ID provided by the UAV will be used to locate the USS/UTM for UAV authentication and authorization.

NOTE:	The details of how the CAA Level UAV ID is provided (e.g. a specific parameter or a transparent container) will be defined during normative work.

- UAV authentication and authorization by USS/UTM is conditional on the UE having performed successfully a primary 3GPP authentication and with Aerial UE function as part of the subscription.

- An UAV is authenticated and authorized by USS/UTM using a CAA-level UAV ID. The credentials and related authentication method used by the UAV and UTM/USS are outside of the 3GPP scope.

- The 3GPP network shall be informed by the USS/UTM of the UAV authentication and authorization result and enforce the result accordingly. Upon successful UAV authentication and authorization by USS/UTM, UAV is authorized to establish limited connectivity to communicate with USS/UTM.

- If the result of the UAV authentication and authorization indicated by the USS/UTM is negative, network and MNO policies in the 3GPP system will decide if the UAV is kept registered to the 3GPP system or that the UAV is to be deregistered. In either case, the UAV is informed by the 3GPP system that USS/UTM authentication and authorization but it is still registered to the 3GPP system or the UAV is informed of the reason for its deregistration from the 3GPP system

A UAV request for user plane connectivity to the 3GPP system for UAV operations (i.e. C2 between a UAV and a networked UAV controller and/or flight authorization request) may also require additional authorization by the UTM/USS.

Other Aspects:

Single PDU session/PDN connection for USS and C2 connectivity, and separate PDU sessions/PDN connections for USS and C2 connectivity are supported. The mechanism that may be used is up to deployment.

The USS/UTM is not assumed to have knowledge of PDU sessions or PDN connections: the USS/UTM authorizes connectivity requests sent from the 3GPP system for a UAV or UAV controller, can revoke such authorization, and can provide information to control such connectivity (e.g. ACL, QoS information, etc.).

The functionality to support authorization of UAV and UAV controller pairing applies to networked UAV Controllers and non-networked UAV controllers that are connected to UAV via internet.

For geofencing, enable both the "direct query from USS" model, the "direct USS subscription" model, and the "area of interest subscription" model.

For geo-caging, both the option of the 3GPP system providing the UAV location to the USS during procedures, and the option where the USS retrieves it on demand, are supported.

Enable a USS UAV discovery mechanism where the USS/UTM query MNO for UAVs served by the MNO in a specific location.

Editor's note:	For NR to be used for UAVs, "aerial features" as defined in TS 36.300 [9] for E-UTRA, must be enabled and RAN work is needed.