3GPP TSG-WG SA2 Meeting #141E	S2-2007389r08

Elbonia, October 12 – 23, 2020	(revision of S2-2007389+6860+7014+7278+7292+7724)

Source:	Qualcomm Incorporated, InterDigital

Title:	Interim conclusion for Key Issue #8: UE data as an input for analytics generation

Document for:	Approval

Agenda Item:	8.1

Work Item / Release:	FS_eNA_Ph2 / Rel-17

Abstract: This contribution proposes the interim conclusion for KI#8.

# 1. Introduction

This contribution proposes the interim conclusion for KI#8: UE data as an input for analytics generation.

# 2. Text Proposal

It is proposed to capture the following new text in TR23.700-91.

* * * * Start of Changes * * * *

# 8	Conclusions

Editor's note:	This clause is intended to list conclusions that have been agreed during the course of the study item activities. This should also capture the guiding principles and documentation approach for creating CRs to normative specifications within the responsibility of SA WG2.

## 8.8	Key Issue #8: UE data as an input for analytics generation

The followings are agreed as interim conclusion for Key Issue #8.

- 	The user plane based solution is agreed for UE data collection. NWDAF interacts with an AF in the MNO domain to collect UE data as an input for analytics generation. The data provided by the MNO AF to NWDAF is based on collected data from UE over user plane.

The UE is configured with the address of the AF to contact with. The UE may be configured to contact a MNO AF or an AF external to the MNO domain. How the AF is configured for new UE data collection will be developed by SA4 and the LS reply from SA4 will be considered if there is any SA2 work to be done.

The NWDAF to MNO AF interface based on Naf_EventExposure is reuised and should be extended if new input data from the UE based on user plane data collection as above is needed.

NOTE 1: For existing input data/ parameters, NWDAF contacts a trusted AF (directly) or untrusted AF (via NEF) as defined in Rel-16.

Editor’s Note:	Whether solution#27 or solution#64 are selected as baseline or both solution #27 and solution #64 are captured in normative work will be further evaluated based on SA4’s reply LS.

-	The user plane based solution can be applied to collect input data for both 3GPP defined services and non-3GPP defined services.

NOTE 2: 		The new NWDAF input data/ parameters from MNO AF is FFS.

* * * * End of Changes * * * *