3GPP TSG-WG SA2 Meeting #141E e-meeting 	S2-2007465r14

Elbonia, October 12 – 23, 2020	(revision of S2-200xxxx)

Source:	Huawei, HiSilicon, CATT?, Vivo?, ZTE, OPPO

Title:	KI#7: Evaluation and Conclusion of KI#7

Document for:	Approval

Agenda Item:	8.9

Work Item / Release:	FS_5MBS / Rel-17

Abstract: This contribution contains the evaluation and conclusion for KI#7.

- Introduction

The following key issue #7 is documented in TR 23.757, this contribution propose evaluation and conclusion for KI#7.

## 5.7 Key Issue #7: Reliable delivery method switching between unicast and multicast ### 5.7.1 Description This key issue aims at providing support for dynamic delivery method switching between unicast and multicast in the 5GS. Depending on the number of devices receiving a specific content, their location, and RAN considerations, it may be necessary to support reliable and efficient delivery method switching between unicast and multicast. In addition, when a UE is receiving a multicast session, it may move across NG-RAN nodes and it is possible that the UE moves from a NG-RAN node that supports MBS to one that does not support MBS, or vice versa. The following aspects will be studied: - Triggers for delivery method switching between unicast and multicast. - How switching between unicast delivery method and multicast delivery method is performed in the 5GS (including the UE) while supporting service continuity. NOTE 1: The terms unicast delivery and multicast delivery methods may have different meaning depending on which part of the 5G system a solution is referring to as defined in clause 4.4, e.g. over-the-air it may mean PTP vs. PTM, between UPF and RAN it may mean shared tunnel vs unicast tunnel, etc. NOTE 2: Towards application, the terms unicast delivery and multicast delivery methods may also the application is using multicast session vs PDU session, which is in SA6 scope. NOTE 3: During the study of this key issue, RAN WGs, SA4 and SA6 will be involved, if needed.

- Discussion

There are 15 candidate solutions proposed to address key issue#7, i.e. solution#11/#12/#18/#22/#23/#24/#25/#26/#27/ #28/#29/#30/#31/#39/#40.

# 3. Text Proposal

It is proposed to capture the following changes VS. TR 23.757.

# * * * * First change * * * *

# 8	Conclusions

## 8.X	Key Issue #7: Reliable delivery method switching between unicast and multicast

For delivery method switching due to mobility, the following principle are agreed,

- When the UE moves from a NG-RAN node that supports 5MBS to a RAN node that does not support 5MBS, the network and UE shall support switch from 5GC Shared MBS traffic delivery method to 5GC Individual MBS traffic delivery method.

- When the UE moves from a RAN node that does not support 5MBS to a NG-RAN node that supports 5MBS, the network and UE shall support switch from 5GC Individual MBS traffic delivery method to 5GC Shared MBS traffic delivery method.

Editor’s Note: It is FFS before the handover procedure, whether the associated PDU Session needs to be activated and the UE and NG-RAN need to be aware of the linkage between the MBS Session and the associated PDU Session before mobility.  If yes, 5GC needs to provide this information to UE and NG-RAN.

- During the handover from RAN not supporting 5MBS to NG-RAN supporting 5MBS, PDU sessions, including the one associated with MBS session, are handed over to target RAN. After the handover, the switch is triggered at the 5GC from the 5GC Individual MBS traffic delivery method to 5GC Shared MBS traffic delivery method.

- For the handover from NG-RAN supporting 5MBS to RAN not supporting 5MBS, the MBS Session flow is converted to the QoS flow within the associated PDU session at the target RAN.

Editor’s Note: It is FFS whether the support for lossless handover with data forwarding from source NG-RAN to the target NG-RAN is needed, which needs confirmation by RAN.

For delivery method switching not due to mobility, the following principle are agreed,

- On switching between PTP and PTM delivery methods for 5GC Shared MBS traffic delivery, NG-RAN is the decision point of the switching.

- If the NG-RAN node support 5MBS, the network shall use the 5GC Shared MBS traffic delivery method for MBS Session packet transferring unless UE move from RAN node not supporting 5MBS. In that case the switch is triggered at the 5GC from 5GC Individual MBS traffic delivery method to 5GC Shared MBS traffic delivery method.

Editor’s Note: For the switch from 5GC Individual MBS traffic delivery method to 5GC Shared MBS traffic delivery method, it is FFS whether the trigger NF is SMF or MB-SMF or MBSF.

# * * * * End of changes * * * *