SA WG2 Meeting #S2-141E	S2-2007505

12 October - 23 October 2020, Electronic, Elbonia	(revision of S2-200xxxx)

Source:	ZTE

Title:	KI#3B, sol#9, Update the solution on BMCA

Document for:	Agreement

Agenda Item:	8.5

Work Item / Release:	FS_IIoT/ Rel-17

Abstract of the contribution: This paper modify the solution 9 to clarify the required parameters and impacts to network.

1. Discussion

In the solution#9 (method#4 in sol#7), the DS-TT act as the GM clock towards to end station behind of it.

To achieve this in the network support 802.1AS or IEEE 1588, the BMCA shall be supported. The DS-TT shall send the Announce message to the network behind of it.

How the Announce message is created, there are two option:

- The AF provides the BMCA information to DS-TT, the DS-TT create the Announce message by itself

- The AF creates the Announce message on behalf of DS-TT and send it to DS-TT in PMIC.

Both option can work, the option 2 may impact DS-TT less than option 1.

2. Proposal

It is proposed to agree the text added to TR 23.700-20

*****************************************START of CHANGE **********************************************

6.9	Solution #9: (g)PTP GM support by DS-TT

6.9.1	Introduction

Figure 6.9.1-1: DS-TT distributes 5GS time to devices attached to DS-TT using (g)PTP

This solution addresses KI#3B. As illustrated in Figure 6.9.1-1, key idea of this solution is that DS-TT acts as a (g)PTP grand master (GM) for devices attached to DS-TT. The time information used in (g)PTP is based on the time information received from the gNB as defined in TS 38.331 [11].

6.9.2	Functional Description

The solution is based on the following principles

-	DS-TT indicates to the network inside a Port Management Information Container (PMIC) whether DS-TT is capable of acting as (g)PTP GM and which version(s) it supports

-	IEEE 802.1AS [6] (i.e. gPTP), and/or

-	PTP over UDP/IPv4 as per IEEE 1588-2008 [9] Annex D, and/or

-	PTP over UDP over IPv6 as per IEEE 1588-2008 [9] Annex E.

-	If DS-TT has indicated that it is capable of acting as (g)PTP GM, then the network may request the DS-TT to generate (g)PTP messages to devices attached to the UE by providing (g)PTP parameters (incl. (g)PTP version, domain number(s), sending rate, BMCA information (i.e. attributes required for generating Announce message), etc.) and an instruction to start sending (g)PTP messages inside PMIC to the DS-TT.

NOTE:	DS-TT can be configured to support different (g)PTP profiles, e.g. SMPTE ST 2059-2 [9] or IEC/IEEE 61850-9-3 [10] by providing the (g)PTP parameters defined for those profiles to DS-TT.

6.9.3	Procedures

For IEEE TSN integration, i.e. if TSN AF is deployed, TSN AF controls the gPTP functionality in DS-TT using existing Port Management Information Container (PMIC) signaling between DS-TT and TSN AF.

Otherwise (i.e. for IP or Ethernet PDU Sessions not part of an IEEE TSN network), the NEF terminates the PMIC signaling instead of TSN AF. To achieve this, NEF generates PMIC signaling to control the (g)PTP client in DS-TT based on time synchronization API requests from an AF and sends PMIC to DS-TT using existing Rel-16 PCF functionality. Similarly, NEF receives PMIC from PCF using Rel-16 functionality.

It is assumed that a single NEF is handling all time synchronization API requests for a given UE.

Existing Port Management Information Container (PMIC) signalling is reused between DS-TT and the network.

The following figure illustrates an overview of the procedure to activate (g)PTP grandmaster functionality in DS-TT.

Figure 6.9.3-1: Procedure for synchronizing (g)PTP clients attached to DS-TT with the 5G clock

6.9.4	Impacts on services, entities and interfaces

DS-TT:

-	Optionally send (g)PTP capabilities; receive (g)PTP parameters.

-	Optionally generate (g)PTP messages based on network request.

-	Generates and sends the Announce message according to the BMCA information received from AF/NEF.

NEF:

-	Support Rel-16 PMIC signaling via PCF.

AF/NEF:

-	Send BMCA information to DS-TT.

*******************************************End of CHANGE **********************************************