SA WG2 Meeting #141E	S2-2007545

Oct 12 – 23, 2020, Elbonia	(revision of S2-200xxxx)

Source:	Ericsson

Title:	KI#1, Sol#2: Solution Update for Autonomous Steering Mode

Document for:	Approval

Agenda Item:	8.6

Work Item / Release:	FS_ATSSS_Ph2 / Rel-17

Abstract: This paper provides an update to Solution #2 for ATSSS Rel-17.

# Introduction/Discussion

Key Issue #1 in TR 23.700-93, titled “Additional Steering Modes” aims to study whether and how to support additional steering modes for Rel-17 eATSSS work. Solution #2, titled “New Steering Mode – Autonomous steering mode” is one of the many solutions in the TR addressing this key issue.

It is however important to put new steering mode proposals in relation to existing rel-16 steering modes. The current definition of Rel-16 steering modes do not give much freedom level, are focused only on a single aspect (or goal) and dictate steering more like a scheduling option. However, the rel-16 steering modes can be extended by focusing on multiple aspects or attributes described under the “Autonomous Steering Mode”, that can help enhance the existing steering mode behaviours in order to provide a better QoE for the user. Therefore, when proposing this new steering mode for Rel-17, it is important to describe how the existing Rel-16 steering modes can be applied together or enhanced with the proposed aspects of autonomous steering mode.

Therefore, Solution #2 is updated to portray the possibility of incorporating the aspects into Rel-16 steering modes instead of defining a new steering mode.

# 2. Text Proposal

It is proposed to update TR 23.700-93 as described below.

*** Start of Changes ***

## 6.X	Solution #X: Enhancements to steering mode operation

### 6.X.1	Introduction

This solution addresses KI#1 on Additional Steering Modes.

As specified in ATSSS Rel-16, the traffic of MA PDU session could be distributed across one or both accesses by using different steering modes. There are four steering modes as defined in Rel-16, i.e. Active-Standby, Smallest Delay, Priority-based and Load balancing. All the Rel-16 steering modes are decided by the network side and performed by the UE and UPF based on the link performance measurement. For example, if one access becomes unavailable, the UE and UPF can switch all the traffic to the other available access. However, except the access available/unavailable status, the UE and UPF cannot flexibility distribute the traffic over both accesses according to the link performance in real time. To be more specific, for the Load balancing mode, the traffic splitting weight is statically set by the network based on the operators' requirement without considering the link performance measurement. For the Priority-based mode, the traffic can be steered on alternative access only when one access is congested. In summary, none of the rel-16 steering modes allow the UE or the UPF to adjust the usage of the accesses dynamically based on the link status. Not accounting link performance conditions in existing modes can impact service experience for some applications which are sensitive to delay and/or loss.

### 6.X.2	High-level Description

Extending the steering mode configuration with link performance conditions as an option enables PCF to authorize when needed extra flexibility for both the UE and the UPF to perform the traffic steering accounting the mode intention but also the dynamic link characteristics.

It is proposed to extend the configuration of Load Balancing mode with access performance indicator thresholds as an option. Since PCF does not have a mean to provide different performance settings per access it is proposed that setting of such thresholds is common to both accesses. Thresholds are based on existing QoS parameters and inline with existing QoS model. Threshold values should thus be based on QoS requirements of the applocations/SDFs.

In case of Load Balancing steering mode, the PCF can indicate any combination of Maximum UL/DL RTT and UL/DL Maximum Packet Loss Rate in the PCC rule. The parameters are common for 3GPP and non-3GPP access and provided per PCC rule (i.e. per SDF). Jitter, as proposed in Solution 3, is not considered in the current 3GPP QoS model and is thus not taken into account in ATSSS.

Alternatively, the SMF derives Maximum UL/DL RTT and UL/DL Maximum Packet Loss Rate based on the rel-15/16 QoS parameters in the PCC rule.

Editor’s note: It is FFS whether the ATSSS QoS parameters/thresholds would be derived by PCF or SMF.

The SMF determines Maximum UL/DL RTT and UL/DL Maximum Packet Loss Rate per QoS Flow and provides to the UE and UPF as part of the ATSSS information.

The UE (and UPF) shall load-balance the traffic by accounting the weight factor configured and initiate the monitoring of the configured access performance indicators. If the performance of both accesses is within all the configured thresholds, the UE (or UPF) shall consider both accesses valid for steering and load balance based on the configured weight factor. If the performance of an access is not within any of the configured thresholds, the UE (or UPF) shall not consider this access as valid for steering. UE (or UPF) shall consider an access valid again when all thresholds are met again. In practice this means the UE (or UPF) will adjust dynamically the Load Balancing weight to the link performance so the actual Load Balancing factor resulting can differ from the one configured by PCF but will the benefit of providing a better service experience to subscribers. If none of the access are considered valid, UE (or UPF) shall steer traffic to a default access configured by PCF and notify PCF.

Editor’s note: It is FFS if thresholds, and if so what thresholds, are applicable to other steering modes.

See Figure 6.X.2-1 for details considering the load balancing example, where one single packet flow is shown as an example for UL and DL respectively. The weight factor for the traffic over each access is provided by PCF 50% for 3GPP/N3GPP in both UL and DL. In this example the application is latency sensitive and PCF provides UL/DL maximum RTT.

Initially, both the 3GPP and non 3GPP access RTT is below the threshold and UE and UPF schedulers e.g. use round robin mechanism in such a way that 50% weight is enforced. However, at some point of time, the UL delay in 3GPP access increases beyond the maximum threshold during a short period preventing the UE to steer during that access as long as the situation lasts. Once the situation is recovered, the UE scheduler will apply again the round robin mechanism considering the 50% weight. If the traffic distribution is observed for a longer period, e.g. during the busy hour, the effective load balancing weight achieved in UL will be 30% for UL and 50% for DL on 3GPP access, and 70% for UL and 50% for DL on non 3GPP access, as shown in the figure. The “longer term” load balancing weight may thus differ from the weight factor prescribed in the PCC rule, since the thresholds and access link conditions have been taken into account based on the thresholds provided to UE and UPF. The target for UE/UPF is however always to fulfil the operator’s prescribed weight provided by PCF, if the link conditions allow it. In this way the operator use case is maintained at the same time as access link conditions are taken into account to ensure a good QoE.

NOTE 1:	It is up to the UE and UPF scheduler implementation on how to schedule when several performance conditions are configured. It is up to the implementation how frequently the KPI measurements are monitored.

Figure 6.X.2-1: Steering mode example (average weight over some time)

This steering mode enhancement can be applied by the MPTCP, ATSSS-LL and (MP)QUIC steering functionalities.

### 6.X.3	Procedures

The MA PDU session establishment procedure is based on the signalling flow in clause 4.22.2 with the following changes:

-	In step 1, the UE provides Request Type as "MA PDU Request" in UL NAS message and its ATSSS capabilities as defined in TS 23.501 [3] clause 5.32.2. The UE includes in its ATSSS capabilities MPTCP functionality with any steering mode and/or ATSSS-LL functionality with any steering mode. The UE also indicates its capability to use a steering mode with thresholds.

-	In step 7, if dynamic PCC is to be used for the MA PDU session, the SMF includes the ATSSS capabilities of the MA PDU session. The PCF provides PCC rules which include the MA PDU session control information which includes the Steering Functionality and the Steering mode. When the steering mode in the MA PDU session control information indicates Load Balancing, it also includes the traffic splitting weight set by the network based on the operators' requirement together with the thresholds for Maximum RTT, UL/DL Maximum Packet Loss Rate as defined in Solution 3 of the present TR. Alternatively the SMF derives these values based on the existing rel-15/16 QoS parameters in the PCC rule. The SMF, from the received PCC rules, derives the ATSSS rules and N4 rules that will be sent to the UE and the UPF respectively. Both the UE and UPF will control the traffic steering, splitting, and switching in the UL/ DL direction based on the combination of link status and the threshold for RTT, UL/DL Maximum Packet Loss Rate.

-	In step 8, the SMF selects one or more UPFs as defined TS 23.502 [4] clause 6.3.3.3.

### 6.X.4	Impacts on services, entities, interfaces and IETF Protocols

This solution will impact the following entities in 5GS:

-	SMF: Supports to select the UPF with support of the new steering mode capabilities.

-	PCF: Supports to authorize the new steering mode capabilities for the SDF.

-	UPF: Supports the new steering mode capabilities.

-	UE: Supports the new steering mode capabilities.

-	5G-AN/ NG RAN: No impact.

*** End of Changes ***