SA WG2 Meeting #141E	S2-2007547

Oct 12 – 23, 2020, Elbonia	(revision of S2-200xxxx)

Source:	Ericsson

Title:	KI#1: Evaluation and interim conclusions for KI#1 (Steering Modes)

Document for:	Approval

Agenda Item:	8.6

Work Item / Release:	FS_ATSSS_Ph2 / Rel-17

Abstract: This paper provides an evaluation and interim conclusions of the steering modes proposed for KI #1.

- Introduction

Key Issue #1 in TR 23.700, titled “Additional Steering Modes” aims to study whether and how to support additional steering modes for Rel-17 eATSSS work. The solutions addressing this key issue are Sol#2, Sol#3, Sol#4, Sol#11 and Sol#12. In addition, solution #1 describes a few possibilities for new steering modes.

- Discussion

When evaluating KI#1 solutions, it can first be noted that there are no clear requirements that can guide the evaluation and conclusion for steering mode enhancements, and the evaluation of the proposals in the TR thus needs to be associated with a discussion about whether there are important new use cases and/or clear gaps compared to rel-16 steering modes. As alternative to defining new steering modes in rel-17, it may be useful to enhance rel-16 steering modes.

The enhancements proposed in the TR aims at adding new capabilities compared to rel-16 in the following four areas:

A.	Enable more flexibility in UE/UPF to select best traffic distribution (Sol #2, 3)

B.	Take additional performance parameters into account, such as loss rate, RTT differences, specific thresholds etc (Sol #2, 3, 11 and the steering modes suggested in Sol #1)

C.	Add packet duplication (bi-casting) as a way to reduce impacts of packet loss (Sol #4)

D.	Add a possibility for the UE to influence the steering mode and/or steering mode parameters such as the weight (Sol #12)

When it comes to aspects A and B above, it can be noted that rel-16 steering modes are rather “single minded” and only focus on a single goal. For example, the Load-balancing steering mode requires a certain distribution (based on weight) without any description for how to act in case both accesses are available, but one access has a high loss rate, or a high RTT. The Smallest-delay steering mode indicates that the access with lowest RTT shall be used, independent of other factors such as loss rate. With rel-16, only in case one access is not available does the UE/UPF stop the steering mode intent and use the single available access instead. In other words, the definition of the Rel-16 steering modes imply one specific scheduler behaviour, rather than pointing towards a scheduler intention or use case value. In practice however, an implementation needs to consider also other aspects in order to provide a good QoE for the user. At the same time, the PCF needs to retain policy control of which steering strategy/constraints should apply to a given data flow. It is important to avoid the scenarios where the PCF provides a steering mode and the UPF does not follow that steering mode. “A strict “single minded” steering mode thus risks to either result in bad QoE (if followed too strictly) or not to follow the steering mode required for the application (if UE/UPF starts acting on its own to improve the QoE).

If more flexibility needs to be added, it needs to be understood whether this should be in the form of new steering modes, extensions to existing steering modes or total delegation to UPF/UE in which case the PCF does not provide a steering mode and the UPF can apply local rules with the most suitable steering mode. The solutions #2, 3, 11 tries to resolve some of the rel-16 issues by defining new steering modes. However, since the rel-16 steering mode implementations anyway need to take such aspects into account to provide a good end-to-end-result, a better approach seems to be to enhance the existing steering modes with type A and B properties, to make them “more complete” and less “single minded”.

Therefore, an evaluation is performed on these lines on the various steering modes proposed in Rel-17 by emphasizing on the use case value, considering the possibility of extending the existing Rel-16 steering modes and schedulers with the characteristics defined for Rel-17 steering modes.

3. Proposal

It is proposed to update TR 23.700-93 as follows.

**** First Change ****

7	Evaluation

Editor's note:	This clause will provide a general evaluation of the solutions.

7.X 	Evaluation for KI#1: Steering Modes

General:

The following steering modes are proposed in Rel-17 for addressing KI#1:

- Smallest Loss Rate, Loss Rate Threshold and RTT Threshold (Solution #1)

- Autonomous Steering Mode (Solution #2):

- Autonomous Steering Mode with Advanced PMF (Solution #3)

- Redundant Steering Mode (Solution #4)

- RTT Difference based Steering Mode (Solution #11)

- UE Assisted Traffic Steering Mode (Solution #12)

They can be divided into four categories depending on what they are trying to achieve:

A.	Enable more flexibility in UE/UPF to select best traffic distribution (Sol #2, 3)

B.	Take additional performance parameters into account, such as packet loss rate of the link vs the maximum packet loss rate for the service, the RTT differences and other thresholds etc as defined in Sol #2, 3, 11 and the steering modes suggested in Sol #1

C.	Add packet duplication (bi-casting) as a way to reduce impacts of packet loss (Sol #4)

D.	Add a possibility for the UE to influence the steering mode and/or steering mode parameters such as the weight (Sol #12)

Category A, B proposals:

These type of solutions aims to address some limitations where each rel-16 steering mode is focusing too much on one specific aspect (e.g. load-balancing weight or lowest RTT).

The additional performance parameters of the new steering modes described for Categories B can also be applied to the existing Rel-16 steering modes where thresholds can be added e.g. for maximum packet loss and packet delay budget and then, providing a room of freedom for scheduler implementations

Solutions 2/3:

There are three main aspects proposed on solutions 2/3. They are analysed below:

1)	Autonomous steering by UE and UPF without providing a steering strategy from PCF:

-	Such autonomous steering mode provides flexibility to both the UE and the UPF to maximize the resource usage of the two accesses considering the situation of the link and UE/UPF status, so that the user can get best service experience, which cannot be reached by Rel-16 steering modes which focus on a single characteristic (e.g. RTT or weight) and fixed splitting rate.. The UE and UPF behavior may be the same as the existing modes within a certain period of time, such as active-standby, or load balance with some fixed weight, prefer 3GPP over WiFi, or prefer WiFi over 3GPP, in order to make full use of the transport resource, or satisfy the UE power saving requirement, etc.. Taking solution #12 into account, the UPF can consider the information provided by the UE for uplink steering when making decision of the steering behaviour for downlink.

2) 	Thresholds:

Thresholds provided to the UPF is a useful addition as proposed in Solution #2/#3:

-	Sol#3 suggest that PCF provides thresholds in PCC rules and that thresholds are set per IP Flow and per access.

-	There is no explicit relation between the thresholds described in Sol#3 and the QoS requirements for an SDF/application but the PCF when setting the threshold needs to consider the QoS requirements for an SDF/application. Thresholds parameter defined in  Sol#3 are independent parameters per access, but can also be set to the same thresholds value for both accesses, considering the QoS requirements are independent of access type. Sol#3 also proposes jitter thresholds which can be configured based on the application type. It is also proposed to introduce loss rate thresholds for non-GBR traffic. Although for now the Maximum Packet Loss Rate value is currently only applied to GBR, it doesn’t mean the packet loss impact can be ignored for the non-GBR traffic. Obviously, the more packets are lost, the worse service quality will be, not only for GBR but also for non-GBR.

3)	Enhanced PMF measurements:

-	The benefits of performing RTT measurements per QoS flow improves the precision of RTT measurements and has small impacts to the PMF protocol.

-	Jitter measurements reuses RTT measurement message for Jitter calculation. Some services such as game, IMS voice are sensitive to jitter, therefore it is possible that PCF can configure such threshold.

-	Packet loss rate measurements may be useful in case loss rate thresholds are introduced. In case a PMF echo packet is lost, the UE or the UPF can resend PMF request or carry the lost information in the subsequent PMF message. Details can be left to stage 3.

Solution 11:

Solution #11 proposes a new steering mode to take RTT difference into account. The functionality of RTT difference based steering mode can however be achieved through proper implementation of other steering modes by UE/UPF implementation simply taking the RTT difference into account when making traffic switching/splitting decisions.

Category C proposals:

Solution 4:

The Solution states that it is targeting “loss rate sensitive traffic, such as IMS signalling, video, and some TCP-based traffic.”. This redundant transport is only initiated when one access cannot satisfy the loss rate so it could be useful, which is much better than retransmission after packet loss.

Editor’s Note: It is FFS if there is requirement for a “Redundant steering mode” in rel-17.

Category D proposals:

Solution 12:

On UE-assisted traffic steering mode proposes to let the UE influence the steering mode and/or steering mode parameters. This may be based e.g. on UE battery consumption and battery level. Getting such feedback from the UE may be valuable to achieve a good user experience for ATSSS, but it needs to be ensured that operator control is maintained. For example, there should be support for the PCF to set the boundaries of the steering decisions by UE and UPF, e.g. by explicitly providing a list of the authorized steering modes that can be used for an SDF.

**** Next Change ****

8	Conclusions

Editor's note:	This clause will capture conclusions from the study.

8.X 	Interim Conclusions for KI#1: Steering Modes

It is proposed that:

-	The following principle is recommented for the normative phase:

-	The UE and the UPF may determine how to steer, switch or, if allowed, split traffic over each access for UL and DL directions, respectively, subject to e.g., link status, access conditions and UE states.

-	The UE distributes UL traffic based on UE’s decision on traffic split ratio or preferred access for UL transmission.

-	For the DL, the UE may request the UPF that the DL transmission match the UL transmission traffic distribution. The UPF then decides the DL transmission traffic distribution.

Editor’s Note: It is FFS whether new steering mode(s) need to be defined in Rel-17 that support the following list of performance criteria.

-	Thresholds are based on existing QoS parameters and inline with existing QoS model. Threshold values should be based on QoS requirements of the applications/SDFs. The thresholds are used to judge whether the access can be used to fulfil the traffic transmission. For example:

o	Maxiumum RTT: The value can be derived from PDB.

o	DL Maximum Packet Loss Rate: The value can e.g. be derived from the Maximum Packet Loss Rate (MPLR) or the PER

Editor’s note: It is FFS whether measurement on jitter needs to be supported.

-	These parameters should be the same for both 3GPP and non-3GPP accesses since QoS requirements are per SDF/service. Access specific thresholds may be studied in a later release, if justified.

- 	These ATSSS thresholds parameters could be either derived by PCF and provided as new parameters in the ”Multi-access” part of the PCC rule, or derived by SMF based on the rel-15/16 QoS parameters in the PCC rule.

Editor’s note: It is FFS whether the ATSSS QoS parameters/thresholds would be derived by PCF or SMF.

-	The SMF performs QoS Flow binding as usual, and derives ATSSS threshold parameters per QoS Flow, based on the PCC rules received from PCF.

-	UE and UPF uses these ATSSS threshold parameters for steering/switching/splitting decisions.

-	Thresholds are limited to the Load Balancing steering mode

Editor’s note: It is FFS whether thresholds could apply also to other steering modes than Load Balancing

-	In addition, PMF is enhanced to support RTT measurements per QoS Flow.

Editor’s note: It is FFS whether SMF provides QoS rules / N4 rules for per QoS Flow measurement or UE and UPF can override existing QoS rules / N4 rules to send PMF messaage over a specific QoS Flow.

**** End of Changes ****