3GPP TSG-WG SA2 Meeting #141E	S2-2007553r25

Elbonia, October 12 – 23, 2020	(Revision of S2-200xxxx)

Source:	NTT DOCOMO, Lenovo, Motorola Mobility, ZTE, NEC, Huawei, Ericsson. Matrixx

Title:	Key Issue #1 solution evaluation and interim conclusions

Document for:	Approval

Agenda Item:	8.4

Work Item / Release:	FS_eNS_ph2 / Rel-17

Abstract of the contribution: This contribution evaluates solutions addressing the KI#1.

# Discussion

The following set of evaluation criteria are used to evaluate the proposed solutions for KI#1 to support network slice related quota on the maximum number of UEs:

1)	Comply to the working assumptions and requirements as described in clause 4 of TR23.700-40.

2)	Address all the issues identified in the KI#1 as described in clause 5.1.

3)	Impact to the UE

4)	Impact to the CN, the serving PLMN, the home PLMN, e.g. AMF, NSSF, UDM, CHF, PCF in term of signalling interfaces, control flows and internal logic etc. including changes to the semantics of existing services/parameters.

5)	Impact to (R)AN

6) 	Impact to O&M

# Proposal

It is proposed to capture the summary of the analysis and the table of solution comparison and the interim conclusions in the TR23.700-40.

********************************  START OF 1st CHANGES **************************************

# 7	Overall Evaluation

## 7.1	Evaluation on solutions of KI#1

From all 10 solutions proposed for KI#1 (Solution #1, #2, #3, #4, #8, #9, #15, #18, #19, #38), some may have a complete solution and some not. But overall, we can summarize that there are three main functionalities for supporting quota management on the maximum number of UEs as described below.

- NW Slice information storage functionality: This functionality is responsible for storing a NW Slice information, which includes one or more of the following information.

- An indication whether a S-NSSAI is subject to the network slice quota management

- The maximum number of UEs for the S-NSSAI .

- NW Slice quota management functionality: This functionality is responsible for managing and updating NW Slice quotas of the maximum number of UEs in a S-NSSAI, which includes one or more of the following functionalities.

- Monitoring for counting, collecting and updating the number of UEs that have been registered for a S-NSSAI that is subject to the network slice quota management.

- NW Slice quota enforcement functionality: This functionality is responsible for enforcing a network slice SLA, which includes one or more of the following functionalities

- Accept or reject the UE registration request in a S-NSSAI by taking the network slice quota into account.

- In case of rejection, the function may provide a rejection cause and a back off time.

Centralized Quota Determination vs Decentralized Quota Determination: In a centralized quota determination, the NW Slice quota enforcement functionality checks every new UE registration against the quota at centralized quota enforcement point. In a decentralized quota determination, the NW Slice quota enforcement functionality checks every new UE registration against the quota at decentralized quota enforcement point. When the quota are consumed, the NW Slice quota enforcement functionality in the decentralized quota enforcement point interacts with the centralized quota enforcement points NW Slice quota enforcement functionality for additional instructions.

Editor’s note: It is FFS whether the NW Slice quota enforcement functionality is decentralized or centralized.

Table 7.1-1 illustrates 5GS system impacts for all solutions proposed for the KI#1. Moreover, it also compares where to place the afore-mentioned network slice quota functionalities in the 5G system.

Table 7.1-1: Key impacts of the solutions

|  |  | UE Impact |  | RAN Impact |  | 5GC CN Impact |  |  |  |  |  |  |  | Notes |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | Existing NF (Note 8) |  |  | New NF or new service operation in existing NF |  |  |  |  |  |  |
|  |  |  |  |  |  | NW Slice InformationStorage |  | NW Slice Quota management | NW Slice Quota Enforce | NW Slice Information Storage | NW Slice Quota Managment | NW Slice Quota Enforce |  |  |  |  |
| Sol#1 |  | Yes |  | No |  | UDR, PCF |  | PCF | PCF AMF(back-off timer handling) | - | - | - |  | (Note 1) (Note 5) |  |  |
| Sol#2 |  | Yes |  | No |  | - |  | - | AMF | NSQ |  | NSQ |  | (Note 1) (Note 5) (Note 7) |  |  |
| Sol#3 |  | Yes |  | No |  | NSSF |  | NSSF | NSSF, AMF | - | - | - |  | (Note 1) (Note 5) |  |  |
| Sol#4 |  | Yes |  | No |  | NWDAF |  |  | AMF, PCF (roaming case) | - | - |  | - |  |  | (Note 1) (Note 5) |  |
| Sol#8 |  | Yes |  | No |  | O&M, AMF |  | O&M, AMF | OAM AMF | - | - | - |  | (Note 1) (Note 2) (Note 5) |  |  |
| Sol#9 |  | Yes |  | No |  | UDM, NWDAF |  | NWDAF, CHF | AMF | - | - | - |  | (Note 1) (Note 5) |  |  |
| Sol#15 |  | Yes |  | No |  | - |  | - | AMF | - | - | - |  | (Note 3) |  |  |
| Sol#18 |  | No |  | No |  | SQM(NSSF for global) AMF for local- |  | AMF NSSF | AMF | - |  | - |  | (Note 4) (Note 5) (Note 7) |  |  |
| Sol#19 |  | No |  | No |  | NWDAF |  |  | - | QCF |  |  | QCF QEF |  |  | (Note 4) (Note 6) (Note 7) |  |
| Sol#38 |  | No |  | No |  | CHF |  | CHF | AMF, CHF | - | - | - |  | (Note 5) (Note 9) |  |  |
| NOTE 1: Solution requires a UE to support a (new/existing) rejection cause and a (new/existing) back-off timer due to the network slice quota has been reached. NOTE 2: Solution has an impact on O&M to support either a network slice quota monitoring and/or a network slice quota distribution (if any). Multi-vendor case is unclear on how it works. NOTE 3: Solution only addresses the aspect of back-off timer to be sent to the UE for network slice quota enforcement. NOTE 4: Solution does not describe whether the UE should be aware of a rejection cause due to a network slice quota has been reached. NOTE 5: Solution requires a change in both the H-PLMN and the V-PLMN to support a network slice quota monitoring and a network slice quota enforcement. NOTE 6: No descriptions of roaming aspect NOTE 7: Although the solution proposes a new NF, this new NF could also be deployed together with an existing NF. In such case, no new NF is needed. NOTE 8: Even when the existing NF is reused, the new NF service may need to be introduced. NOTE 9: Solution requires a AMF to support a (new/existing) rejection cause due to the network slice quota has been reached. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

From Table 7.1-1 above, one can derive a commonality among those solutions as following:

-	All solutions do not require changes in RAN

-	All solutions propose to store an information related to the network slice quota information in the CN and the UE is not aware of it.

- 	All solutions propose to monitor the number of UEs accessing a network slice in the CN and to enforce the quota in the CN. Difference among those solutions are a) where to store the network slice quota information, b) where to place the network slice quota management and the network slice quota enforcement. There are two main approaches, one is to put them into an existing 5GC network function or introducing a new network function/new service operation to be deployed in the existing 5GC network function.

- 	Almost all solutions imply some form of interaction by the AMF with another function which performs the counting.

Furthermore, there are some other aspects that we could also draw some remarks, for example,

-	Rejection cause / Back-off timer: When a network slice quota in terms of number of UEs is overflown, a CN NF sends a rejection to the UE requesting for a S-NSSAI. To suppress further signalling load for a subsequent request for the same S-NSSAI, the CN NF may provide a back-off timer to the UE. The CN NF responsible for sending a rejection for the requested S-NSSAI and optionally the back-off timer for the rejected S-NSSAI is the AMF. It is up to the Stage 3 to determine whether a new rejection cause and back-off timer or an existing rejection cause and back off timer should be used.

********************************  START OF 2nd CHANGES **************************************

# 8	Conclusions

Editor's note:	This clause will capture conclusions from the study.

## 8.1	KI#1 Interim conclusion

To enable a 5GS to support network slice related quota on the maximum number of UEs, no change is required in the RAN. The following new functionalities in the 5GS are needed, i.e.

-	Storing of network slice related information: If a network slice is subject to a network slice quota checking on a maximum number of UEs, it is assumed that the O&M should have for this network slice a) the information that indicates it is subject to the network slice related quota management, and b) the information of the quota of maximum number of UEs. To enable the network slice related quota enforcement, this information is configured and stored to one network functions in 5GC.

Editor's note:	It is FFS which network function in 5GC needs to be configured to store the network slice related quota information and how it gets the network slice related quota information.

-	Managing and updating the network slice related quota on maximum number of UEs registered for the network slice: This functionality is part of the 5GC and it manages the NW Slice quota of maximum number of UEs in a S-NSSAI, monitors the current number of UEs being registered for the network slice subject to a network slice quota checking.

Editor's note:	It is FFS which network function in 5GC should manages the NW Slice quota of the maximum number of UEs in a network slice.

-	Enforcing the network slice related quota on the maximum number of UEs: This functionality is part of the 5GC and it controls the registration request on the S-NSSAI subject to the quota management by accepting or rejecting the request. In case of rejection, the function may provide a rejection cause and optionally with a back-off timer.

Editor’s note: 	It is FFS which network function in 5GC (new NF or existing NF) should enforce the network slice related quota on the maximum number of UEs.

Editor’s note: It is FFS whether the NW Slice quota enforcement functionality is decentralized or centralized.

NOTE:	Whether to use an existing rejection cause and back-off timer or a new rejection cause and back-off timer, this is to be determined in Stage 3.