3GPP TSG-WG SA2 Meeting #141E	S2-2007559r04

Elbonia, October 12 – 23, 2020	(Revision of S2-200xxxx)

Source:	NTT DOCOMO, Lenovo, Motorola Mobility

Title:	Key Issue #2 solution evaluation and interim conclusions

Document for:	Approval

Agenda Item:	8.4

Work Item / Release:	FS_eNS_ph2 / Rel-17

Abstract of the contribution: This contribution evaluates solutions addressing the KI#2.

# Discussion

The following set of evaluation criteria are used to evaluate the proposed solutions for KI#2 to support network slice related quota on the maximum number of PDU Sessions:

1)	Comply to the working assumptions and requirements as described in clause 4 of TR23.700-40.

2)	Address all the issues identified in the KI#2 as described in clause 5.2.

3)	Impact to the UE

4)	Impact to the CN, the serving PLMN, the home PLMN, e.g. AMF, NSSF, UDM, CHF, PCF in term of signalling interfaces, control flows and internal logic etc. including changes to the semantics of existing services/parameters.

5)	Impact to (R)AN

6) 	Impact to O&M

# Proposal

It is proposed to capture the summary of the analysis and the table of solution comparison and the interim conclusions in the TR23.700-40.

********************************  START OF 1st CHANGES **************************************

# 7	Overall Evaluation

## 7.2	Evaluation on solutions of KI#2

From all 13 solutions proposed for KI#2 (Solution #5, #10, #11, #18, #19, #32, #35, #36, #38), some may have a complete solution and some not. It is noted that Sol#35 describes a solution for KI#2 on a different aspect, i.e., considering a service type associated with the PDU Session to be established and use it for differentiation of service(s) and even for prioritization of granting a PDU Session when the status of slice is close to the quota limitation. Since Sol#35 could be put on top of any other solutions, Sol#35 is then not listed for comparison below.

Looking at all these solutions for KI#2, we can summarize that there are three main functionalities for supporting quota management on the maximum number of PDU Sessions as described below.

- NW Slice quota information storage functionality: This function is responsible for storing a NW Slice quota information, which consists one of the following information.

- An indication whether a S-NSSAI is subject to the network slice quota checking

- The global maximum number of PDU Sessions for the S-NSSAI.

- NW Slice quota management functionality: This function is responsible for managing and controlling global NW Slice quotas of the maximum number of UEs in a S-NSSAI, which consists one or more of the following functionalities.

- Monitoring for counting and collecting the number of PDU Sessions that have been established in a S-NSSAI that is subject to the network slice quota checking

- Sending an indication whether the network slice quota has been reached.

- NW Slice quota enforcement functionality: This function is responsible for enforcing a network slice SLA, which consists one or more of the following functionalities.

- Accept or reject the PDU Session Establishment Request for the S-NSSAI subject to the network slice quota checking by taking into account the network slice quota on the maximum number of PDU Sessions and the monitored number of established PDU sessions.

- In case the global network slice quota has been reached, function may accept or reject the request. In case of rejection, the function may provide a rejection cause and a back off timer.

- Centrally Global Quota check vs Local Quota Check: There are 2 categories of solutions. First category, where the NW Slice quota enforcement functionality checks every new UE registration against the global S-NSSAI quota. The second category distributes quota to all NW Slice quota enforcement functionalies and where every new UE registration is checked against local quota. Only when the Local quota is reached then the NW Slice quota enforcement functionality checks against the global quota for additional instructions.

Table 7.2-1 below shows an overview of key impacts of all solutions and in particular where the above functions are placed in the 5G system.

Table 7.2-1: Key impacts of the solutions

|  | UE Impact | RAN Impact | 5GC CN Impact |  |  |  |  |  | Notes |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Existing NF |  |  | New NF or new service operation in existing NF |  |  |  |
|  |  |  | Quota Storage | Quota Management | Quota Enforce | Quota Storage | Quota Management | Quota Enforce |  |
| Sol#5 | Yes | No | NWDAF |  | NWDAF SMF, PCF (roaming case) | - | - | - | (Note 1) (Note 5) |
| Sol#6 | Yes | No | UDR, PCF | PCF | PCF, SMF | - | - | - | (Note 1) (Note 5) |
| Sol#7 | Yes | No | UDR, PCF | PCF | PCF, SMF | - | - | - | (Note 1) (Note 5) |
| Sol#8 | Yes | No | O&M, AMF | O&M, AMF | O&M AMF | - | - | - | (Note 1) (Note 2) (Note 5) |
| Sol#9 | Yes | No | UDM, NWDAF | NWDAF, CHF | NWDAF AMF | - | - | - | (Note 1) (Note 5) |
| Sol#10 | Yes | No | - | - | NSQ SMF | NSQ |  | - | (Note 1) (Note 5) (Note 7) |
| Sol#11 | Yes | No | NRF | NRF | AMF | - | - | - | (Note 1) (Note 5) |
| Sol#18 | No | No | - | - | SQM AMF | SQM |  | - | (Note 4) (Note 5) (Note 7) |
| Sol#19 | No | No | NWDAF |  | - | QCF |  | QCF QEF | (Note 4) (Note 6) (Note 7) |
| Sol#32 | No | No | SMF | NF |  | NF |  |  | (Note 4) (Note 6) |
| Sol#38 | No |  | CHF | CHF | SMF, CHF | - | - | - | (Note 8) (Note 2) (Note 5) |
| NOTE 1: Solution requires a UE to support a (new/existing) rejection cause and a (new/existing) back-off timer due to the network slice quota has been reached. NOTE 2: Solution has an impact on O&M to support either a network slice quota monitoring and/or a network slice quota distribution NOTE 3: Solution only addresses the aspect of back-off timer to be sent to the UE for network slice quota enforcement. NOTE 4: Solution does not describe whether the UE should be aware of a rejection cause due to a network slice quota has been reached. NOTE 5: Solution requires a change in both the H-PLMN and the V-PLMN to support a network slice quota management and a network slice quota enforcement. NOTE 6: No descriptions of roaming aspect. NOTE 7: Although the solution proposes a new NF, this new NF could be deployed together with existing NF. In such case, no new NF is needed. NOTE 8: Solution requires a SMF to support a (new/existing) rejection cause due to the network slice quota has been reached. |  |  |  |  |  |  |  |  |  |

From Table 7.2-1 above, one can derive a commonality among those solutions as following:

-	All solutions do not require changes in RAN

-	All solutions propose to store an information related to the network slice quota information in the CN.

- 	All solutions propose to monitor the number of PDU Sessions associated with a network slice in the CN and to enforce the quota in the CN. Difference among those solutions are a) where to store the network slice quota information, b) where to place the network slice quota management and the network slice quota enforcement. There are two main approaches, one is to put them into an existing 5GC network function or introducing a new network function/new service operation to be deployed in the existing 5GC network function.

- 	Almost all solutions imply some form of interaction of the SMF with another function which performs the counting.

Furthermore, there are some other aspects that we could also draw some remarks, for example,

-	Rejection cause + Back-off timer: When a network slice quota in terms of number of PDU Sessions is reached, and a CN NF sends a rejection to the UE’s PDU Session Establishment Request for the network slice, it is up to the Stage 3 to determine whether a new rejection cause or an existing rejection cause should be used. To suppress further signalling load for a subsequent request of the network slice, the CN NF may provide a back-off timer to the UE.

********************************  START OF 2nd CHANGES **************************************

# 8	Conclusions

Editor's note:	This clause will capture conclusions from the study.

## 8.X	KI#2 Interim conclusion

To enable a 5GS to support network slice related quota on the maximum number of PDU Sessions, the following new functionalities in the 5GS are needed, i.e.

-	Storing of network slice related quota information: If a network slice is subject to a network slice quota checking on a maximum number of PDU Sessions, it is assumed that the O&M should have for this network slice a) the information that it is subject to the network slice related quota checking, and b) the information of the quota of maximum number of PDU Sessions. To enable the network slice related quota enforcement, this information is configured and stored to one or more network functions in 5GC.

Editor's note:	It is FFS which network function located in 5GC needs to be configured to store the network slice related quota information and how it gets the network slice related quota information.

NOTE 1:	It is preferable that the network slice quota on the maximum number of UEs (KI#1) and the network slice quota on the maximum number of PDU Sessions (KI#2) share a common storage.

-	Managing a number of PDU Sessions established in a S-NSSAI: This functionality is part of the 5GC and it manages and controls the network slice related quota on maximum number of PDU sessions established in a S-NSSAI and monitors the current number of PDU Sessions successfully established in the network slice subject to a network slice quota checking on a maximum number of PDU Sessions.

Editor's note:	It is FFS which network function located in 5GC should manage and control a number of PDU Sessions successfully established in the network slice.

NOTE 2:	It is preferable that the number of UEs (KI#1) and the number of PDU Sessions (KI#2) share a common NW Slice management function.

-	Enforcing the network slice related quota on the maximum number of PDU Sessions: This functionality is part of the 5GC and it enforces (i.e., accept or reject) the Requested PDU session of a S-NSSAI by taking the network slice quota and the monitored number of the registered UEs of the S-NSSAI into account. In case, the quota on the maximum number of PDU Sessions is reached, this functionality in 5GC may request the update of the quota or rejects the PDU establishment request with a rejection cause and optionally with a back-off timer. In addition, this functionality may inform the charging system (e.g. CHF) that the quota has been reached for a proper charging purpose.

-  Centrally Global Check count is equivalent to Local Quota check with 0 quota distributed to the NW Slice quota enforcement functionality, notwithstanding the different behaviour in the enforcing function.

Editor's note:	It is FFS which network function located in 5GC should enforce the network slice related quota on the maximum number of PDU Sessions.

NOTE 3:	Whether to use an existing rejection cause or a new rejection cause, this is to be determined in Stage 3.