3GPP TSG-WG SA2 Meeting #141E e-meeting 	S2-2007615r01

Elbonia, October 12 – 23, 2020

Source:	Orange

Title:	KI #2: Evaluation and interim conclusions

Document for:	Approval

Agenda Item:	8.1

Work Item / Release:	FS_eNA_ph2 / Rel-17

Abstract of the contribution: This contribution proposes evaluation and interim conclusions on KI#2

# Discussion

This contribution proposes evaluation and conclusions for KI#2 “Multiple NWDAF instances”.

The solutions proposed are: 9 to 26, 35, 39, 53, 56 to 60, 69.

This contribution identifies the themes proposed in the various solutions, and assesses the impacts of each solution on 5GC.

A) Hierarchy of NWDAFs (solutions 9, 10, 11, 14, 15, 16, 17, 18, 21, 22, 24, 25, 35, 39, 53, 58, 60)

B) Migration of one NWDAF instance to another NWDAF instance (NWDAF handover) (solutions 26, 57, 59)

C) Inter-NWDAF cooperation and data aggregation (solutions 10, 13, 14, 19, 20, 24, 56, 58, 60)

# 2.	Proposal

It is proposed to apply the following changes to the TR 23.700-91.

* * * * Start of changes * * * *

## 7.2	Key Issue #2: Multiple NWDAF instances

The Key Issue #2 aims to address the following topics, with relevant enhancements/extensions concerning analytics types, input/output data analytics and/or procedures covered by eNA (Rel-16):

1)	Relevant deployment scenarios of multiple NWDAF instances

2)	Need for architectural changes e.g. hierarchies

3)	Various cooperation between NWDAF instances

The different solutions have indirectly addressed these topics. The solutions can be classified into three categories according to the mechanisms and issues that they mainly address:

A)	Hierarchy of NWDAFs, or other specialized NFs (e.g., data collection, storage) (solutions 9, 10, 11, 14, 15, 16, 17, 18, 21, 22, 24, 25, 35, 39, 53, 58, 60)

B)	Migration from one NWDAF instance to another NWDAF instance (NWDAF handover) (solutions 26, 57, 59)

C)	Inter-NWDAF cooperation and data aggregation (solutions 10, 12, 13, 14, 19, 20, 24, 56, 58, 60)

Table 7.2-1: Classification of solutions for KI#2

| Solution | Additional information | Topic |
| --- | --- | --- |
| #09: Data Management Framework for 5GC | DCCF, Data Adaptators, Data repository | A |
| #10: Handling of mixed and distributed NWDAF deployments | Discovery of relevant NWDAF | A,C |
| #11: Two-level Hierarchical NWDAFs Architecture | Architecture with two NWDAF levels | A |
| #12: Support for NWDAF interactions within SBA framework | Improved registration of NWDAF in NRF. NWDAF dedicated to entites, areas. | C |
| #13: Time Coordination for Multiple NWDAFs | Waiting time coordinated by NF consumer | C |
| #14: Support flexible Analytics aggregation for multiple NWDAFs | NWDAF acting as aggregation point | A, C |
| #15: Data Collection Coordination | DCCF function | A |
| #16: Roles and inter-NWDAF instance cooperation from lower level NWDAF to upper level NWDAF in hierarchical architecture | Architecture with up to three NWDAF levels | A |
| #17: Alternatives for Interactions among Hierarchical NWDAFs | Architecture with up to three NWDAF levels | A |
| #18: Roles and inter-NWDAF instance cooperation from upper level NWDAF to lower level NWDAF in hierarchical architecture | Architecture with up to three NWDAF levels | A |
| #19: Multiple NWDAFs interactions for analytics consumption and composition related to large areas | Composition of analytics by NWDAFs | A, C |
| #20: UE abnormal behaviour analytics interaction between multiple NWDAF instances | Use of NWDAF specialized in certain analytics | A, C |
| #21: Inter-NWDAF instance cooperation based on NWDAF profile | NWDAF hierarchy with low-level NWDAF collocated with one NF | A |
| #22: mitigation of the load for Data Collection |  | A |
| #23: flexible data collection and data analysis for hierarchical NWDAF architecture |  | A |
| #24: Federated Learning among Multiple NWDAF Instances | Exchange of models via model server | A+C |
| #25: Exposing UE mobility analytics for multiple NWDAFs case |  | A |
| #26: Reselection of NWDAF | Exchange of relocation information between NWDAFs | B |
| #35: Using a dedicated NF for data collection | DCNF : data collection | A |
| #39: Efficient data management for minimizing signalling. | DRSF : data storage common to NWDAFs | A |
| #53: Support of Multiple NWDAF with Efficient Cooperation | Exchange of profile and information between NWDAFs | A,C |
| #56: Trained Data Model Sharing between NWDAF instances |  | C |
| #57: Reselection of NWDAF due to mobility change | Exchange of NWDAF relocation information | B |
| #58: Efficient Service for Data Collection in NWDAF-to-NWDAF Interactions | Exchange of NWDAF information | A,C |
| #59: Analytics handover | Exchange of NWDAF relocation information | B |
| #60: Distributed NWDAFs deployment and Aggregation Function |  | A,C |
| #69: NWDAF associated to the User-plane | NWDAF collocated to UPF. Registration NRF. | A |

* * * * Next changes * * * *

## 8.2	Key Issue #2: Multiple NWDAF instances

For Inter-NWDAF cooperation, it is recommended to perform normative work based on solutions 13, 19, 20 and possibly 10 (restricted to a non-hierarchical architecture).

Editor's Note: Which of these solutions, or components thereof, will be used as a basis for normative work is FFS.

For migration of one NWDAF instance to another NWDAF instance (NWDAF handover), it is recommended to perform normative work based on solutions 26, 57 and 59.

Editor's Note: Which of these solutions, or components thereof, will be used as a basis for normative work is FFS.

* * * * End of changes * * * *