SA WG2 Meeting #141E	S2-2007701r06

12 - 23 October 2020, Elbonia

Source:	Nokia, Nokia Shanghai Bell

Title:	KI #2, Interim conclusion – re-selection category

Document for:	Discussion/Approval

Agenda Item:	8.1

Work Item / Release:	FS_eNA_Ph2 / Rel-17

Abstract of the contribution: Proposes interim conclusions related to NWDAF re-location.

1		Discussion

In Solution #26 the old NWDAF notifies the NWDAF service consumer about the relocation. In solutions #10, #57, #59 the NWDAF service consumer is informed by the target NWDAF, which is inline with AMF relocation procedures as specified in Release-16.

Solutions #26 and #57 propose a new NWDAF AnalyticsRelocation service, while solutions #10 and #59 re-use existing service operations. Thereby, in solution #10 the target NWDAF requests the analytics context from the source NWDAF ("pull"), whereas in solution #59 the source NWDAF will include the analytics context in the handover or handover preparation request to the target NWDAF ("push").

Solution #10 describes a procedure, where the re-location is triggered by AMF (due to UE mobility). Solutions #26, #57, #59 describes re-location being triggered by the NWDAF (due to UE mobility, load balancing, etc.).

Based on those observations, a set of principles is derived for the KI#2 conclusions.

2	Proposal

It is proposed to update TR 23.700-91 with the following conclusions.

*** Start Change (all new text) ***

*** Next Change (all new text) ***

8.2	Key Issue #2: Multiple NWDAF instances

8.2.x	Re-selection of NWDAF (category #2)

The following principles are proposed for the normative work on "Re-selection of NWDAF":

- a)	Extend or re-use existing Release-16 service operations, i.e. do not introduce new AnalyticsRelocation service.

- b)	Support different triggers conditions for the NWDAF re-selection corresponding to UE mobility, load balancing, etc.

- c)	Support the handover of analytics subscriptions between NWDAFs.

- d)	Support analytics context transferring:

- - For NWDAF initiated relocation scenario: NWDAF re-selection is initiated by the source NWDAF, and the analytics context is pushed to the target NWDAF from the source NWDAF;

- - For consumer NF relocation scenario: NWDAF re-selection initiated by the target consumer NF, and the analytics context is pulled by the target NWDAF from the source NWDAF.

NOTE:	Analytics context includes the following information:

Information related to entities that are subscribed to receive analytics.

Historical data available at the source NWDAF and related to the Analytics to be handed over to the target NWDAF (including not only the raw data collected from the data source(s), but also "analytics metadata" such as the time period of the collected data, information about the data source(s), etc.).

Pending Analytics (i.e. not yet notified to the consumer) or historical Analytics Output information – if available at the source NWDAF.

- e)	The target NWDAF notifies the NWDAF service consumer about the re-location.

*** End Change ***