SA WG2 Meeting #141E (e-meeting)	S2-2007716

Elbonia, October 12 – 23, 2020

Source:	vivo Mobile Communications Ltd

Title:	Interim conclusions

Document for:	Discussion/Approval

Agenda Item:	8.12

Work Item / Release:	FS_eV2XARC_Ph2/Rel.17

Abstract of the contribution: This contribution adds some interim highlevel conclusions.

- Background

Solution#1 to Solution#6 in TR 23.776 are about NR PC5 DRX operations. It is proposed to capture some interim conclusions based on these solutions.

.

* * * * Start of Change * * * *

7	Conclusions

For Key Issue #1 (Support of QoS aware NR PC5 power efficiency for pedestrian UEs), regarding NR PC5 DRX operations the followings are taken as interim conclusion:

-	The AS layer determine the PC5 DRX for V2X communication over PC5 reference point to enable pedestrian UE power saving.

-	The V2X layer may provide the PC5 DRX assistant information to the Access Stratum (AS) layer

Editor's note:	What the PC5 DRX assistant information provided to the AS layer by the V2X layer is FFS.

Editor's note:	What case type the PC5 DRX assistant information applies to is FFS.

Editor's note:	Conclusions need to be completed.

* * * * End of Change * * * *