SA WG2 Meeting #141E (e-meeting)	S2-2007716r06

Elbonia, October 12 – 23, 2020

Source:	vivo Mobile Communications Ltd, LG Electronics

Title:	Interim conclusions

Document for:	Discussion/Approval

Agenda Item:	8.12

Work Item / Release:	FS_eV2XARC_Ph2/Rel.17

Abstract of the contribution: This contribution adds some interim highlevel conclusions.

- Background

Solution#1 to Solution#6 in TR 23.776 are about NR PC5 DRX operations. It is proposed to capture some interim conclusions based on these solutions.

.

* * * * Start of Change * * * *

7	Conclusions

For Key Issue #1 (Support of QoS aware NR PC5 power efficiency for pedestrian UEs), regarding NR PC5 DRX operations the following principles are taken as the interim conclusion:

- 	The V2X application layer provides inputs for V2X layer to determines PC5 DRX assistance information

Editor's note:	It is FFS whether the inputs described in TS 23.287 [5] (e.g. V2X Application Requirements) are sufficient or additional inputs (e.g. traffic pattern) are needed from the V2X application layer.

-	The Access Stratum (AS) layer determines the PC5 DRX for V2X communication over PC5 reference point to enable pedestrian UE power saving.

-	The V2X layer may provide the PC5 DRX assistance information to the AS layer.

Editor's note:	Contents of the PC5 DRX assistance information provided to the AS layer by the V2X layer is FFS.

Editor's note:	It is FFS if the PC5 DRX assistance information is per mode of communication (i.e. unicast/broadcast/groupcast).

-	The Access Stratum (AS) layer determines the PC5 DRX for V2X communication over PC5 reference point to enable pedestrian UE power saving.

-	The AS layer provides the applied PC5 DRX information to the V2X layer.

-	For unicast, the PC5 DRX is negotiated between the UEs in AS layer, pending on the feedback from RAN2.

Editor's note: 	The design principles need to be evaluated by RAN WGs.

Editor’s Note:	It is FFS whether DRX related parameters are needed to be (pre-)configured on the UE to assist the V2X layer determine the PC5 DRX assistance information.

Editor's note: The design principles need to be evaluated by RAN WGs.

* * * * End of Change * * * *