SA WG2 Meeting #141E (e-meeting)	S2-2007734r02

Elbonia, Oct. 12th – 23rd, 2020

Source:	Interdigital Inc.

Title:	Evaluation and Interim Conclusion for KI #3 on handling of PDU Session related attributes for L3 NW relay

Document for:	Discussion/Approval

Agenda Item:	8.8

Work Item / Release:	FS_5G_ProSe/Rel.17

Abstract of the contribution: This contribution proposes an evaluation and conclusion for KI #3 on handling of PDU Session related attributes for L3 NW relay.

# Discussion

For the following aspects in KI#3

-	How to support end-to-end requirements between Remote UE and the network via a UE-to-Network Relay, including QoS (such as data rate, reliability, latency) and the handling of PDU Session related attributes (e.g. S-NSSAI, DNN, PDU Session Type and SSC mode).

There are 3 candidate solutions for Layer 3 UE-to-Network relay case, i.e. sol#28, sol#38, sol#42.

Solution #28 proposes the Remote UE provides PDU session parameters in Direct Communication Request message, then the UE-to-Network relay establishes/modifies PDU session based on the Remote UE’s request.

Solution #38 proposes "5G ProSe Layer 3 UE-to-Network" Connection Capability in UE-to-Network relay’s URSP for Remote UE traffic.

Solution #42 proposes the Remote UE provides the traffic descriptors to the UE-to-Network relay and the UE-to-Network relay establishes/modifies PDU session based on UE-to-Network relay’s URSP rule.

Solution#38 uses the same set of PDU session parameters for all Remote UEs and the UE-to-Network relay cannot identify the different PDU session requirements from the Remote UEs.

Solution#28 and Solution#42 allow the UE-to-Network relay to establish PDU sessions with different sets of PDU session parameters based on request from Remote UE.

The difference between solution#28 and solution#42 is

- In solution#28, the Remote UE provides PDU session requirements to the UE-to-Network relay;

- In solution#42, the Remote UE provides Traffic descriptor to the UE-to-Network relay then the UE-to-Network relay retrieves the PDU session parameters based on UE-to-Network Relay’ URSP rules.

Table 1: Comparison of sol#28 and sol#42

|  | Solution#28 | Solution#42 |
| --- | --- | --- |
| Consistency | - PDU session requirement provided by the Remote UE only depends on the application launched in the Remote UE; no matter which UE-to-Network relay is selected | - Different UE-to-Network relay may retrieve different sets of PDU session parameters even though the Remote UE provides the same Traffic descriptor. |
| Privacy | - Remote UE only provides PDU session requirement to the UE-to-Network relay (no privacy sensitive data) | - Remote UE provides privacy sensitive data to the UE-to-Network relay, e.g. Remote UE’s OS info (OSId), Remote UE’s App info (OSAppId), Remote UE’s traffic info (Destination FQDN, protocol ID of the protocol above IP), etc. |
| Compatibility | - Remote UE provides standardized PDU session info, e.g. PDU session type, SSC mode, which can be understood well by the UE-to-Network relay even if the Remote UE and the UE-to-Network relay belong to different PLMNs. | - Remote UE provides Traffic descriptor which may not be understood by the UE-to-Network relay, e.g. OSAppId_A may only be configured in PLMN_A (Remote UE) and may not be configured in the PLMN_B (UE-to-Network relay) |
| Efficiency | - Remote UE may only provide necessary PDU session requirement, e.g. only provide PDU session type | - In all case, the Remote UE provides all available information related to the Traffic Descriptor, no matter which one will be used by the UE-to-Network relay |

Considering the above table, in all Consistency/Privacy/Compatibility/Efficiency aspects, it proposes solution#28 is selected for Layer 3 UE-to-Network relay’ normative work on the handling of PDU Session related attributes (e.g. S-NSSAI, DNN, PDU Session Type and SSC mode).

Considering the default PDU session parameters for the Layer 3 UE-to-Network relay, e.g. the case that the Remote UE doesn’t provide any/all PDU session requirements, Solution #38 may also be selected for Layer 3 UE-to-Network relay normative work on default PDU session parameters configuration.

# Proposal

It is proposed to document the evaluation and interim conclusions for Key Issue #3 in TR 23.752.

* * * * First Change * * * *

## 7.3	Key Issue #3: Support of UE-to-Network Relay

For the handling of PDU Session related attributes (e.g. S-NSSAI, DNN, PDU Session Type and SSC mode), there are 3 solutions for Layer 3 UE-to-Network relay case, i.e. sol#28, sol#38, sol#42.

- Solution#28 and Solution#42 allow UE-to-Network relay to establish PDU sessions with different sets of PDU session parameters based on request from Remote UE.

- Solution#38 uses the same set of PDU session parameters for all Remote UEs and the UE-to-Network relay cannot identify the different PDU session requirements from the Remote UEs.

Table 1: Comparison of sol#28 and sol#42

|  | Solution#28 | Solution#42 |
| --- | --- | --- |
| Consistency | - PDU session requirement provided by the Remote UE only depends on the application launched in the Remote UE; no matter which UE-to-Network relay is selected | - Different UE-to-Network relay may retrieve different sets of PDU session parameters even though the Remote UE provides the same Traffic descriptor. |
| Privacy | - Remote UE only provides PDU session requirement to the UE-to-Network relay (no privacy sensitive data) | - Remote UE provides privacy sensitive data to the UE-to-Network relay, e.g. Remote UE’s OS info (OSId), Remote UE’s App info (OSAppId), Remote UE’s traffic info (Destination FQDN, protocol ID of the protocol above IP), etc. |
| Compatibility | - Remote UE provides standardized PDU session info, e.g. PDU session type, SSC mode, which can be understood well by the UE-to-Network relay even if the Remote UE and the UE-to-Network relay belong to different PLMNs. | - Remote UE provides Traffic descriptor which may not be understood by the UE-to-Network relay, e.g. OSAppId_A may only be configured in PLMN_A (Remote UE) and may not be configured in the PLMN_B (UE-to-Network relay) |
| Efficiency | - Remote UE may only provide necessary PDU session requirement, e.g. only provide PDU session type - Beside the Remote UE, the Relay UE may also need to check URSP rule for default PDU session parameters | - In all case, the Remote UE provides all available information related to the Traffic Descriptor, no matter which one will be used by the UE-to-Network relay |

* * * * Second Change * * * *

## 8.3	Key Issue #3: Support of UE-to-Network Relay

For key issue#3 “Support of UE-to-Network Relay”, it is concluded that for handling of PDU session parameters:

- For Layer 3 UE-to-Network relay, solution#28 is selected for Layer 3 UE-to-Network relay normative work  on the handling of PDU Session related attributes (e.g. S-NSSAI, DNN, PDU Session Type and SSC mode).

- For Layer 3 UE-to-Network relay, Solution #38 is also selected for default PDU session parameters configuration, which is used when the Remote UE doesn’t provide any/all PDU session requirements .

* * * * End of Changes * * * *