SA WG2 Meeting #141E (e-meeting)	S2-2007757r05

# 12 – 23 October, 2020, Electronic, Elbonia

Source: 	Nokia, Nokia Shanghai Bell, Ericsson, Huawei, HiSilicon

Title: 	KI#1, Conclusion – architecture approach for AAA support

Document for: 	Agreement (P-CR)

Agenda Item: 	8.3

Work Item / Release:	FS_eNPN / Rel-17

Abstract of the contribution: This contribution proposes to update the KI#1 conclusions on Enhancements to Support SNPN along with credentials owned by an entity separate from the SNPN.

# 1 Discussion

As part of the Ki#1 solutions also the deployment of a AAA has been considered. This point raises some general questions related the impact on existing 5G NFs.

In the framework of Ki#1 also deployment variants have been discussed, in which the entity separate from the SNPN wants to utilize a AAA server for UE authentication.

In this context the following issues need to be considered:

- In a 5GS subscription related functionality includes Authentication, Subscriber Data Management and Context Management

- By intention (e.g. different from LTE legacy) the 5G architecture distributes this functionality on two 5G NFs. While AUSF offers authentication-based functions, UDM takes care about Subscriber Data and Context Management.

- A AAA server is per se used only for authentication purposes

Different approaches have in discussed related to the realization of Subscriber Data and Context Management in combination with a AAA server.

Sol#4 proposes two different architectural approaches for the handling of subscription related functionality.

In the first architectural approach, called "AMF triggered EAA of UEs with E-Sub" (cf. Figure 6.4.3.1) 5G Authentication, Subscriber Data Management and Context Management functions are provided by a new NF, called AAA-P. The AAA-P acts as a proxy towards the AAA server with regard to all three interfaces.

In the second architectural approach, called "UDM triggered EAA of UEs with E-Sub" (cf. Figure 6.4.3.2) the AUSF provides 5G Authentication function. In case of a subscriber identity, which is associated with the AAA server, the AUSF forwards the EAP messages to the AAA server, which is acting as the EAP server. 5G Subscriber Data and Context Management functions are handled by a 5G UDM NF.

Sol#8 is similar to the second approach in Slo#4 in the sense that 5G Authentication function is handled by AUSF (which forwards EAP messages to AAA, if necessary), whereas Subscriber Data and Context Management are handled by UDM.

We believe that integration of a AAA server should have minimal impact on the 5GS. This means that, if possible, no new NFs should be introduced and that also changes on interfaces between existing NFs should be minimized.

This paradigm is fulfilled in case of the second approach in Sol#4 and in Sol#8.

# 2 Proposal

It is proposed to update Conclusion Section of TR 23.700-07 as follows

*** Start Change ***

#### 8.1.X.X	Conclusions for scenario where the SNPN offers connectivity for UE(s) with credentials owned by separate entity offering AAA Server

The scenario where the separate entity owning the credentials has only an AAA Server is supported. It is proposed to adopt the following conclusion principles:

-	The SNPN will host a function (e.g. enhanced AUSF or new NF) supporting primary authentication and authorization of SNPN UEs that use credentials from the AAA Server.

NOTE:	Whether to use a new NF or enhanced AUSF will be determined based on feedback from SA3.

-	The function (see above) forwards EAP messages to the AAA server that is acting as the EAP Server for the purpose of primary authentication and authorization.

Editor's note:	Details of External UE ID used for this primary authentication and authorization and related aspects are FFS.

-	The UDM supports common mobility management and session management subscription information that is needed at the time of Registration and Session Management procedure.

Editor's note:	It is FFS if the AAA server supports providing the subscription information needed for registration and session management procedure.

Editor's note:	It is FFS if subscription information for individual UEs (with credentials stored in AAA) are supported in the UDM.

-	After completing the primary authentication and authorization successfully, the AMF or SMF can retrieve the SNPN UE subscription data (i.e. mobility management and session management subscription) using SUPI from UDM.

-

Editor's Note: 	Furthermore, SA2 should decide, if interfaces between existing/enhanced AUSF (or new NF) and AAA server should be in scope of SA2. The architecture to support AAA interfaces to the AAA server will be revised based on SA3's feedback.

Editor’s note: The Registration procedure supporting AAA interfaces towards the AAA server will be revised based on input from SA3 including the aspect which subscription data the SNPN should use in this scenario.