SA WG2 Meeting #141E (e-meeting)	S2-2007764r01

12 – 23 October, 2020, Elbonia

Source:	NEC

Title:	KI#7 Sol#31: Solution #31 update

Document for:	Approval

Agenda Item:	8.4

Work Item / Release:	FS_eNS_Ph2 / Rel-17

Abstract: This pCR proposes update to solution #31 of KI#7

# 1. Background

This contribution proposes the following updates to solution #31 ‘Steering the UE to a network slice in a different frequency band’ for Key Issue #7 in TR23.700-40 v1.0.0

******* start of the change ********

## 6.31	Solution #31: Steering the UE to a network slice in a different frequency band

### 6.31.1	Introduction

This solution addresses the below requirements from Key Issue #7: Support of 5GC assisted cell selection to access network slice.

-	How does 5GS steer UEs to a 5G-AN (e.g. a specific frequency band) that can support the network slices that the UE can use.

-	What information does 5GS need to take a decision to steer UE to a proper 5G-AN.

### 6.31.2	High Level Description

The solution is based on the following high-level assumptions and principles:

-	The UE is allocated Allowed NSSAI which can contain S-NSSAIs supported in different frequency bands, however all S-NSSAIs are supported in all Tracking Areas or the Registration Area.

### 6.31.3	Procedures

Figure 6.31.3.1-1 - Steering the UE to a network slice in different frequency band

1)	The UE is in idle mode, registered via RAN-1 for S-NSSAI-1, which is preferred in frequency band 1 (FB-1) and S-NSSAI-2, which is preferred in frequency band 2 (FB-2).

2)	An application in the UE needs to establish service on S-NSSAI-2 in FB-2.

3)	The UE establishes RRC connection with RAN-1.

4)	The UE triggers PDU Session Establishment Request on S-NSSAI-2 via RAN-1.

5)	The AMF is aware that S-NSSAI-2 is supported with preference in FB-2.

6)  AMF requests RAN-1 to steer the UE to a RAN Node supporting S-NSSAI-2 in FB-2.

7a)	If the UE has no other active PDU Session(s) on RAN-1, RAN-1 triggers inter-frequency cell change to RAN-2 in FB-2 which supports S-NSSAI-2 and is in UE's location.

NOTE: It is for RAN Working Groups to define the inter-frequency cell change in connected mode e.g. Handover, Cell Change Order, RRC release with re-direction or RRC Connection reconfiguration.

8a)	AMF continues with the PDU Session establishment procedure on S-NSSAI-2 via RAN-2.

9a)	After PDU session on S-NSSAI-2 is released, the UE falls-back (e.g. re-selects back) to a cell in FB-1. This step is optional. If not implemented, the UE will stay on FB-2 until it is steered back to FB-1 due to a new service request on FB-2.

7b) If the UE has already other active PDU Session(s) via RAN-1, the UE stays on RAN-1 and RAN-1 triggers the Dual Connectivity configuration with RAN-2 as a secondary RAN in order to establish the new PDU Session associated with S-NSSAI-2 while the existing PDU Session(s) via RAN-1 associated with S-NSSAI-1 stay on RAN-1.

### 6.31.4	Impacts on services, entities and interfaces

-	UE cell fall-back to a cell on the initial frequency band (optional).

-	AMF decision and request for handover to a cell from another frequency band.

-	RAN: can decide to stay on the current cell and deploy Dual Connectivity with a cell from another frequency band for the new PDU Session when there are existing PDU Sessions

******* end of the change *******