SA WG2 Meeting SA2-141e	S2-2007778

12 – 23 October, 2020, E-meeting	(revision of S2-2007620, S2-2007087, S2-2007576)

Source:	Intel, Charter, Comcast, Sony

Title:	KI #1, Proposed way forward

Document for:	Discussion / Approval

Agenda Item:	8.10

Work Item / Release:	FS_MUSIM / Rel-17

Abstract of the contribution: Proposes to interim conclusions on KI#1.

# 1	Proposal

Based on the discussion in S2-2007085 (refer to Proposals 2-6 and Slide #13) it is proposed to agree an interim conclusion on KI#1 as indicated below. Revision S2-2005685r19 (that could not be agreed during SA2#140E) is used as the basis. Yellow-highlighted text is new.

Relevant discussion part from S2-2007620. For KI#1, the paging cause/not to response to paging/response with BUSY are all depended on RAN and SA3 feedback. However, UE can be in an environment where network-A does not support MUSIM while network-B does. In this case, we suggest the following behaviour: - For UE operating in network-A that does not support MUSIM feature and the UE is being paged by network-A while engaging activity with network-B (which supports MUSIM feature), UE will try to respond to paging by network-A. If UE decides to ignore paging, then it is due to UE internal hardship or implementation and not according to 3GPP. - This above suggestion is to ensure the existing network deployment is not impacted by new feature developed in 3GPP. Furthermore, it is also suggested that even when network-A supports MUSIM operation, UE should not ignore paging from network-A. Hence, we propose BUSY indication (via NAS) to be agreed by SA2. If RAN came back with positive AS based option then SA2 can add this option as deployment choice. In any case, SA2 can decide to have NAS based solution to go forward as basis as this solution would allow common mechanism to be applied for both EPS/5GS system as much as possible.

* * * * Next Change * * * *

# 8	Conclusions

Editor's note:	This clause will capture conclusions from the study.

## 8.1	Key Issue #1: Handling of Mobile Terminated service with Multi-USIM device

The following interim conclusions are agreed for the baseline functionality:

Editor’s note: These are interim conclusions. Additional solution principles can still be proposed for the final conclusions.

-	One or more Paging Causes for both EPS and 5GS, pending feedback from SA3 and RAN WGs.

Editor’s note: The Paging Causes under consideration are:

One Paging Cause: MMTel Voice or Important service. Whether an additional Paging Cause is needed to differentiate a supporting RAN from a non-supporting RAN is FFS

Multiple Paging Causes: MMTel Voice, MMTel Video, SIP Signalling, NAS Signalling, SMS over NAS Signalling and Other data are considered

Editor’s note: Whether Paging Cause will be pursued to final conclusions will be confirmed in SA2#142E taking into account feedback from RAN2, RAN3 and SA3.

Editor’s note: It will be determined whether the Paging Cause is applied 1) only for UEs with the request, i.e. the UEs that have MUSIM capability and have multiple USIMs, etc, or 2) to all UEs indiscriminately.

Editor's note:	Whether AS or NAS BUSY indication is used to respond to Network A, when a Multi-USIM device that is paged by Network-A while the device decides not to accept the paging and the device supports the Busy indication,  is pending RAN and SA3 WGs’ feedback.

Editor's note:	The handling of the Busy indication requires further refinement at SA2#142. E.g. it is FFS whether the Multi-USIM device uses the Busy indication with a “shall”, “should”, “may” or “best effort” condition.

- The UE MMI shall not require input from the User in order to decide whether to respond to paging.

* * * * End of Change * * * *