3GPP TSG-WG SA2 Meeting #141E e-meeting 	S2-2007796

October 12 – 23, 2020, Electronic

# Title:	[DRAFT] Reply LS to Clarification on the assumption of only one AMF region to a tracking area

# Release:	Rel-16

# Work Item:	5GS_Ph1

Source:	3GPP SA2

To:	3GPP SA3-LI

Cc:

Contact Person:

Name:	Devaki Chandramouli

Tel. Number:

E-mail Address:	Devaki.chandramouli@nokia.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

# Attachments:	-

1. Overall Description:

3GPP SA WG2 thanks SA3-LI for their LS on Clarification on the assumption of only one AMF region to a tracking area.

Question: SA2 is kindly asked to clarify whether SA3 LI can assume that an AMF Region ID can uniquely be determined from the TAI.

SA2 Answer:

To avoid high signalling load at AMF (region) boundaries, some networks may choose to make AMF regions overlap. In such cases, one TAI can map to AMFs in more than one AMF region.

Some associated information:

Multiple AMFs can be connected to one gNB.

In order for the Service Request and Paging procedures to work, all of the AMFs connected to one gNB need to ensure that they do not allocate the same 5G-S-TMSI to more than one UE.

The coverage area served by an AMF is an integer number of Tracking Areas, i.e. Tracking Area boundaries do not cross AMF boundaries. The List of TAIs allocated to a UE by an AMF is also contained within the coverage are of that AMF.

2. Actions:

To 3GPP SA3-LI

ACTION:

SA WG2 requests SA3-LI to take the above answer into account.

3. Date of Next TSG SA WG2 Meetings:

TSG-SA2 Meeting #142E   		16 - 20 November 2020