SA WG2 Meeting #142E (e-meeting)	S2-2008702r03

Nov 16 – Nov 20, 2020, Elbonia

Source:	CATT, Huawei, HiSilicon, LG Electronics

Title:	KI#13 conclusion update

Document for:	Approval

Agenda Item:	8.1

Work Item / Release:	FS_eNA_ph2 / Rel-17

Abstract of the contribution: This contribution proposes the conclusion update for Key Issue #13.

- Discussion

For Key Issue #13, on configuration of triggers used by the NWDAF for data collection/analytics and triggers used by analytics consumers to request for or subscribe to analytics, no conclusion has been reached yet.

The configuration of triggers is important because it supports the NWDAF or analytics consumers to have the proper parameterization for initiating, respectively, (real-time) data collection and/or analytics (e.g., correct timing, conditions, target and/or filters for subscribing to data from NF sources of data or OAM), or analytics request/subscription (e.g. proper conditions, target and/or filter information for requesting for/subscribing to analytics information from the NWDAF).

Therefore, this contribution focus on covering the gap in current conclusions on trigger configuration.

In current TR, configuration of triggers for NWDAF on (real-time) data collection/analytics is mainly addressed in solution #73, and configuration of trigger for analytics request/subscription taking into account of service requirements and/or network policy is mainly addressed in solution #75 and #73. Other solutions, e.g. solution #44, #45 and #74, address the triggers themselves instead of configuration of triggers.

In Solution #73, the following aspects are proposed:

- On configuration of triggers used by the NWDAF for (real-time) data collection/analytics:

- The PCF provides Network Analytics Policy to the NWDAF.

- The AF provides service specific analytics control information to the PCF, and based on this the PCF derives the Network Analytics Policy for specific services/applications.

- On configuration of triggers used by analytics consumers to request for or subscribe to analytics information:

- The PCF provides network analytics control information to the AMF and SMF.

- The AF provides service specific analytics control information to the PCF, and based on this the PCF derives the network analytics control information for specific services/applications.

For the above aspects 1) and 2):

-	Provision of Network Analytics Policy to the NWDAF by the PCF can be realized with the introduction of the NWDAF to 5GC, as an enhancement to Rel-16 eNA features, to facilitate real-time data collection/analytics which is very important for the applicability/usage of analytics information. So it can be considered for normative work in Rel-17. As a complement, the AF provides service specific analytics control information to the PCF as one input for deciding on Network Analytics Policy for specific services/applications.

-	Considering the impacts to the AMF/SMF, provision of network analytics control information to the AMF/SMF by the PCF may be considered beyond Rel-17.

Regarding Solution #75 the following points are identified:

-	The analytics subscription information proposed in Solution #75 is complementary to solutions #45 and #74, as it can support the refinement of the configuration of the target of analytics IDs that is used by the NFs to subscribe/request analytics IDs to NWDAF.

-	Currently the signaling used in Sol #75 is based in AF, NEF and UDM interaction; and then analytics consumers and UDM interactions. However, a different set of entities or a different procedure can be used for providing the analytics subscription information to analytics consumers to support the parameterization of the analytics subscription when triggered. For instance, UDR could be used instead of UDM to store the analytics subscription information, and the existing service specific parameter provisioning in TS 23.502 could be used/extended in order for NFs to obtain the analytics subscription information. Therefore, it is proposed to consider the principle of Solution #75 of AF providing the analytics subscription information to serve as configuration information related to the triggers for NFs analytics consumers, and determine during normative phase the procedure to be used for providing such information to the consumer NFs of analytics IDs.

In this contribution, it is proposed the following aspects to be considered during the normative work:

-	From Solution #73: For configuration of triggers used by the NWDAF for data collection and analytics, the PCF provides Network Analytics Policy to the NWDAF. The AF provides service specific analytics control information to the PCF, and based on this the PCF derives the Network Analytics Policy for specific services/applications.

-	From Solution #75: For configuration of triggers used by analytics consumers to request for or subscribe to analytics from the NWDAF, the AF provides to 5GS the analytics subscription information containing the restriction and/or permission for specific UEs or group of UEs for analytics IDs. The analytics subscription information supports the parameterization of the target of analytics for analytics IDs in triggered requests or subscriptions to NWDAF by analytics consumers. The specific procedure of how analytics consumers obtain the analytics subscription information will be determined during normative work.

Besides, the following NOTE is missing when implementing S2-2008061 in TR 23.700-91 v1.1.0 and should be added to the conclusion:

NOTE:	Solution #10 is to be concluded under KI#2 and/or KI#11. Solution #37 is to be concluded under KI#11.

# 2. Proposal

It is proposed to include the following changes in TR 23.700-91.

***** Start of 1st Change *****

## 8.X	Conclusion for Key Issue #13: Triggering conditions for analytics

Regarding triggers used by analytics consumers to request for or subscribe to analytics from the NWDAF, solution #45 and #74 are selected for the normative work. Solution #44 is selected for the normative work with introduction of the new Analytics ID "Session Management Congestion Control Experience (SMCCE)".

Regarding triggers used by the NWDAF for data collection from the UE, solution #65 is to be discussed under Key issue#8.

NOTE 1:	Solution #10 is to be concluded under KI#2 and/or KI#11. Solution #37 is to be concluded under KI#11.

***** End of Changes *****