3GPP TSG-WG SA2 Meeting #142E e-meeting 	S2-2008397

Elbonia, Nov 16 – 20, 2020	(revision of S2-200xxxx)

Source:	Huawei, HiSilicon

Title:	KI#1, sol#44, add the impacts

Document for:	Approval

Agenda Item:	8.2

Work Item / Release:	FS_eNPN / Rel-17

Abstract: This contribution introduces impacts of sol#44.

1. Introduction/Discussion

This contribution introduces impacts of sol#44.

2. Text Proposal

It is proposed to capture the following changes vs. TR 23.700-07 V1.1.0.

# * * * * First change * * * *

6.44	Solution #44: UE external subscription data management in the SNPN

6.44.1	Introduction

In the case, it is to address how the SNPN to obtain the UE's subscription data owned by 3rd party (e.g., AAA-Server). Due to business requirement or other reason, the 3rd party has an agreement with the SNPN that can store/manage the UE's subscription data locally in SNPN UDM so that the necessary procedures (e.g., registration procedure, PDU session management procedure, etc.) can be completed with the UE's subscription data.

The subscription data includes:

-	Session related subscription data.

-	Access and mobility subscription data.

-	Slice related and others.

The assumptions are following:

-	The 3rd party has an agreement with SNPN to store/manage the 3rd party UE's subscription data locally.

-	The 3rd party reformats the subscription data to fit the SNPN UDM based on the agreement.

NOTE:	3rd party can provide the subscription data that may include the slice information fit for the SNPN based on agreement if needed.

-	The security parameters are not sent to SNPN so that the authentication and authorization between SNPN and the 3rd party is not impacted.

-	There is one dedicated UDM in the SNPN which handle the external UE subscription data, when there are multiple UDMs in the serving SNPN. Additionally, AMF/SMF/PCF may be preconfigured with the dedicated UDM ID for UDM selection when the subscription of the UE is stored in 3rd party. Alternatively, AMF/SMF/PCF may act as the NF consumer to NRF to discover the UDM instance(s) and selects a UDM instance based on the available UDM instances obtained from NRF, as described in clause 6.3.8 in TS 23.501 [4].

6.44.2	Functional Description

Figure 6.44.2.2-1, it describes overview of the architecture.

Figure 6.44.2.2-1 architecture for UE external subscription data exchange between UDM and AAA-Server

New potential interface:

-	Interface Y1: between UDM and AAA-Server

NOTE:	it is also possible to exchange the subscription data between UDM and AAA-Server via AUSF, or other 3GPP function defined in future.

6.44.3	Procedures

The figure 6.44.3-1 below shows a high-level flow how to enable the SNPN to obtain UE external subscription data.

Figure 6.44.3-1: High-level flow for UE external subscription exchange between serving SNPN and AAA-Server

The general procedure is described as following:

1.	3rd party UE turns on and initiates registration request to the SNPN AMF via AN, including UE ID

NOTE 1:	How the UE discoveries and selects the SNPN can be based on any applicable mechanisms described in other TR solutions in which the Home SP can be the 3rd party.

2.	AMF triggers to perform external authentication and authorization between UE and 3rd party AAA-server.

NOTE 2:	external authentication and authorization is in scope of SA WG3.

3.	Once the authentication and authorization is success, AAA-Server determines whether to send the UE subscription data to the SNPN UDM via based on the 3rd party policy/service agreement with the SNPN, including UE ID, and subscription data, e.g., session management subscription data, access and mobility subscription data, etc.

Since 3rd party has the service agreement with other networks (e.g., SNPN), it could create and maintain a set of subscription data that is suitable for the network in advance or on demand.

4.	3rd party AAA-Server sends the UE's subscription to SNPN UDM, including UE ID, subscription data.

5.	UDM determines if accept and store the UE's subscription to the UDR when receiving the request from 3rd party AAA-Server based on local policy and the agreement between SNPN and the 3rd party.

6.	UDM feedbacks the result to 3rd party AAA-Server. 3rd party AAA-Server records the SNPN for which AAA-Server has provisioned UE subscription data.

7.	SNPN completes the registration procedure. During this step, SNPN may generate UE configuration parameters (e.g. NSSAI, DNN, etc) based on the subscription data provided by 3rd party and send the UE configuration parameters to UE.

8.	If the UE subscription data in 3rd party AAA-Server is changed, and 3rd party AAA-Server has provisioned UE subscription data to SNPN UDM before UE subscription data is changed, 3rd party AAA-Server sends the updated UE subscription data to corresponding SNPN UDM.

9.	UDM feedbacks the result to 3rd party AAA-Server.

6.44.4	Impacts on services, entities and interfaces

Impacts on UDM and AAA-Server

-	Exchange the subscription data between UDM and AAA-Server.

Impacts on UE, SNPN and AAA-Server

-	SNPN supports External authentication and authorization between UE and AAA-Server.

# * * * * End of changes * * * *