3GPP TSG-WG SA2 Meeting #142E 	S2-2008478

Nov 16 -20, 2020, Emeeting	(revision of S2-200xxxx)

Source:	Ericsson

Title:	KI#7, updates to evaluation

Document for:	Approval

Agenda Item:	8.4

Work Item / Release:	FS_eNS_Ph2 / Rel-17

Abstract: This paper provides further updates to the evaluation for KI#7.

# Introduction

This pCR provides further updates of the evaluation for Key Issue #7, titled, “Support of 5GC assisted cell selection to access network slice”.

# Discussion

The following new solutions have been added to the TR for KI#7:

-	Solution #45 "Compatibility of S-NSSAIs operating frequency bands with UE Radio Capabilities"

-	Solution #46 "Support RAN-Based UE redirection with enhanced RRC-level information"

Solution #45 principle is to cover the case when "UE's current TA has some S-NSSAIs not available on all cells", and it does so by AMF using an extension of the UE Capability Match procedure to check whether the UE's radio capabilities "matches" those required for the S-NSSAIs to be seen as available in the TA.

Solution #46 principle is to enable RAN to perform a redirection of the UE based on 5GC providing to RAN the Rejected S-NSSAI, Configured NSSAI and direct AMF selection information, and optionally RAN provides to the UE in, RRC connection release, the FBs supported by slices and whether to request NSSAI without inclusion of GUAMI/GUTI.

The evaluation description and the table 7.7-1 should be updated with the above solutions.

There are also a number of ENs to be addressed.

Editor's note:	Whether different cells within a tracking area can support different S-NSSAIs is based on RAN WGs feedback.

As RAN3 reported to TSG RAN in see RP-201401, states the following:

"Status Quo in Rel-16 is that the slices included in an Allowed NSSAI are available anywhere (i.e. in any cell) within the UE’s Registration Area"

Therefore we can already now finally agree on what Rel-15/16 assumes i.e. that all cells of a TA support the network slices that NG-RAN indicated as supported during NG SETUP, and as per SA2 specifications, therefore all cells of the derived RA also supports the network slices provided in the Allowed NSSAI.

Editor's note:	Whether there are use cases where the 5GC cannot determine suitable slices to register for a UE requesting slices that cannot be used at the same time due to the slices are not defined to be available in the same frequencies is FFS.

It is not obvious if there are such valid use cases, i.e. EN can be kept until such use case is provided.

On the table 7.7-1 solutions #45 and #46 needs to be added, and also row 8 " Can the UE change registration to network slices in CM-CONNECTED state?" needs to be corrected.

Solutions 29 and 30, from the current descrption it seems answer is conditional and if slice is not available in frequency of current cell UE then release RRC connection and performs autonomous Cell Reselection, and answer is Yes if slice is available in same frequency as current cell.

# Proposal

Based on the above observations and proposals, it is proposed to add the following in the TR 23.700-40.

*** Start of Changes ***

## 7.7	Evaluation on solutions of KI#7

Editor's note:	This clause will need further updates based on RAN WGs feedback. This clause will need further update taking in consideration new solutions added in SA2#141e.

The main issue to be resolved in KI#7 is how the 5GS steers UEs to a 5G-AN (e.g. a specific frequency band supported by the UE) that supports the network slices that the UE can use. Solutions 17, 29, 30, 31, 44, 45 and 46 are for Key Issue #7. All solutions, except solution 45, are compatible with the Rel-16 assumption that each cell in the tracking area supports the same S-NSSAI(s), and all cells of the derived RA also supports the network slices provided in the Allowed NSSAI.

Solutions 17, 29, 30 and 45, 46 focus on the Registration procedure while solutions 31 and 44 focus on the PDU session establishment procedure.

Solution 17 allows the UE to request registration for S-NSSAIs outside of the Allowed NSSAI. The 5GC then determines which slices are preferred, and steers the UE towards the preferred S-NSSAIs, or rejects the requested slice(s). Thus, the solution assumes that the network always can decide which network slices to register without explicitly knowing what network slice(s) the user prefers.

Editor's note:	Whether there are use cases where the 5GC cannot determine suitable slices to register for a UE requesting slices that cannot be used at the same time due to the slices are not defined to be available in the same frequencies is FFS.

Solution 29 also allows the UE to request registration for S-NSSAIs outside of the Allowed NSSAI. The UE is provided with frequency band information per network slice in the Configured NSSAI. When the UE wants to register to slice(s) the UE may perform cell selection/reselection based on the received frequency band information of the slice(s). It is then understood that the UE will only request slices that are accessible simultaneously in the selected cell and the AMF will include in the Allowed NSSAI only S-NSSAI(s) that are accessible simultaneously. Solution 29 has an impact on the UE.

Solution 30 also allows the UE to request registration for S-NSSAIs outside of the Allowed NSSAI. The UE is provided with frequency band information per network slice in the Configured NSSAI. Each of a slice's target carrier frequencies can also be associated with a priority. The UE uses this information during cell selection in Idle mode when the UE wants to use a network slice (as in solution 29). In Solution 30, the UE is allowed to register to multiple network slices which are accessible on different preferred frequency band(s) within the Registration Area. The solution mentions that secondary cell (e.g. Dual Connectivity) can be used in some scenarios, but this assumption is not valid in all scenarios (e.g. if the UE is registered to 3 slices that are not all available in the on a common operating band or if the UE is not capable of NR/NR DC). Solution 30 has an impact on the UE.

Editor's note:	Impacts on the cell selection/reselection mechanisms would require feedback from RAN WGs and CT WG1.

Solution 31 and solution 44 propose mechanisms to steer UEs to a 5G-AN (e.g. a specific frequency band) during the PDU Session establishment procedure. Both solutions assume that the network can be configured with preferred frequency band information for each slice but this information is not provided to the UE. The difference is in solution 31 the AMF decides to steer the UE to a target RAN Node while in solution 44 the RAN decides to steer the UE to the target RAN Node.

Solution 44 doesn't have any impact on the system. Solution 44 does not handle the case where the operator manages a different range of radio spectrums for some network slices (i.e. some network slices cannot be accessed via certain frequencies).

Solutions 30 and 31 do not explain how to handle the case where the UE attempts to maintain PDU Sessions in slices that are not all accessible in the same operating band. Both solutions propose to optionally use NR/NR Dual Connectivity in order to use different frequency bands simultaneously when the UE supports NR/NR Dual Connectivity. This is not possible in all scenarios (e.g. if the UE is registered to 3 slices that are available on a common operating band in the primary cell or if the UE is not capable of NR/NR DC).

Table 7.7-1 summarized some differences between solutions 17, 29, 30, 31, and 44.

Table 7.7-1: Key Issue #7 Solution Comparison

| # | Criteria | Solution 17 | Solution 29 | Solution 30 | Solution 31 | Solution 44 | Solution 45 | Solution 46 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | When slice(s), to which the UE wants to register, are not available in a current cell, is the UE able to select a different cell/band that supports the slice(s)? (i.e. Is the UE aware of what slices are supported by each operating band?) | No UE can select a slice not available in current cell and get re-directed to wanted slice/TA | Yes Supported bands are provided in the Configured NSSAI. | Yes Supported bands of the Serving PLMN are provided in the Configured NSSAI. | No Slice availability based on UP resources | No Slice availability based on UP resources | No | Conditional. UE can select a slice not available in current cell and get re-directed to wanted slice/TA. UE may be provided by RAN with Supported bands of slices in Configured NSSAI. |
| 2a | Can the UE have slices in its Allowed NSSAI that are not all available on some common operating band(s)? FFS: The need of this Criteria depends on RAN feedback. | No | No | Yes | No | No | No | No |
| 2b | Does the solution allow the UE to simultaneously use all slices in the Allowed NSSAI (i.e. simultaneously maintain PDU Sessions in all of the Allowed slices per TS 23.501 [2], clause 5.15.5.1.1)? FFS: The need of this Criteria depends on RAN feedback. | Yes | Yes | Yes Valid if all S-NSSAIs of the Allowed NSSAI operate common frequency band (e.g. in the primary cell). | Yes | Yes | Yes | Yes |
| 3 | How does 5GS steer UEs to a 5G-AN (e.g. a specific frequency band) that can support the network slices that the UE wants to use. | RAN Redirects the UE when it detects, as per NSSAI and RFSP provided by AMF, that the UE wants to access a slice that is not available in the current band. | The UE is told what operating bands will work with each slice. The UE in Idle mode selects a cell in the frequency band of the requested NSSAI. The network can consider the Allowed NSSAI when determining what cell to handover the UE to The network can consider the Allowed NSSAI when configuring the UE's cell selection priorities. | The UE is told what operating bands will work with each slice. The UE in Idle mode selects a cell in the frequency band of the requested NSSAI. The network can consider the Allowed NSSAI when determining what cell to handover the UE to The network can consider the Allowed NSSAI when configuring the UE's cell selection priorities. | RAN steers the UE based on the requested S-NSSAI provided by the AMF in the PDU Session Establishment Request procedure. | RAN steers the UE based on the S-NSSAI provided by the SMF in the PDU Session Establishment Request procedure, and when activating UP resources. | Not described | RAN Redirects the UE when it detects, as per Rejected S-NSSAIs provided by AMF, that the UE wants to access a slice that is not available in the current band. |
| 4 | What information does the 5G-AN need to steer UE to a proper 5G-AN. | RAN uses the Target NSSAI and corresponding RFSP from the AMF | Existing Mechanisms (i.e. RFSP Index and the RAN can consider the Allowed NSSAI) | Existing Mechanisms (i.e. RFSP Index and the RAN can consider the Allowed NSSAI) | RAN uses the S-NSSAI provided by the AMF in the PDU Session Establishment Request procedure | RAN uses the S-NSSAI provide by the SMF in the PDU Session Establishment Request procedure and Existing Mechanisms (i.e. RFSP Index and the RAN can consider the Allowed NSSAI) for subsequent steering. | Not described | RAN uses the Rejected S-NSSAIs from the AMF |
| 5 | What information should be provided to the UE to select a proper 5G-AN and how it is sent to the UE. | No new information is sent to the UE. | Supported bands are provided in the Configured NSSAI. | Supported bands of the Serving PLMN are provided in the Configured NSSAI. | No new information is sent to the UE. | No new information is sent to the UE. | Not described | Optional information Supported bands of S-NSSAIs in the Configured NSSAI. |
| 6 | RAN Impacts | Support getting additional Target NSSAI and RFSP Index for the Target NSSAI, and support moving the UE accordingly. | None, the Allowed NSSAI is already provided to the RAN via N2. | None, the Allowed NSSAI is already provided to the RAN via N2. | RAN uses the S-NSSAI provided by the AMF in the PDU Session Establishment Request procedure to steer the UE. | None, the Requested NSSAI is already provided to the RAN via N2 SM message. | NGAP information to AMF that NG-RAN's TA has some S-NSSAIs not available on all cells, and NG-RAN verifies the operating frequencies corresponding to the S-NSSAIs received from the AMF with respect to the UE radio capabilities, and determines the combinations of S-NSSAIs that are compatible with the UE Radio Capability | Support taking into account the Configured NSSAI and rejected S-NSSAIs when providing CellReselectionPriority list to the UE, storing Configured NSSAI and any direct AMF selection information, provision of additional information in RRC connection release or RRC connection reconfiguration |
| 7 | UE Impacts | No | Yes | Yes (UE needs to support DC) | No (UE needs to support DC) | No (UE needs to support CA/DC) | No | Yes, Optional |
| 8 | Can the UE change registration to network slices in CM-CONNECTED state? | Yes Assumes AMF supports the slices or can re-allocate to new AMF | Conditional Yes if slice is available in same frequency as current cell. No, if slice is not available in frequency of current cell, and UE then release RRC connection and performs autonomous Cell Reselection | Conditional Yes if slice is available in same frequency as current cell. No, if slice is not available in frequency of current cell, and UE then release RRC connection and performs autonomous Cell Reselection | Yes | Yes | Yes, but there is no redirection of the UE i.e. new slices may rather be rejected per RA. | Yes Assumes AMF supports the slices or can re-allocate to new AMF |

Editor's note:	This table will need to be updated with new solutions from SA2#141e.

*** End of Changes ***