3GPP TSG-WG SA2 Meeting #142E e-meeting 	S2-2008504

Elbonia, November 16 – 20, 2020	(revision of S2-200xxxx)

Source:	Huawei, HiSilicon

Title:	KI#3: Conclusion for the solutions

Document for:	Approval

Agenda Item:	8.4

Work Item / Release:	FS_eNS_ph2/ Rel-17

Abstract: This contribution introduces the conclusion of all the solutions of KI#3.

1. Introduction/Discussion

According to the evaluation on solutions of KI#3 as described in clause 7.3 of TR 23.700-40, Solution #22 could resolve KI#3 with the least impact. And Solution #13 could also have an accurate enforcement of SMBR, but for multiple PSA UPF scenario, it may need to select the same I-UPF.  Hence, for the sake of progress, it is proposed to have an interim conclusion as following : It is recommended to use Solution #22 to resolve KI#3 if RAN Groups confirm it is feasible.

2. Text Proposal

It is proposed to capture the following changes vs. TR 23.700-40.

# * * * * First change * * * *

8	Conclusions

Editor's note:	This clause will capture conclusions from the study.

8.x	Interim conclusion on solutions of KI#3

An accurate limitation of data rate per network slice in UL and DL per UE is preferred, and that’s why one of the UP based solution (i.e. Solution #13 or Solution #22) is selected as the baseline to resolve KI#3:

- If it is feasible for RAN to support the enforcement of SMBR, Solution #22 is recommended as the baseline.

- If it is not feasible for RAN to support the enforcement of SMBR, Solution #13 is recommended as the baseline with some potential enhancement, (e.g. select the same I-UPF to better support the multiple PSA scenario).

# * * * * End of changes * * * *